APPENDIX 3:

SUMMARY LIST OF INFORMATION TO BE GATHERED BY DRY CLEANERS

The following is a summary list of the information which must be gathered for a period of 12 months by dry cleaners. It is a summary only - for more detail see the excel spreadsheet of sample records. You use this information to compile the solvent management plan (see section 7) to see if you meet the requirements of the Regulations. The approved assessor will then review the solvent management plan and carry out spot checks on the back up documentation.

Information to be gathered by dry cleaners for a period of 12 months and kept on record prior to obtaining an Approved Assessor:

1. Weight of every load of product dry cleaned in the 12 month period.

2. Volume of perc added to the machine(s) in the 12 month period.

3. Purchases of perc in the 12 month period, including back-up invoices.

4. The weight of every load of perc containing waste sent off site in the 12 month period, and the appropriate waste transfer documentation from the waste contractor.

5. An analysis of a representative sample of waste for perc content (preferably in mg/kg). See Appendix 4 for accurate sampling methods. This does not have to be repeated annually, only when there is a change in equipment or a major change in operating practices.

6. An analysis of a representative sample of separator water for perc content. This does not have to be repeated annually, only when there is a change in equipment or a major change in operating practices.

7. An estimate of the volume of separator water generated in the 12 month period.

8. The stock levels on the opening day of the 12 month period of:

a. perc volume in machine tanks.

b. perc volume in machine carbon adsorber (if present).

c. perc volume in fresh containers, if any.

d. weight of perc containing waste on the premises.

9. The stock levels on the closing day of the 12 month period of:

a. perc volume in machine tanks.

b. perc volume in machine carbon adsorber (if present).

c. perc volume in fresh containers, if any.

d. weight of perc containing waste on the premises.

10. Information on the weight of any carbon from adsorbers disposed of in the 12 month period.

11. A written record of any training carried out.

A written record of any machine maintenance carried out.