| Microbiological standards |  |  |
| --- | --- | --- |
| Parameter | Description | Standard |
| E. coli and Enterococci | Bacteria found in the gut of humans and warm blooded animals. Their detection in water is an indication of faecal contamination. They are readily killed by disinfection but the occasional detection in drinking water can occur. | 0 per 100 ml |
| Coliforms | Group of organisms widely found in the environment including soil, water and vegetation. Their presence in water may indicate faecal pollution has occurred. They are readily killed by disinfection but the occasional detection in drinking water can occur. | 0 per 100 ml |
| Clostridium perfringens | Bacterium that can produce spores that can persist in the environment for long periods of time. Their presence in water can indicate historic contamination. | 0 per 100 ml |
| Colony counts measured at 22°C | General measure of the bacterial population within drinking water. | No abnormal change |

Parameter; Description; Standard. 1,2- dichloroethane; Found in industrial solvents and can be detected in trace amounts in some source waters.; 3µg/l. Acrylamide; Monomer is found in polyacrylamide which can be used in the treatment of water to remove impurities. Use of polyacrylamide is tightly controlled.; 0.1µg/l. Aluminium; Occurs as a natural constituent of many waters. During the treatment process aluminium salts are used to remove impurities.; 200µg/l. Ammonium; Ions are found naturally in most waters and removed by treatment.; 0.3mg/l. Antimony; Rarely found in water and when this does occur it is likely to be due to the water being in contact with brass fittings or lead free solder.; 5µg/l. Arsenic; Specific treatments can be used but it is not normally found in the Dublin Region.; 10µg/l. Benzene; Used in the petrochemical and plastics industry. Occasionally it is found in source water but is removed by treatment.; 1µg/l. Benzo(a)pyrene; One of several compounds known as polycyclic aromatic hydrocarbons. Trace levels can be found in drinking water where coal tar lining of mains was historically practiced to prevent corrosion.; 0.01µg/l. Boron; Occurs naturally at low levels in all waters. Some industrial discharge and detergents can increase the concentrations in river water. Boron is not removed by normal water treatment.; 1mg/l. Bromate; Formed during the disinfection of drinking water though the reaction with natural bromide. It can also occasionally be detected in water through industrial pollution.; 10µg/l. Cadmium; Specific treatments can be used but it is not normally found in the Dublin Region water supplies.; 5µg/l. Chloride; Found as natural salts in all waters. It is not removed during normal treatment. Concentrations depend on the local water source.; 250mg/l. Chromium; Very rarely found in drinking water.; 50µg/l. Colour; Occurs naturally in many water sources. It is readily removed by treatment.; Acceptable to customers, no abnormal change.. Conductivity; Measure of the dissolved inorganic substances.; 2500µS/cm at 20°C. Copper; Rarely found in source waters but can leach into drinking water as it passes through domestic copper pipes and fittings.; 2mg/l. Cyanide; Very rarely found in drinking water.; 50µg/l. Epichlorohydrin; Found in polyamines which can be used in the treatment of water to remove impurities. The use of polyamines is tightly controlled.; 0.1µg/l. Fluoride; Occurs naturally is many water sources. Fluoride is added to the water supply to prevent tooth decay at the request of the Department of Health and Children.; 0.6 – 0.8mg/l. Hydrogen Ion (pH); Measure of the acidity of the water. pH values above 7 indicate alkaline conditions, pH 7 is neutral and below pH 7 is acidic.; ≥6.5 - ≤9.5 pH. Iron; Naturally present in most water sources. Iron can also be present in drinking water through corrosion of iron water mains.; 200µg/l. Lead; Rarely found in source waters but can be found in drinking water due to pick up from lead pipes and fittings in domestic properties.; 25µg/l. Manganese; Occurs naturally in many water sources. It is readily removed by treatment.; 50µg/l. Mercury; Very rarely found in drinking water.; 1µg/l. Nickel; Rarely found in source water.; 20µg/l. Nitrate; Occurs naturally in most source waters but concentrations can be increased as a result of fertiliser use.; 50mg/l. Nitrite; Occurs naturally at low levels in some waters but is removed by treatment.; 0.5mg/l at consumers' tap 0.1mg/l at water treatment works. Oxidisability; The oxidisability parameter provides an indication of changes in water quality and allows an estimate of the organics removal during water treatment.; 5.0 mg/l O2. Pesticides; Diverse group of organic compounds that include herbicides, insecticides, fungicides. Many water sources contain traces of one or more pesticides as a result of both agricultural and non-agricultural uses.; 0.03µg/l for aldrin, dieldrin, heptachlor and heptachlor epoxide. 0.1µg/l for other individual pesticides. 0.5µg/l for the total of all pesticides detected. Polycyclic aromatic hydrocarbons; Found in drinking water where coal tar lining of mains was historically practiced to prevent corrosion. The standard is a measure of benzo(b)fluoranthene, benzo(k)fluoranthene, benzo(ghi)perylene and indeno(1,2,3-cd)pyrene.; 0.1µg/l. Radioactivity-Tritium; Found naturally in water at very low concentrations. Elevated levels may indicate the presence of other artificial radionuclides.; 100Bq/l. Radioactivity- total indicative dose; Measure of radiation exposure through drinking water. Radioactivity is naturally present in all water sources. Levels of radioactivity are normally monitored by measuring gross alpha or beta activities.; 0.1 mSv/year. Selenium; Very rarely found in drinking water.; 10µg/l. Sodium; Naturally present in many water sources.; 200mg/l. Sulphate; Naturally present in many water sources..; 250mg/l. Taste & odour; Measure of the aesthetic quality of drinking water. Unusual tastes or odours may indicate a problem that requires investigation.; Acceptable to customers, no abnormal change.. Tetrachloroethane & trichloroethane; These are solvents which are very occasionally found in water sources. The standard is a measure of the combined concentrations.; 10µg/l. Total organic carbon; Measure of the amount of organic material in the water, most of which comes from natural sources.; No abnormal change (mg/l). Trihalomethanes total; Formed by the reaction of chlorine with natural organic substances in water. The standard is a measure of chloroform, bromoform, dibromochloromethane and bromodichloromethane.; 100µg/l. Turbidity; Measure of the clarity of water.; Acceptable to consumers, no abnormal change.. Vinyl chloride; Found in water pipes containing polyvinyl chloride (PVC). Concentrations are strictly controlled by product specification.; 0.50µg/l.