Comhairle Chontae an Cabhain

CAVAN  COUNTY  COUNCIL

THE QUALITY OF PUBLIC DRINKING WATER IN COUNTY CAVAN

A Report for the Year 2009

J. Keyes						E Doyle					Environment Section

County Manager					Director of Services	 			Cavan County Council

September 2010

### Index

1.	Introduction

2.	Summary of the Monitoring Results for Public Drinking Water Supplies

3.	Parameter By Parameter Analysis of Drinking Water

1.	Introduction

This report presents the summary of the monitoring data for 2009 for the drinking water of County Cavan. It is intended as a companion volume to those issued in previous years.

Legal Requirements: European Communities (Drinking Water) Regulations, 2000 (S.I. 439 of 2000), European Communities (Drinking Water) Regulations 2007 (S.I. No. 106 of 2007), European Communities (Drinking Water) (No. 2) Regulations 2007 (S.I. No. 278 of 2007)

Monitoring programme: The regulations governing drinking water monitoring changed in 2007. European Communities (Drinking Water) (No. 2) Regulations 2007 (S.I. No. 278 of 2007) replaced European Communities (Drinking Water) Regulations 2007 (S.I. No. 106 of 2007) which had replaced European Communities (Drinking Water) Regulations, 2000 (S.I. 439 of 2000). The monitoring regime remains rationalised into 2 categories: check monitoring and audit monitoring. The purpose of check monitoring is to regularly provide information on the organoleptic and microbiological quality of the water supplied for human consumption as well as information on the effectiveness of drinking water treatment. Audit monitoring requires fewer number of samples but is more demanding in the number of parameters to be analysed. Additional operational monitoring is carried out in order to monitor the effectiveness of the water treatment system. The current monitoring programme is based on the legal requirements and the need to provide an effective management tool in order to maintain quality assurance in our public water supply. Where problems arise, monitoring is increased in order to confirm the effectiveness of corrective action taken. Consequently the number of samples analysed reflect this additional monitoring. The vast bulk of the monitoring is undertaken in-house by Cavan County Council’s own laboratory which carries out the basic organoleptic, physicochemical and microbiological tests on drinking water. Additional specialised monitoring, for a small number of samples, is undertaken by the EPA’s regional laboratory in Monaghan on behalf of Cavan County Council.

This report considers the findings of our drinking water monitoring programme for the year 2009 and assesses the quality of each public drinking water supply.  The majority of Cavan public water is supplied by surface waters. The geomorphology of  County Cavan has the effect that these supplies are generally from shallow surface sources and this raw water can be difficult to treat. Invariably, monitoring results can indicate elevated colour levels. The new regulations do not specify a specific MAC (Maximum Admissable Concentration) for colour and turbidity. This is subject to drinking water being ‘deemed acceptable by the consumer.’ Colour and turbidity are naturally occurring in water and vary from season to season and according to weather patterns.  As a result it can be difficult to achieve consistent treatment. Aluminium based compounds are used to remove colour and turbidity and where difficulties arise with the treatment process there can be exceedances for this parameter also. A total of 143 samples were analysed in 2009 for E Coli in public water. The overall percentage compliance rate was 100% with no exceedances. The overall percentage compliance rate, for E Coli, has generally improved over the last ten years see table below.

| Year | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| % Compliance | 90.9 | 93.8 | 97 | 93.9 | 98.4 | 100 | 99.5 | 99.5 | 99.3 | 98.1 | 100 |

The public drinking water supplies of County Cavan are generally of good potable quality. All legal monitoring requirements for 2010 have been complied with

- Summary of the Results of Monitoring for Public Drinking Water Supplies

# Public supplies are listed alphabetically.

# Summary data

2.1      Details of source treatment facilities

Provides details on the name of the source, the type of source, source treatment.

2.2	Sampling programme

# Provides details of the supply, the minimum number of samples per year as a legal

# requirement, and the number of samples monitored in 2009

# .

  - Details of exceedances of regulations by parameter

Lists parameter, the supply and the excess value recorded, for 2009

  - Analysis of exceedances by parameter.

Details exceedances of a variety of parameters for the last number of years. The table is abbreviated for clarity

Clo= Clostridia perfringens,  Al= Aluminium, THM= TriHaloMethanes, Coli= Coliform bacteria(includes E Coli), Co= Colour , F= Fluoride,

Fe= Iron, Mn= Manganese,  NO3= Nitrate, NO2= Nitrite, Od= Odour,  pH= (Hydogen Ion),  Ta= Taste, Tu= Turbidity, NH4=Ammonium,	 N/A= Not Applicable

.

  - General Assessment of  position

Overall assessment of quality for this supply.

##### Supply

| ###### Arva |  |
| --- | --- |
| Bailieborough Regional |  |
| Ballinagh |  |
| Ballyconnell |  |
| Ballyhaise |  |
| Ballyjamesduff Regional |  |
| Bawnboy |  |
| Belturbet |  |
| Blacklion |  |
| Cavan Regional |  |
| Cootehill |  |
| Dowra |  |
| Gowna |  |
| Killeshandra |  |
| Kingscourt |  |
| Shercock |  |
| Swanlinbar |  |

Arva

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Arva |

SOURCES:		SW: Surface water;

TREATMENT:	RGF: Rapid Gravity filtration, Chlor: Chlorination

Coag: Coagulation/Flocculation, pHC: pH Correction

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Arva | 225 | 525 | 1 | 4 | # 1 | # 5 | # 6 |

2.3      Details of exceedences of regulations by parameter

________________________________________________________________________________________________________

Parameter		MAC					Excess values recorded			Number of values

__________________(Maximum Admissable Concentration)____________________________________________________________

pH (pH units)			6.5-9.5				{5.89, 5.96, 6.37, 6.4}					4

Aluminium(mg/l)		0.200				{0.342, 0.552,0.76, 0.47, 0.49, 0.328}		6

Iron 	(mg/l)			0.200				{0.443}						1

Lead     (ug/l)			25				{59.3}							1

Pesticides			0.1				{0.17}							1

2.4     	Analysis of exceedances by parameter.

____________________________________________________________________________________________________________________

Year			No. of Samples____________________Analysis of Exceedances by Parameter__________________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu	NH4

___________________________________________________________________________________________________________________

1999			13			  4	   1	  0	  8	 0	 0	 0	  0	  0	  1	 0	 0	 9

2000			13			  2	   1	  0	  2	 0	 0	 0	  0	  0	  0	 0	 0	 2

2001			13			  0	   0	  0	  1	 0	 0	 0	  0	  0 	  0	 0	 0	 2

2002			13			  0	   0	  0	  2  	 0	 0	 0	  0	  0	  0	 0	 0	 2

2003			13			  0	   0	  0	 4	 0	 0	 0	  0	  0	  0	 0	 0	 2

2004			13		0	  2	   0	  0	N/A	 0	 0	 0	  0	  0	N/A	 3	N/A	N/A

2005			8		0	  2           0         0	N/A	 0	 1	 0	  0	  0	N/A	 2	N/A 	N/A  	0

2006			8		0	  2	   0	  0	N/A	 0	 0	 0	  0	  0	N/A	 1	N/A 	N/A  	0

2007			9		0	  2	   0	  0	N/A	 0  	 1	 0	  0	  0	N/A	 4	N/A 	N/A  	0

2008			22		1	  11	   0	  2	N/A	 0  	 1	 0	  0	  0	N/A	 3	N/A 	N/A  	0

2009			12		0	  6	   0         0	N/A	 0	 1	 0	  0	  0	N/A	 4	N/A 	N/A  	0

2.5      	General assessment of position

Results reflect problems at the plant in 2009. Overall quality is not good. An alternative supply is being put in place for 2010.

Bailieborough Regional

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Bailieborough |

SOURCES:		SW: Surface water;

TREATMENT:	RGF: Rapid Gravity filtration, Chlor: Chlorination

Coag: Coagulation/Flocculation, pHC: pH Correction

DAF: Dissolved Air Flotation	, Fluor:	Fluoridation

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Bailieborough | 1950 | 2925 | 2 | 10 | # 2 | # 11 |  |

2.3      Details of exceedances of regulations by parameter

_____________________________________________________________________________________

Parameter		MAC			Excess values recorded		Number of values

______________________________________________________________________________________________________

Lead			25µg/l			{55.7,}					1

2.4     	Analysis of exceedances by parameter.

____________________________________________________________________________________________________________________

Year			No. of Samples____________________Analysis of Exceedances by Parameter__________________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu	NH4

___________________________________________________________________________________________________________________________________________

1999			13			0	0	0	0	0	0	0	0	0	0	0	0	0

2000			14			0	0	1	0	0	0	0	0	1	0	0	0	0

2001			13 			0	0	0	0	1	0	0	0	0	0	0	0	0

2002			26			0	0	0	0	6	0	0	0	0	0	3	0	1

2003			45			0	0	3	0	6	0	0	0	0	3	0	2	1

2004			34		0	0	0	0	N/A	0	0	2	0	0	N/A	0	N/A	N/A	0

2005			22		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			16		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			14		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			13		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2009			13		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5	General assessment of position

Satisfactory.  Exceedances for lead relates to internal plumbing in private property.

Ballinagh

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Ballyjamesduff/Kilnaleck/Ballinagh |

SOURCES:		SW: Surface water;

TREATMENT:	RGF: Rapid Gravity filtration 	Coag: Coagulation/Flocculation

DAF: Dissolved Air Flotation		Chlor: Chlorination

pHC: pH Correction			Fluor:	Fluoridation

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Ballinagh | See BJD | 840 | 1 | 4 | # 1 | # 6 | # 2 |

2.3      Details of exceedances of regulations by parameter

__________________________________________________________________________________

Parameter			MAC		Excess values recorded		Number of values

__________________________________________________________________________________________________

Pesticides			0.1 mg/l		{0.246}				1

THMs				100ug/l		{100.6}				1

2.4     	Analysis of exceedances by parameter.

___________________________________________________________________________________________________________________

Year			No. of Samples____________________Analysis of Exceedances by Parameter__________________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			15			0	0	0	11	0	0	0	0	0	0	0	0	11

2000			18 			0	0	7	14	0	0	0	0	0	0	0	0	15

2001			16			0	0	2	8	0	0	0	0	0	1	0	1	10

2002			15			0	1	0	11	0	0	0	0	0	0	0	0	13

2003			14			0	0	0	10	0	0	0	0	0	5	0	4	1

2004			12		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2005			8		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			8		0	0	0	0	N/A	0	1	1	0	0	N/A	0	N/A	N/A	0

2007			15		0	0	0	1	N/A	0	1	1	0	0	N/A	0	N/A	N/A	0

2008			7		0	0	0	1	N/A	0	1	0	0	0	N/A	0	N/A	N/A	0

2009			9		0	0	1	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5      	General assessment of position

This agglomeration is now supplied by Ballyjamesduff regional water supply scheme which has resulted in improved water quality.

The THM exceedance is marginal. See note at rear of document in relation to pesticides.

Ballyconnell

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Ballyconnell |

SOURCES:		SSp: Surface Springs

TREATMENT: 	Chlor: Chlorination

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Ballyconnell | 700 | 1220 | 1 | 4 | # 1 | # 8 | # 3 |

2.3      Details of exceedances of regulations by parameter

_________________________________________________________________________________

Parameter		MAC			Excess values recorded		Number of values

_________________________________________________________________________________________________

Total Coliforms	0/100mls		{3}						1

2.4     	Analysis of exceedances by parameter.

___________________________________________________________________________________________________________________

Year			No. of Samples____________________Analysis of Exceedances by Parameter__________________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

## ___________________________________________________________________________________________________

1999				13		0	0	0	1	0	0	0	0	0	5	0	5	2

2000				14		0	0	0	1	0	0	0	0	0	4	0	3	1

2001				13		0	0	0	0	0	0	0	0	0	3	0	3	0

2002				17		0	0	0	0	0	0	0	0	0	2	0	2	6

2003				13		0	0	0	1	0	0	0	0	0	0	0	0	1

2004				12	0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2005				10	0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006				10	0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007				8	0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008				7	0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2009				12	0	0	0	1	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5      	General assessment of position

Satisfactory. Single bacteriological exceedance was a once off and not typical.

Ballyhaise

  - Details of source treatment facilities

Ballyhaise is now supplied by Annagh GWS.

SOURCES:			SW: Surface water;

TREATMENT:		RGF: Rapid Gravity filtration, Coag: Coagulation/Flocculation

Chlor: Chlorination,  pHC: pH Correction,  SED: Sedimentation

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement**. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Ballyhaise | Annagh GWS | 620 | 1 | 4 | # 1 | # 4 | # 0 |

**Ballyhaise is supplied by the Annagh Group Water scheme which was monitored separately.

2.3      Details of exceedances of regulations by parameter

_________________________________________________________________________________________

Parameter				MAC			Excess values recorded		Number of values

___________________________________________________________________________________________________________

THM					100ug/l		{154.3}				1

Bromate				10ug/l			{30.7}					1

2.4     	Analysis of exceedances by parameter.

___________________________________________________________________________________________________________________

Year			No. of Samples__________________________Analysis of Exceedances by Parameter____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			15			9	0	3	7	0	0	0	0	0	4	0	4	4

2000 			15			2	0	1	1	0	0	0	0	0	5	0	5	1

2001			13			7	0	0	1	0	0	0	0	0	3	0	3	5

2002			21			3	0	3	1	0	0	0	0	0	9	2	8	3

2003			20			7	0	0	2	0	0	0	0	0	4	0	4	3

2004			12		  0	1	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2005			18		  0	0	0	1	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			34		  0	0	0	9	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			5		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			4		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2009			5		  0	0	1	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5      	General assessment of position

The bromate exceedance was treatment related and has been resolved. The THM issue is under investigation.

Ballyjamesduff Regional

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Ballyjamesduff/Kilnaleck |

SOURCES:		SW: Surface water;

TREATMENT:	RGF: Rapid Gravity filtration 	Coag: Coagulation/Flocculation

DAF: Dissolved Air Flotation		Chlor: Chlorination

pHC: pH Correction			Fluor:	Fluoridation

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Ballyjamesduff/ | 1950 | 2000 | 2 | 10 | # 2 | # 15 | # 3 |

2.3 	Details of exceedances of regulations by parameter

_________________________________________________________________________________

Parameter			MAC			Excess values recorded	Number of values

_________________________________________________________________________________________________

Pesticides			.1 ug/l			{0.168, .246}				2

Aluminium			0.2mg/l		{0.25}					1

2.4     	Analysis of exceedances by parameter.

____________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter_____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			14			0	0	0	0	3	0	0	0	0	4	0	4	1

2000			25			0	0	0	0	1	0	0	0	0	2	0	2	0

2001			24			0	0	0	0	2	0	0	0	0	5	0	5	1

2002			25			0	0	0	1	7	1	1	0	0	1	0	1	0

2003			26			1	1	0	3	5	0	0	0	0	1	0	0	1

2004			32		0	0	0	0	N/A	2	0	0	0	0	N/A	3	N/A	N/A	0

2005			19		0	0	0	2	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			15		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			14		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			13		0	0	0	0	N/A	0	0	1	0	0	N/A	0	N/A	N/A	0

2009			20		0	1	0	0	N/A	0	0	1	0	0	N/A	0	N/A	N/A	0

  - General assessment of position

Generally satisfactory. Aluminium exceedance is marginal. See note at rear of document in relation to pesticides.

Bawnboy

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Bawnboy |

SOURCES:		GW: Ground Water;

TREATMENT: 	Chlor: Chlorination

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Bawnboy | 256 | 286 | 1 | 4 | # 1 | # 4 | # 0 |

2.3  	Details of exceedances of regulations by parameter

_________________________________________________________________________________

Parameter		MAC			Excess values recorded	Number of values

_________________________________________________________________________________________________

None

2.4     	Analysis of exceedances by parameter

___________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

____________________________________________________________________________________________________________________________________

1999			29			0	0	11	8	0	0	0	0	0	2	0	1	21

2000			26			0	0	5	10	0	0	0	0	0	0	0	0	20

2001 			13			0	0	0	2	0	0	0	0	0	2	0	2	5

2002			14			0	0	0	5	0	0	0	0	0	0	0	0	9

2003			20			0	0	2	13	0	0	0	0	0	0	0	0	11

2004			13		0	0	0	0	N/A	0	1	0	0	0	0	0	N/A	N/A    0

2005			8		0	0	0	0	N/A	0	0	0	0	0	0	0	N/A	N/A	0

2006			9		0	0	0	0	N/A	0	0	0	0	0	0	0	N/A	N/A	0

2007			7		0	0	0	0	N/A	0	0	0	0	0	0	0	N/A	N/A	0

2008			5		0	0	0	0	N/A	0	0	0	0	0	0	0	N/A	N/A	0

2009			5		0	0	0	0	N/A	0	0	0	0	0	0	0	N/A	N/A	0

2.5      	General assessment of position

Satisfactory.

Belturbet

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Belturbet |

SOURCES:		SW: Surface water;

TREATMENT:	RGF: Rapid Gravity filtration		Coag: Coagulation/Flocculation

DAF: Dissolved Air Flotation		Chlor: Chlorination

pHC: pH Correction			Fluor:	Fluoridation

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Belturbet | 720 | 1774 | 1 | 4 | # 1 | # 4 | # 0 |

2.3 	Details of exceedances of regulations by parameter

_________________________________________________________________________________

Parameter		MAC			Excess values recorded		Number of values

_________________________________________________________________________________________________

Aluminium		0.2 mg/l			{0.208}					1

Ammonia		0.3 mg/l			{0.311}					1

Pesticides		0.1 ug/l			{0.106}					1

Fluoride		0.6-0.8				{0.82}						1

2.4     	Analysis of exceedances by parameter

___________________________________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter_____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			18			2	0	3	0	0	0	0	0	0	0	0	2	2

2000			13			9	0	0	0	2	0	0	0	0	0	0	0	0

2001 			14			9	0	0	0	0	0	0	0	0	0	0	0	2

2002			14			6	1	0	0	1	0	0	0	0	0	0	0	1

2003			16			5	1	0	1	3	0	0	0	0	1	0	1	1

2004			15		 0	2	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2005			10		0	3	0	0	N/A	0	0	1	0	0	N/A	0	N/A	N/A	0

2006			10		0	1	0	0	N/A	1	0	0	0	0	N/A	0	N/A	N/A	0

2007			10		0	3	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			5		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2009			5		0	1	0	0	N/A	1	0	0	0	0	N/A	0	N/A	N/A	1

  - General assessment of position

Generally satisfactory water , produced from a difficult raw water source, all exceednaces are marginal. See note at rear of document in relation to pesticides.

Blacklion

2.1	Details of source treatment facilities

Blacklion is now supplied by Gowlan GWS.

2.2	Sampling programme

| Supply | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Audit | Check | # Audit | # Check | # Operational |
| Blacklion | 250 | 1 | 4 | # 1 | # 4 | # 0 |

2.3 	Details of exceedances of regulations by parameter

_________________________________________________________________________________

Parameter		MAC			Excess values recorded	Number of values

_________________________________________________________________________________________________

None

2.4     	Analysis of exceedances by parameter

___________________________________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter_____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

____________________________________________________________________________________________________________________________________________

1999			13			0	0	0	0	0	0	0	0	0	4	0	4	0

2000			13			0	0	0	0	0	0	0	0	0	3	0	3	0

2001			13			0	0	0	0	0	0	0	0	0	0	0	0	0

2002			14			0	0	0	1	0	0	0	0	0	0	0	0	0

2003			14			0	1	0	0	0	0	0	0	0	0	0	0	0

2004			12       		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A     0

2005			8		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			10		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			5		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			5		  0	0	0	0	N/A	0	1	0	0	0	N/A	0	N/A	N/A	0

2009			5		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5      	General assessment of position

Satisfactory

Cavan Regional

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Cavan/Butlersbridge |

SOURCES:		SW: Surface water;

TREATMENT: 	RGF: Rapid Gravity filtration Coag: Coagulation/Flocculation

Chlor: Chlorination	pHC: pH Correction

Fluor:	Fluoridation 	SED: Sedimentation

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Cavan | 4500 | 7525 | 3 | 19 | # 3 | # 20 | # 4 |

2.3 	Details of exceedances of regulations by parameter

_______________________________________________________________________________________________

Parameter			MAC			Excess values recorded			Number of values

__________________________________________________________________________________________________________________

Aluminium			0.2mg/l		{0.375}						1

Manganese			0.50mg/l		{0.65}							1

Pesticides			0.1ug/l			{0.53,0.18}						2

THM				100ug/l		{103.3}						1

Fluoride			0.6-0.8			{0.83,0.85,0.86,0.88,0.89,0.9}			6

2.4     	Analysis of exceedances by parameter

____________________________________________________________________________________________________________________

Year			No. of Samples	____________________Analysis of Exceedances by Parameter_____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			29			16	0	0	 0	11	0	0	0	0	0	2	0	3

2000 			36			6	0	0	 0	3	0	0	0	0	0	0	0	2

2001			38			10	0	0	 0	0	0	0	0	0	0	0	0	4

2002			50			19	0	6	4	10	0	1	0	0	0	0	0	9

2003			45			13	1	0	3	8	0	1	0	0	0	0	0	3

2004			50		0	18	0	0	N/A	0	0	3	0	0	N/A	0	N/A	N/A

2005			29		0	7	0	0	N/A	1	0	0	0	0	N/A	0	N/A	N/A

2006			25		0	4	0	0	N/A	2	0	2	0	0	N/A	1	N/A	N/A

2007			25		0	1	0	0	N/A	1	0	2	0	0	N/A	1	N/A	N/A

2008			32		0	9	0	2	N/A	1	0	3	0	0	N/A	0	N/A	N/A

2009			27		0	1	1	0	N/A	0	0	1	0	0	N/A	0	N/A	N/A

2.5      	General assessment of position

Manganese exceedance is source related. Fluoride and THM exceednaces are marginal. See note at rear of document in relation to pesticides.

Cootehill

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Cootehill |

SOURCES:		SW: Surface water;

TREATMENT: 	RGF: Rapid Gravity filtration, Coag: Coagulation/Flocculation

Chlor: Chlorination	pHC: pH Correction

Fluor:	Fluoridation 	SED: Sedimentation

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Cootehill | 500 | 1900 | 1 | 4 | # 1 | # 4 | # 1 |

2.3 	Details of exceedances of regulations by parameter

_________________________________________________________________________________

# Parameter			MAC			Excess values recorded	Number of values

_________________________________________________________________________________________________

Aluminium mg/l		0.200			{1.600}				1

2.4     	Analysis of exceedances by parameter

_________________________________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			13			5	0	0	0	0	0	0	0	0	0	0	0	0

2000 			13			2	0	0	0	1	0	0	0	0	0	0	0	1

2001			13			3	0	0	0	4	0	0	0	0	0	0	0	0

2002			14			6	0	0	0	3	0	0	0	0	0	0	0	0

2003			13			1	0	0	1	3	0	0	0	0	0	0	0	0        	      2004		            14		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A

2005                            12		0	1	0	2	N/A	1	0	0	0	0	N/A	0	N/A	N/A	0

2006			8		0	0	0	0	N/A	0	0	1	0	0	N/A	0	N/A	N/A	0

2007			10		0	0	0	1	N/A	0	0	1	0	0	N/A	0	N/A	N/A	0

2008			5		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5      General assessment of position.

Generally satisfactory. Aluminium excedance was as a result of network maintenance being undertaken at the time of sampling.

Dowra

2.1	Details of source treatment facilities

Scheme is now supplied by Doobally GWS.

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2008 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Dowra | 80 | 228 | Discretionary | 2 | # 1 | # 3 | # 9 |

2.3 	Details of exceedances of regulations by parameter

___________________________________________________________________________________________________

Parameter			MAC				Excess values recorded			Number of values

_______________________________________________________________________________________________________________________

Aluminium(mg/l)		0.200				{0.382, 0.250, 0.41,0.406 }				4

2.4     	Analysis of exceedances by parameter

____________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter_____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			13			0	0	0	8	0	0	0	0	0	0	0	0	10

2000 			13			0	0	0	9	0	0	0	0	0	0	0	0	11

2001 			13			0	1	0	6	0	0	0	0	0	0	0	0	11

2002			36			0	1	18	20	0	0	0	0	0	0	6	0	26

2003			29			0	0	2	10	0	0	0	0	0	0	21	0	2

2004			13		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A     0

2005			12		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			15		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			9		  0	2	0	0	N/A	0	0	0	0	0	N/A	1	N/A	N/A	0

2008			15		  0	5	0	2	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			13		  0	4	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

  - General assessment of position.

Unsatisfactory. Dowra is now supplied by Doobally GWS. Upgrading took place in 2008. Aluminum exceedances are related to both raw water and process problems.

Gowna

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Gowna |

SOURCES:		SW: 	Surface water;

TREATMENT: 	PrF: 	Pressure Filtration	Chlor: Chlorination

pHC: 	pH Correction

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Gowna | 280 | 350 | 1 | 4 | # 1 | # 4 | # 0 |

2.3 	Details of exceedances of regulations by parameter

_______________________________________________________________________________________

Parameter			MAC			Excess values recorded		Number of values

_________________________________________________________________________________________________________

None

2.4 	Analysis of exceedances by parameter

____________________________________________________________________________________________________________________

Year		No. of Samples_________________________Analysis of Exceedances by Parameter_____________________________

Bromate	Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			22			0	0	7	18	0	0	0	0	0	5	0	1	18

2000 			14			0	0	1	8	0	0	0	0	0	3	0	3	12

2001			16			0	0	1	7	0	0	0	0	0	4	0	4	14

2002			22			0	1	7	5	0	0	0	0	0	6	0	6	17

2003			13			0	0	0	6	0	0	0	0	0	1	0	1	2

2004			13	    1	  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2005			9	    0	  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			6	    0	  0	0	1	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			8	    0	  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			4	    0	  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2009			5	    0	  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5      	General assessment of position

Satisfactory.

Killeshandra

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Killeshandra |

SOURCES:		SW: Surface water;

TREATMENT:  	PrF: Pressure Filtration, Chlor: Chlorination

pHC: pH Correction,

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Killeshandra | 320 | 420 | 1 | 4 | # 1 | # 6 |  |

2.3	Details of exceedances of regulations by parameter

_________________________________________________________________________________

Parameter		MAC			Excess values recorded		Number of values

_________________________________________________________________________________________________

None

  - Analysis of exceedances by parameter

____________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter_____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			19			0	0	3	6	0	0	0	0	0	4	0	3	14

2000 			14			0	1	0	1	0	0	0	0	0	2	0	2	4

2001 			16			0	1	2	2	0	0	0	0	0	3	0	3	9

2002			20			0	1	4	4	0	0	0	0	0	5	0	5	17

2003			17			0	1	2	6	0	0	0	0	0	8	0	8	3

2004			16		  0	0	0	1	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2005			10		  0	0	0	0	N/A	0	0	1	0	0	N/A	0	N/A	N/A	0

2006			9		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			5		  0	0	0	0	N/A	0	0	1	0	0	N/A	0	N/A	N/A	0

2008			5		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2009			7		  0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5      	General assessment of position

Generally satisfactory.

Kingscourt

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Kingscourt |

SOURCES:		SW: Surface water;	GW: Ground Water;	Ssp: Surface Springs

TREATMENT: 	Sed: Sedimentation; RGF: Rapid Gravity filtration

Coag: Coagulation/Flocculation  Chlor: Chlorination Fluor:	Fluoridation

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2009 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Kingscourt | 700 | 1748 | 1 | 4 | # 1 | # 6 | # 1 |

2.3   	Details of exceedances of regulations by parameter

____________________________________________________________________________________________

Parameter			MAC			Excess values recorded			Number of values

_______________________________________________________________________________________________________________

Aluminium			0.2 mg/l 		{0.463}						1

Iron				0.2 mg/l		{0.281}						1

  - Analysis of exceedances by parameter

___________________________________________________________________________________________________________________

Year			No. of Samples____________________Analysis of Exceedances by Parameter__________________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

__________________________________________________________________________________________________________________________________________

1999			14			2	0	0	0	2	0	0	0	0	2	0	2	1

2000			15			2	0	0	0	2	0	0	0	0	2	0	3	1

2001			18			3	0	2	0	3	0	0	0	0	1	0	1	1

2002			21			4	0	1	1	1	0	0	0	0	5	0	5	2

2003			17			8	0	1	3	2	0	0	0	0	0	0	0	2

2004			13		0	1	0	0	N/A	1	0	0	0	0	N/A	0	N/A	N/A	0

2005			8		0	1	0	0	N/A	0	0	0	0	0	N/A	1	N/A	N/A	0

2006			8		0	1	0	0	N/A	1	0	0	0	0	N/A	1	N/A	N/A	0

2007			7		0	1	0	0	N/A	1	0	0	0	0	N/A	1	N/A	N/A	0

2008			6		0	2	0	0	N/A	0	0	0	0	0	N/A	1	N/A	N/A	0

2008			8		0	1	0	0	N/A	0	1	0	0	0	N/A	1	N/A	N/A	0

2.5      	General assessment of position

Iron exceedance is source related.

Shercock

2.1	Details of source treatment facilities

Shercock is now supplied by Dhuish GWS

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2008 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Shercock | 180 | 650 | 1 | 4 | # 1 | # 4 | # 0 |

2.3 	Details of exceedances of regulations by parameter

_________________________________________________________________________________________________________

Parameter			MAC			Excess values recorded		Number of values

__________________________________________________________________________________________________________

None

2.4    	Analysis of exceedances by parameter

___________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter_____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

__________________________________________________________________________________________________________________________________________

1999			13			0	0	0	4	0	0	0	0	0	2	0	1	7

2000			13			0	0	0	2	0	0	0	0	0	3	0	3	3

2001			13			0	0	0	1	0	0	0	0	0	2	0	2	1

2002			13			0	0	0	3	0	0	0	0	0	1	0	1	4

2003			13			0	0	0	7	0	0	0	0	0	0	0	0	0

2004			12		 0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A     0

2005			11		0	0	0	1	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			7		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			7		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			5		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2009			5		0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2.5      	General assessment of position

# Satisfactory

Swanlinbar

2.1	Details of source treatment facilities

| Source | Details of Treatment | Name of Supply |
| --- | --- | --- |
| SW/GW/SSp | Sed/SSF/RGF/PrF/Coag/DAF/Chlor/Fluor/pHC | Swanlinbar |

SOURCES:		GW: Ground water;

TREATMENT: 	RGF: Rapid Gravity filtration Coag: Coagulation/Flocculation

Chlor: Chlorination	pHC: pH Correction

2.2	Sampling programme

| Supply | Volume Produced (M3/day) | Population Served | Minimum number of samples per year, Legal Requirement. |  | Number of samples monitored in 2008 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Audit | Check | # Audit | # Check | # Operational |
| Swanlinbar | 130 | 315 | 1 | 4 | # 1 | # 4 | # 0 |

2.3 	Details of exceedances of regulations by parameter

_________________________________________________________________________________________________

Parameter		MAC			Excess values recorded	Number of values

_________________________________________________________________________________________________

Aluminium		0.2 mg/l 		{0.466}				1

Iron			0.2 mg/l		{0.659}				1

pH			6.5-9.5			{6.4}					1

2.4    	Analysis of exceedances by parameter

____________________________________________________________________________________________________________________

Year			No. of Samples_________________________Analysis of Exceedances by Parameter_____________________________

Clo	Al	THM	Coli	Co	F	Fe	Mn	NO3	NO2	Od 	pH	Ta	Tu    NH4

___________________________________________________________________________________________________________________________________________

1999			17			0	0	3	12	0	0	0	2	0	1	0	1	12

2000 			14			3	0	0	0	0	0	0	0	0	3	0	3	5

2001 			13			0	0	0	0	0	0	0	0	0	6	0	6	0

2002			14			1	0	0	0	0	0	0	0	0	1	0	1	0

2003			14			0	0	0	1	0	0	0	0	0	0	0	0	0

2004			12		 0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A     0

2005			7		 0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2006			9		 0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2007			7		 0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2008			5		 0	0	0	0	N/A	0	0	0	0	0	N/A	0	N/A	N/A	0

2009			5		 0	1	0	0	N/A	0	1	0	0	0	N/A	1	N/A	N/A	0

2.5	General assessment of position

# pH exceedance is marginal. This spring water source fluctuates in quality as a result of rainfall

3.	Summary of exceedances for each parameter

___________________________________________________________________________________________________

Parameter					Supply Name		Excess values recorded		Number of values

____________________________________________________________________________________________________

Aluminium					Ballyjamesduff	{0.25}						1

Aluminium					Belturbet		{0.208}					1

Aluminium					Cootehill		{1.600}					1

Aluminium(mg/l)				Dowra			{0.382, 0.250, 0.41,0.406 }			4

Aluminium					Kingscourt		{0.463}					1

Aluminium					Swanlinbar 		{0.466}					1

Aluminium(mg/l)				Arvagh			{0.342, 0.552,0.76, 0.47, 0.49, 0.328}	6

Aluminium					Cavan			{0.375}					1

Ammonia					Belturbet		{0.311}					1

Bromate					Ballyhaise		{30.7}						1

Fluoride					Belturbet		{0.82}						1

Fluoride					Cavan			{0.83,0.85,0.86,0.88,0.89,0.9}		6

Iron 	(mg/l)					Arvagh			{0.443}					1

Iron						Kingscourt		{0.281}					1

Iron						Swanlinbar		{0.659}					1

Lead     (ug/l)					Arvagh			{59.3}						1

Lead						Bailieborough		{55.7,}						1

Manganese					Cavan			{0.65}						1

Pesticides					Arvagh			{0.17}						1

Pesticides					Ballinagh		{0.246}					1

Pesticides					Ballyjamesduff	{0.168, .246}					2

Pesticides					Belturbet		{0.106}					1

Pesticides					Cavan			{0.53,0.18}					2

pH						Swanlinbar		{6.4}						1

pH (pH units)					Arvagh			{5.89, 5.96, 6.37, 6.4}				4

THMs						Ballinagh		{100.6}					1

THM						Ballyhaise		{154.3}					1

THM						Cavan			{103.3}					1

Total Coliforms				Ballyconnell		{3}						1

Aluminium

Aluminium Sulphate (Alum) is widely used in water treatment for colour and colloid removal. A total of 143 samples were analysed in 2009 for Aluminium in public water. The overall percentage compliance rate for 2009 was 88.1% the compliance rate in 2008 was 81.25% the 2007 and 2006 rate was 90% compared to the 2005 figure of 89% the 2004 figure of 87% the 2003 figure of 81% and the 2002 figure of 80%, and the 2001 figure of  73%.

Antimony

Antimony is a toxic heavy metal. The most common source of antimony in drinking water appears to be dissolution from metal plumbings and fittings. There were no exceedances in Cavan in 2008

Bromate

This occurs when bromide ions present in water are oxidised either by ozone or chlorine. There were no exceedances in Cavan in 2009. Commercial sodium hypochlorite used in the disinfection process has been found to contain Bromate as a contaminant, the use of this hypochlorite has been phased out by Cavan County Council.

Clostridia Perfringens

Clostridia perfringens are used as indicators of faecal pollution. Their spores survive much longer than organisms of the coliform group and they are more resistant to disinfection. They are used primarily as an indicator of other pathogenic organisms such as Cryptosporidium. There was no exceedances in 2009.

E Coli

# E Coli are bacteria which originate in human and animal wastes; they are used as an indicator organism. Their presence is taken as proof that faecal contamination has occurred and therefore there is a definite indication that pathogens may be present. The absence of these bacteria indicate that it is unlikely or there is a strong probability that these pathogens are not present in the drinking water sample. The exact safety factor depends on the ratio of pathogens to E Coli.  In practice in order to achieve a high margin of safety, zero is accepted as the limit for this parameter.

A total of 140 samples were analysed in 2009 for E Coli in public water. The overall percentage compliance rate was 100%. This follows the recent trend in 2008 the rate of compliance was 97.8% with three exceedances. This compares to 2005 through to 2007 which all had just one exceedance per annum. The general trend in compliance for E Coli is one of improvement with a compliance rate in 1999 of 90.9%, in 2000 of 93.8% and in 2001 of 97 %. There was a dip in compliance for 2002 at 93.9 which can be attributed to a situation at Dowra. The percentage compliance increased again for 2003 at 98.4% and in 2004 was 100%.

Total Coliforms

# Total Coliforms are coliforms which arise from both faecal and nonfaecal sources. If the E Coli are not present then there may still be total Coliforms present. These originate from various sources including soil.  Therefore, if total coliforms are present, without the presence of E Coli, it indicates that the pollution is either recent and of non-faecal nature or of remote faecal origin such that the intestinal coliforms have not survived. A total of 140 samples were analysed in 2009 for Total Coliforms in public water. The overall percentage compliance was 99.3% This compares well with 2008 which had a compliance rate of 95%. The overall percentage compliance rate for 2007 was 98.7%  for 2006 it was 95.6%.  In 2005 the compliance rate was 97.4% and for 2004 was 99.7% which represented an improvement on the 2003 figure of 96.2% and the 2002 figure of  88.8%.

Colour

# Colour arises as a result of organic molecules derived from humic matter. The new EU drinking water directive has not included a limit for colour. This allows the consumers to make their own decision on what is and isn’t acceptable.

Fluoride

Fluoride arises as a result of Fluoridation of water supplies. A total of 85 samples were analysed in 2009 for Fluoride in public water. The overall percentage compliance rate was 91.8%    There were a total of seven exceedances and all exceedances were marginal The EU MAC limit for fluoride is 1.5mg/l, no sample exceeded this level in 2009.

Iron

Iron is generally present as a result of geological formations and is widespread in ground water in County Cavan. However, the majority of Public water sources are surface waters rather than ground waters. In general more appreciable amounts of Iron will be found in ground water. A total of 21 samples were analysed in 2009 for Iron in public water supplies and there was three exceedances .

Lead

Lead is frequently present in pipes within older houses and is present in some old water networks. There is a danger that lead is brought into solution from these old pipes. Lead bioaccumulates in body tissue. There wer two exceedances for lead in county Cavan in 2009.

Manganese

Manganese is generally present in groundwater in County Cavan. Its toxicity is not a factor in drinking water as it will be rejected by the consumer before any danger threshold is reached.  A total of 21 samples were analysed in 2009 for Manganese in public water. There were single low  exceedances recorded in Cavan.

Nitrate

# Nitrate primarily arises from organic and inorganic sources as opposed to mineral sources. Unlike some other counties, Nitrate is not a problem in public water supplies in County Cavan. A total of 21 samples were analysed in 2009 for Nitrate in public water. The overall percentage compliance rate was 100%.

Nitrite

# Nitrite is an indicator of possible pollution. It is also an intermediate between Ammonia and Nitrate. A total of 21 samples were analysed in 2009 for Nitrite in public water. The overall percentage compliance rate was 100%.

Odour/Taste

# Odour and Taste arise from a variety of sources. The new regulations do not specify a specific MAC (Maximum Admissable Concentration) but is subject to being ‘deemed acceptable by the consumer.’ The primary source of odour/taste in public water in County Cavan is due to chlorination. Taste and Odour are assessed by an individual analyst and therefore the results are subjective.

Pesticides

Although the presence of a number of pesticides has been detected all of the individual pesticides exceedances relate to MCPA .This is a weed killer known colloquially as ‘rush killer’. As it is used to kill rushes, the land will tend to be water logged suggesting a shorter pathway to watercourses/abstraction lakes. The exceedances found are significantly lower than the limit advised by the WHO, and therefore are not considered a significant cause for concern. Given the prevalence of this weedkiller and the fact that, according to the EPA in Monaghan it is not simply confined to County Cavan, the HSE has suggested that a national approach should be taken in conjunction with the EPA. This appears to be appropriate. There is some concern that if the solution is to withdraw this weedkiller from the market that an alternative weedkiller may present similar or greater problems

pH

# pH reflects the acidity or basicity of water. pH governs the behaviour of other aspects such as Ammonia toxicity, Chlorine disinfection efficiency and metal solubility. A total of 133 samples were analysed in 2009 for pH in public water. The overall percentage compliance rate was 96.3%. Four of the five exceedances occurred in Arvagh. In 2008 the rate was 97.1%. In 2007 the compliance rate was 98%, in 2006 the compliance rate was 98.5% which was the same compliance rate as that for 2005 and an improvement on the 98% in 2004 and the 94% compliance achieved in 2003.

TriHaloMethanes(THMs)

# THMs are toxic chemicals formed as byproducts of the disinfection by chlorination of water containing organic matter. As a general rule the more coloured the water the greater the level of organic material and therefore on chlorination the greater the levels of THM’s. A total of 21 samples were analysed in 2009 for THM’s in public water. There were three exceedances, two very low exceednaces of <4% and an exceedance for Ballyhaise which is supplied by the Annagh GWS. The MAC in the regulations was 150ug/l and reduced  to 100ug/l for 2009

Turbidity

# Turbidity arises in water due to the presence of very fine particles of solids which are not filterable. Due to the nature of the raw water sources, the geology of County Cavan, and the level of treatment required, turbidity can be a problem in County Cavan. There is no longer a specific limit but it must be ‘deemed acceptable by the consumer with no abnormal change’.