Comhairle Chontae an Cabhain

CAVAN  COUNTY  COUNCIL

THE QUALITY OF RIVER

& LAKE WATER IN COUNTY CAVAN

###### A Report for the Year 2009

J Keyes			E Doyle			Environment Section

County Manager		Director of Services	 	Cavan County Council

Environment			September 2010

Index

Page

- Preamble	1

- Introduction	8

- River Water Quality	11

- Lake Water Quality	50

© Cavan County Council 2009

All or part of this publication may be reproduced without further permission, provided the source is acknowledged.

# Preamble

Cavan County Council has a general statutory responsibility for the quality of river and lake water within its functional area. This responsibility is underpinned by legislation and in particular the Local Government (Water Pollution) Act, 1977 and 1990 and more recently The Water Framework Directive (WFD) (2000/60/EC), which came into force in December 2000 and establishes a framework for community action in the field of water policy. The WFD was transposed into Irish law by the European Communities (Water Policy) Regulations 2003 (S.I. 722 of 2003).

The WFD rationalises and updates existing legislation and provides for water management on the basis of River Basin Districts (RBDs). RBDs are essentially administrative areas for coordinated water management and are comprised of multiple river basins (or catchments), with cross-border basins (i.e. those covering the

territory of more than one Member State) assigned to an international RBD. All of the river basins on the island of Ireland have been distributed within eight RBDs. County Cavan is in four RBDs. This is more than any other county in Ireland.

The aim of the WFD is to ensure that waters achieve at least good status by 2015, unless there is an extension or derogation, and that existing status does not deteriorate in any waters. The mechanism to achieve good status and preserve our best waters, under the WFD, is through the adoption and implementation of River Basin Management Plans (RBMPs) for each of the identified RBDs. The River Basin Management Plan itself will be based on a detailed analysis of the significant pressures and impacts of human activity and on the status of surface waters and groundwater in the River Basin District, including,

- Point source pollution

- Diffuse source pollution

- Pressures on the quantitative status of water including abstractions

- Analysis of other impacts of human activity on water status.

A Characterisation Report, has been completed and the scientific assessment of the river basins that is contained in the Characterisation Report was used for the production of a "Programme of Measures". These consist of policies and strategies, such as monitoring programmes, which are intended to reduce the risk to water bodies and target improvements and monitoring effort on those water bodies most at risk of failing to meet their targets. The programme of measures associated with the RBMPs can be compared to a much more detailed and comprehensive version of the earlier Phosphorous Regulations ‘Measures Report’.

Environmental monitoring programmes were established in 2006/07. These monitoring programmes are the basis of Cavan County Councils surface water monitoring programme. The data from this progamme is used in this report. The RBMPs form the basis of Cavan County Council’s plan for addressing pollution of waters within the county arising from all sources.

This current publication contains the results of the council’s river and lake monitoring programme for the year 2009 and provides a brief overview of progress. It is noted that the classification system for water quality, in accordance with the WFD, has been finalised and these classifications are include in this report .

### WATER QUALITY ISSUES IN COUNTY CAVAN

### Eutrophication

Eutrophication is the term used to describe an increase in the concentration of plant nutrients, particularly phosphorus, in waters. As nutrient concentration increases, the potential growth of algae and other aquatic plants also increases.

Given sufficient light and suitable growing conditions, the maximum amount of plant material that will grow in a given water body is usually dependant on the availability of the nutrients, nitrogen and phosphorus. In most fresh waters, plant growth will lead to the depletion of phosphorus before all the nitrogen is used up. This means that the maximum amount of plant growth is effectively limited by the amount of phosphorus present, so phosphorus is said to be the growth-limiting nutrient.

It has been recognised that many human activities can add extra nutrients to waters and greatly speed up the natural process of eutrophication. During the last two to three decades, because of increased human activity, many lakes in the country have shown signs of increasing enrichment. Lakes differ in their individual capacity to absorb increased nutrient inputs, but shallow lakes as exist in Cavan are particularly sensitive.

Eutrophication itself is not intrinsically bad, but it can cause changes to the ecological balance of a water body which may interfere with human activities and reduce biodiversity. It is because of these consequential effects that eutrophication is often undesirable and needs to be controlled or reversed.

In lakes, the development of algal ‘blooms’ and shoreline scums, as well as a lowering of the transparency of the water, have adverse effects on recreational and amenity uses such as bathing and angling. Fish, especially the more sensitive salmonids, may be endangered by deoxygenation arising from the respiration of the large biomass of algae and other plants.

The main concern in eutrophic rivers and streams is the deoxygenation caused by the enhanced levels of plant respiration.

Lake eutrophication is a recognised problem in County Cavan. This is due to a number of factors. The economy of the County is largely agriculturally based, and this, coupled with sensitive waterways and very difficult soil conditions, has resulted in a more rapid upsurge in cultural eutrophication in Cavan than has been experienced in other parts of the country. However, this problem is manifesting itself nationally, in particular the Killarney, lower Shannon and western lakes.

# Sources of Phosphorous

In the past, in subsistence agricultural communities, the elements necessary for plant and animal growth were recycled. Crops were consumed by animals and man close to their place of production. The resulting animal and human manures, as well as crop wastes, were then applied to cultivated land, returning the nutrients to the soil. The recent development of intensive farming enterprises has broken this nutrient cycle and has largely replaced the closed system with a linear throughput system where nutrients can build up in the soil towards or beyond saturation level and eventually end up in rivers and lakes.

There is therefore a clear distinction between agricultural activities where the number of livestock is limited by the land available (closed system) and intensive units where no such limiting factor applies (linear system).  In the former, animal manures are used as a resource, while in the latter they can be regarded as a waste product.

All farming units can pose problems for water quality in Cavan. These problems relate more to a lack of proper facilities and poor management practices rather than to any “fatal flaw” and manifest themselves particularly in wet summers.

There are of course other problems. Phosphorus at point discharges can be very significant. Consequently, there is the need to upgrade Local Authority sewage treatment works and substantial public sector investment has already been committed to this. Discharges from all major industrial discharges need also to be addressed through consultation with the Environmental Protection Agency, where appropriate, or through the Local Authority’s discharge licencing system.

Agriculture, however, will in the medium term remain the dominant factor in deciding water quality and ultimately, no significant progress will be made unless a meaningful reduction in agricultural losses can be attained.

Single Dwelling Waste Water Issues

Cavan has a dispersed, rural population which results in the majority of domestic sewage being treated by wastewater treatment systems for single houses. In recognition of this and the problems associated with inadequate systems, Cavan County Council drafted bye-laws to ensure the proper assessment, operation and maintenance of all single house wastewater treatment systems in the county. Following a consultation period, the Water Pollution (Wastewater Treatment Systems for single Houses), Bye-Laws, 2004 were adopted on the 19th of April 2004 and came into effect on the 1st of July 2004. These Bye-Laws include for the assessment of all systems subject to the Bye-Laws and the provision of approved systems, if not already in place, by 31st December 2005. A panel of assessors was appointed to undertake these assessments as competent persons. Implementation of the Bye-laws is ongoing and improvement works have already been carried out to a large number of systems. This will ultimately lead to improvements in water quality.

Water Quality in County Cavan

Historically the classification of rivers in Ireland is based upon the results of biological monitoring carried out by the Environmental Protection Agency. A summary of the classification system is provided in table 1 below;

Table 1:  System of Water Quality Classification.

Biotic Index			Quality Status			Classification

Q5, Q4-5, Q4			Unpolluted Waters			Class A

Q3-4				Slightly Polluted Waters		Class B

Q3, Q2-3			Moderately Polluted Waters		Class C

Q2, Q1-2, Q1			Seriously Polluted Waters		Class D

Classification System and Beneficial Uses.

| Unpolluted Waters (Class A) | High quality waters suitable for supply and abstraction. Game fisheries and high amenity value. (Satisfactory). |
| --- | --- |
| Slightly Polluted Waters (Class B) | Usually good game fisheries. Suitable for supply. Moderate to high amenity value. (Transitional). |
| Moderately Polluted Waters (Class C) | ## Coarse fisheries. Not likely to support a healthy game fishery. Suitable for supply after advanced treatment. (Unsatisfactory). |
| Seriously Polluted Waters (Class D) | Fish absent or only sporadically present. May be used for low-grade industrial abstraction. Low amenity value. (Unsatisfactory). |

The above classification system was replaced by new Environmental Quality Standards(EQS). These were finalised in 2009. The new classification divides all waters into one of five classes, High, Good, Moderate, Poor and Bad. The classification of river stations is now complete. The new EQS are defined in S.I. No. 272 of 2009 the EUROPEAN COMMUNITIES ENVIRONMENTAL OBJECTIVES (SURFACE WATERS) REGULATIONS 2009. The Classification tables are defined in schedules 4 , 5 and 6 over 13 pages.

River Water Quality

Because the monitoring programme for the WFD is significantly different from that of the previously used Phosphorous Regulations (a decreased number of samples are required in the new monitoring programme), it is difficult to compare previous years data with the current data. However, a better understanding of the information may be obtained by comparing like with like. Therefore, ‘biological only’ data for Co. Cavan are shown below in table 1.

|  | Q 95-97 (%) | Q 98-00 (%) | Q 01-03 (%) | Q 02-04 (%) | Q 03-05 (%) | Q 04-06 (%) | Q 05-07 (%) | Q 06-08 (%) | Q 07-09 (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | 49.3 | 47.8 | 51.8 | 56.8. | 53.31 | 54 | 43.9 | 42.3 | 44.6 |
| Class B | 24.5 | 29.6 | 27.4 | 26.8 | 30.1 | 29.8 | 39.5 | 41.9 | 39.5 |
| Class C | 26.2 | 21.1 | 20.8 | 16.4 | 16.1 | 16.1 | 16.6 | 15.8 | 15.84 |
| Class D | 0 | 1.3 | 0 | 0.1 | <0.1 | <0.1 |  |  |  |

Table 1: Biological Only Data

As stated earlier, the biological data is supplied by the EPA based on surveys carried out at 3 year intervals.

Note : Statistics presented in river water quality section are based on EPA stations and additional  stations originally programmed in the 2001 river sampling programme. Statistics are based on river channel length

Lake Water Quality

## There are over three hundred lakes in County Cavan, each with their own individual ecology. Nineteen lakes are monitored four times a year by Cavan County Council. The additional resources required in lake water sampling, in comparison to river water sampling, has resulted in a representative lake sampling programme. These lakes were chosen for the monitoring programme by the EPA.

Although many of the inflowing streams to these lakes are showing reduced molybdate reactive phosphate (MRP) levels, this is not always reflected in a corresponding improvement in lake water quality. It is possible that there is internal loading taking place due to a reserve of phosphorous within the lake system (e.g. lake sediment) that is allowing the eutrophic conditions to prevail.

### Progress

The 1999 Phosphorous Measures Report was the Council’s original plan for addressing pollution of waters within the county arising from all sources. A new programme of measures will be implemented as part of the RBMPs. Ultimately, the effectiveness of Cavan County Councils actions will be judged by the improvements achieved in water quality. The monitoring results detailed herein suggest that although some progress has been made, the continued implementation of measures is necessary to achieve our objectives. It is hoped that by continuing this pro-active approach to water quality issues, the quality of rivers and lakes in County Cavan will continue to show improvement towards good status.

2.	Introduction

This report presents the summary monitoring data for 2009 for the rivers and lakes in County Cavan; it can be viewed as a companion volume to those previously

produced.

All chemical monitoring was undertaken by the Council’s Environment Section. Monitoring complies with the requirements of the monitoring programme prescribed by the EPA in compliance with the requirements of the WFD. Approximately 96 river stations are being monitored and 19 lakes are also monitored. The monitoring programme will allow comparison, on an annual basis, of the chemical quality of the rivers and lakes.

The Biological monitoring referred to in this report was undertaken by the EPA as part of its National Programme of Biological Assessment of River Water Quality.

The EPA’s contribution is gratefully acknowledged.

The lake monitoring programme involves the monitoring of 19 lakes designated by the EPA. The number of stations monitored on each lake varies depending on a number of factors including lake size, historical data and uniformity of water body. Samples are taken at time intervals specified by the EPA. This involves a minimum of four samples taken once in April and three times between July and October.

The new classification system of quality standards introduced in 2009 is defined in S.I. No. 272 of 2009 the EUROPEAN COMMUNITIES ENVIRONMENTAL OBJECTIVES (SURFACE WATERS) REGULATIONS 2009. The Classification tables are defined in schedules 4 , 5 and 6 over 13 pages.

In order to allow the reader assess the information regarding the quality of river water three tables are shown below.

Tables A and B set out the standards specified in the Local Government (Water Pollution) Act, 1977 (Water Quality Standards for Phosphorus), 1998 for rivers and lakes respectively. These regulations identify biological ratings and Molybdate-Reactive Phosphorous Annual Median concentrations for the rivers, and Chlorophyll and Total Phosphorous levels for the lakes, either of which must be at least maintained or improved in order to meet the relevant specified target standard.

Table C sets out the standard specified in EC directive 78/659/EEC (Freshwater Fish).

Table A: ‘Phosphorous Regs’, Local Government (Water Pollution) Act, 1977 (Water Quality  Standards for Rivers), 1998

| Existing Biological Quality Q Rating | Molybdate -Reactive Phosphorous annual Median concentration (mg/l P) | Target Biological Quality (Q) Rating |
| --- | --- | --- |
| 5 } 4-5 } Unpolluted 4 } | 0.015 0.020 0.030 | 5 4-5 4 |
| 3-4 }Slightly Polluted | 0.030 | 4 |
| 3 } Moderately 2-3 } Polluted | 0.05 0.070 | 3-4 3 |
| < 2 } Seriously Polluted | 0.070 | 3 |

Table B: ‘Phosphorous Regs’,Trophic Classification System for Lakes

| Lake Trophic Category | Target Trophic Status | Total Phosphorous Average Concentration (g P/l) | Annual Max. Chlorophyll (mg/m3) |
| --- | --- | --- | --- |
| Ultra-Oligotrophic | Ultra-Oligotrophic |  5 | < 2.5 |
| ### Oligotrophic | ### Oligotrophic | > 5  10 | > 2.5 < 8 |
| Mesotrophic | Mesotrophic | > 10  20 |  8 < 25 |
| Eutrophic | Mesotrophic | > 10  20 |  8 < 25 |
| Hypertrophic | Eutrophic | > 20  50 |  25 < 75 |

Table C: EC Directive 78/659/EEC (Freshwater Fish)

| Parameter | ### Units | Salmonid | ### Waters | ### Cyprinid | Waters |
| --- | --- | --- | --- | --- | --- |
|  |  | Guide | #### Mandatory | Guide | #### Mandatory |
| Total Ammonia | mg/l N | < 0.03 | < 0.78 | < 0.16 | < 0.78 |
| B.O.D. | mg/l O2 | 100% < 3 |  | 100% < 6 |  |
| D.O. | mg/l O2 | 50% > 9 | 50% > 9 | 50% > 8 | 50% > 7 |
| Total Phosphorous | mg/l P |  | 0.065 |  | 0.13 |
| PH | pH units |  | 6-9 |  | 6-9 |

3.	River Water Quality

Rivers are listed in alphabetical order.

River name and catchment: Included are the river name and the catchment name (namely: the Erne, the Boyne, the Shannon and the Glyde). Sub-catchments and water management units are not listed.

Sampling stations: Sampling stations are numbered according to the EPA assignment. In the case where the EPA has not assigned a sampling station number, no number is included. Stations are listed in a downstream order and include stations that are biologically assessed. It is not always possible to use the same location for both chemical and biological assessment, as the requirements for both are different. A brief location description for each station is also given. The water body code is given for each station, and there may be more individual stations in a single water body. The current ecological status as defined is given with target ecological status. The target ecological status cannot be lower than Good. A target date for reaching the target ecological status is included for each station.

Biological Quality Ratings: This report includes recent and historical data from the EPA’s National Biological River Monitoring Programme. The Q ratings for 2009 are provisional. The biotic index classification is listed in Table A.

MRP results

Median annual MRP results from 1999-2009 are included in the summary data.

#### Assessment

A summary assessment of the water quality is given.

Change from 2008

Any significant changes in quality from 2008 are also given.

Note: The analysis of MRP involves a specific procedure. Analytical procedures are designed to accurately detect concentration levels over a specific range. The upper and lower limits of this range are assigned as limits of quantification. The lower limit of quantification for MRP is assigned at 10µg P/l. In this section, if a result of  10µg P/l is recorded, this is reported as 10 µg P/l.

##### List of  Rivers Monitored

River 						Catchment

Annalee						Erne

Ballinagh						Erne

Bawnboy 						Erne

Blackwater (Kells)					Boyne

Blackwater (Newtowngore)			Erne

Blackwater (Swanlinbar)				Erne

Bunnoe						Erne

Cavan							Erne

Chapel Lake Stream				Boyne

Cullies						Erne

Dee							Glyde

Dromore						Erne

Derrada	Erne

Drumane						Erne

Drumkeery Lough stream			Boyne

Drumsallagh					Glyde

Erne							Erne

Glyde							Glyde

Inny							Shannon

Knappagh						Erne

Laragh						Erne

Lislea							Boyne

Maudabawn						Erne

Mountnugent					Shannon

Moynalty						Boyne

Nadregeel Lough Stream				Boyne

Owenayle						Shannon

Owenmore (Glangevlin)				Shannon

Owensallagh					Erne

Rag							Erne

Roo							Erne

Shannon						Shannon

Sheelin Feeder Streams				Shannon

Stradone						Erne

Swanlinbar						Erne

Templeport Lough Stream			Erne

Woodford						Erne

River: Annalee

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 2nd br. u/s L.Sillan | 0080 | NW_36_1947 | Moderate | Good | 2021 |
| 2nd br. d/s L. Sillan | 0150 | NW_36_1181 | Poor | Good | 2021 |
| 1st br. d/s L. Tacker | 0250 | NW_36_1072 | Good | Good | 2009 |
| Br. Nr Anns fort | 0350 | - NW_36_1102 | Moderate | Good | 2021 |
| New Grove Br. | 0500 | - NW_36_1102 | Moderate | Good | 2021 |
| Br SE of Fort William | 0600 | - NW_36_1102 | Moderate | Good | 2021 |
| Br.NW of Rakenny Ho | 0800 | NW_36_2417 | Poor | Good | 2021 |
| Ballynallon Br | 0900 | NW_36_2417 | Poor | Good | 2021 |
| Ballyhaise Br. | 1000 | NW_36_2417 | Poor | Good | 2021 |
| Br. Nr Curraghanoe | 1150 | NW_36_2417 | Poor | Good | 2021 |
| 0.2Km u/s Cavan R confl | 1350 | NW_36_2417 | Poor | Good | 2021 |
| 0.2km d/s Cavan R confl | 1400 | NW_36_2417 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0080 | 2nd br. u/s L.Sillan | 3-4 | 3-4 | 3-4 | 4-5 | 3-4 | 4 | 3-4 |
| 0150 | 2nd br. d/s L. Sillan | - | 3-4 | 3 | 3 | 3 | 3 | 3 |
| 0250 | 1st br. d/s L. Tacker | 3-4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 | - |
| 0350 | Br. Nr Anns fort | 4 | 4 | 4 | 3-4 | 4 | 4 | 4 |
| 0500 | New Grove Br. | 4-5 | 4-5 | 4-5 | 3 | 4 | 4 | 4 |
| 0600 | Br SE of Fort William | 2-3 | 4 | 4-5 | 3-4 | 3-4 | 4 | 3-4 |
| 0800 | Br.NW of Rakenny Ho | 3-4 | 4-5 | 4-5 | 4 | 4-5 | 4 | 4 |
| 0900 | Ballynallon Br | 4 | 4 | 4 | 4 | 3-4 | 4 | 3-4 |
| 1000 | Ballyhaise Br. | 3-4 | 4 | 4 | 4 | 3-4 | 3-4 | 3-4 |
| 1150 | Br. Nr Curraghanoe | 3-4 | 4-5 | 4-5 | 4 | 3-4 | 4-5 | - |
| 1350 | 0.2Km u/s Cavan R confl | 4-5 | 4 | 3-4 | 4 | 3-4 | 3-4 | - |
| 1400 | 0.2km d/s Cavan R confl | 3-4 | 4 | 3-4 | 3-4 | 3 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0080 | NM* | NM | 24 | 17 | 17 | 17 | 19 | 15 | 20 | 30 | 16 |
| 0150 | 60 | 50 | 47 | 30 | 36 | 34 | 61 | 62 | 61 | 63 | 24 |
| 0250 | NM | NM | 41 | 24 | 24 | 30 | 25 | 60 | 43 | - | - |
| 0350 | NM | NM | 35 | 21 | 24 | 35 | 33 | 23 | 44 | 30 | 24 |
| 0500 | 38 | 31 | 36 | 27 | 28 | 38 | 24 | 35 | 41 | 49 | 31 |
| 0600 | NM | NM | 32 | 30 | 23 | 35 | 34 | 35 | 49 | 50 | 56 |
| 0800 |  |  |  |  |  |  |  |  |  | 47 | 53 |
| 0900 | 52 | 42 | 40 | 40 | 24 | 42 | 41 | 35 | 44 | 45 | 56 |
| 1000 | NM | NM | 39 | 41 | 36 | 41 | 38 | 31 | 50 | 46 | 53 |
| 1150 | NM | NM | 39 | 41 | 35 | 40 | 36 | 34 | 33 | - | - |
| 1350 | 41 | 40 | 41 | 41 | 35 | 40 | 38 | 38 | 47 | 41 | 36 |
| 1400 | NM | NM | 55 | 47 | 43 | 45 | 40 | 37 | 53 | 29 | 40 |

*Not Measured

Assessment

This river ranges from poor through to good ecological status. The majority of stations are poor or moderate.

Change from 2008

Some improvement in phosphorous levels at station 0150 downstream of the Cavan Lough Sillan.

River: Ballinagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Ballinagh bridge | 0200 | NW_36_2100 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | ### Station Location | Q’86 | Q’89 |
| --- | --- | --- | --- |
| 0200 | Ballinagh bridge | 3 | 2-3 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | 114 | 109 | 128 | 108 | 152 | 117 | 96 | 87 | 112 | 81 | 127 |

Assessment

The current ecological status is good, however water quality is poor according to chemical data.

Change from 2008

Some reduction in water quality.

River: Bawnboy

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br U/S Brackley Lough | 0300 | - NW_36_1034 | Good | Good | 2009 |
| Br U/S Bellaboy Lough | 0500 | - NW_36_1034 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Br U/S Brackley Lough | 4-5 | 4-5 | 5 | NM | 4-5 | 4 |
| 0500 | Br U/S Bellaboy Lough | 4-5 | 4 | 4 | 4 | 4 | 4 |

MRP results

| Station | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | NM | 13 | 10 | 10 | 10 | 11 | 11 | 10 | 10 | 12 |
| 0500 | NM | 19 | 13 | 10 | 12 | 15 | 14 | 10 | 15 | 10 |

Assessment

The ecological status is good and the chemical results correspond with this status

Change from 2008

## No Change.

River: Blackwater (Kells)

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. 2km NE of Balieborough | 0100 | EA_07_581 | Poor | Good | 2015 |
| U/S discharge pipe at Drumbannon | 0160 |  |  | Good |  |
| Br.at Drumbannon | 0170 | EA_07_581 | Poor | Good | 2015 |
| Footbridge u/s Castle lake | 0200 | EA_07_581 | Poor | Good | 2015 |
| 3rd bridge d/s castle Lake | 0280 | EA_07_581 | Poor | Good | 2015 |
| Br. D/s parkers br | 0420 | EA_07_581 | Poor | Good | 2015 |
| Br in Killenkere | 0500 | EA_07_1035 | Good | Good | 2009 |
| Stramaquerty Br (nr Drumagolan) | 0600 | EA_07_1035 | Good | Good | 2009 |
| Br. u/s L. Ramor | 0800 | EA_07_1035 | Good | Good | 2009 |
| Nine eyes Br | 0900 | EA_07_1536_1 | Poor | Good | 2021 |
| Daly’s bridge | 1000 | EA_07_1536_1 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. 2km NE of Balieborough | - | 4 | 4 | 4-5 | 4-5 | 4 | 4 |
| 0160 | Upstream discharge pipe at Drumbannon | - | - | - | - | - | - | - |
| 0170 | Br.at Drumbannon | 1 | 2 | 2-3 | 2 | 2-3 | 2-3 | 3 |
| 0200 | Footbridge u/s Castle lake | 1 | 2 | - | - |  | - | - |
| 0280 | 3rd bridge d/s castle Lake | 3 | 3 | 3 | 3 | 3 | 3 | - |
| 0420 | Br. D/s parkers bridge | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 |
| 0500 | Br in Killenkere | 4 | 3-4 | 3-4 | 3-4 | 4 | 4 | 4 |
| 0600 | Stramaquerty Br (nr Drumagolan) | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 | 4 |
| 0800 | Br. u/s L. Ramor | 4 | 3-4 | 4 | 4 | 4 | 4 | 4 |
| 0900 | Nine eyes Br | 3 | 3 | 3 | 3 | 3 | 3 | - |
| 1000 | Daly’s bridge | 3-4 | 3 | 3 | 3-4 | 3 | 3 | 3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 32 | 18 | 19 | 17 | 10 | 10 | 23 | 17 | 13 | 19 | 15 |
| 0160 | - | - | - | - | 69 | 51 | 71 | 82 | 90 | 105 | 158 |
| 0170 | NM | NM | 40 | 53 | 65 | 59 | 47 | 119 | 99 | 104 | 91 |
| 0200 | 55 | 38 | - | - | - | - | - | - | - | - | - |
| 0280 | NM | NM | 26 | 24 | 17 | 24 | 29 | 38 | 55 | 101 | - |
| 0400 | NM | NM | 16 | 15 | 20 | 26 | 22 | 30 | 49 | 26 | 17 |
| 0450 |  |  |  |  |  |  |  |  |  | 30 | 23 |
| 0500 | 23 | 23 | 34 | 20 | 15 | 27 | 23 | 30 | 48 | 24 | 30 |
| 0600 | NM | NM | 16 | 21 | 16 | 19 | 25 | 26 | 42 | 20 | 27 |
| 0800 | 29 | 28 | 19 | 25 | 15 | 19 | 20 | 20 | 38 | 19 | 29 |
| 0900 | NM | NM | 17 | 15 | 10 | 10 | 32 | 25 | 30 | - | - |
| 1000 | 10 | 23 | 22 | 15 | 14 | 10 | 23 | 17 | 45 | 35 | 23 |

Assessment

The river is in unsatisfactory condition at the upper reaches near Baileborough at Station 0160 and Station 0170, probably as a result of an IPPC licenced discharge, and is classified as poor until Killenkere. The stretch from Killenkere to Lough Ramor is classified as good and chemical results indicate lower phosphorous levels at these stations. Two stations in the lower reaches downstream of Lough Ramor are unsatisfactory. This is probably as a result of lake effects.

Change from 2008

## There is a reduction in quality at station 0280 with an improvement in quality downstream of this.

River: Blackwater (Newtowngore)

Catchment: Erne

Sampling Stations and Biological Quality Ratings

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br west of Knockmore | 0180 | NW_36_323 | Good | Good | 2009 |
| Br. NE of Stadiran | 0200 | NW_36_323 | Good | Good | 2009 |
| Br. D/s Blackwater Br. | 0400 | NW_36_148 | Moderate | Good | 2021 |
| Br. U/s Ballymagauran Lough | 0600 |  |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0180 | Br west of Knockmore | 5 | 5 | 4-5 | 5 | 4-5 | 4 |
| 0200 | Br. NE of Stadiran | 4-5 | 4-5 | 4 | 5 | 4 | 4 |
| 0400 | Br. D/s Blackwater Br. | 5 | 5 | 4 | - | 4 | 3-4 |
| 0600 | Br. U/s Ballymagauran L | 4 | 4 | 4-5 | - | 4 | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg p/l) | 2006 Med (µg p/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0180 |  |  |  |  |  |  |  |  | 10 | 10 | 10 |
| 0200 | NM | NM | 13 | 11 | 10 | 12 | 10 | 10 | 10 | 15 | 12 |
| 0400 | 12 | 11 | 13 | 14 | 10 | 12 | 10 | 10 | 14 | 16 | 10 |
| 0600 | NM | NM | 15 | 14 | 10 | 12 | 11 | 10 | 15 | - |  |

Assessment

The river is in satisfactory condition. The two stations in the upper reaches are classified as good. The bridge downstream of the blackwater bridge is classified as moderate. Chemical results indicate that river is in satisfactory condition.

Change from 2008

## No significant change.

River: Blackwater (Swanlinbar)

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. D/s Blackwater Br. | 0400 | - NW_36_1161 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | Br. D/s Blackwater Br. | 4-5 | 4 | 4-5 | 5 | 5 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | NM | NM | 12 | 13 | 10 | 10 | 14 | 14 | 12 | 10 | 16 |

Assessment

Satisfactory.

Change from 2008

No Change

River: Bunnoe

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. W of Killynenagh L | 0500 | NW_36_596 | Moderate | Good | 2021 |
| Br. U/s Annalee R confl | 0700 | NW_36_596 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Br. W of Killynenagh L | 3-4 | 3-4 | 3-4 | 3-4 | 3 | 3-4 | - |
| 0700 | Br. U/s Annalee R confl | 3-4 | 4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | NM | NM | 39 | 61 | 29 | 55 | 38 | 28 | 56 | 80 | 60 |
| 0700 | 39 | 50 | 47 | 63 | 37 | 54 | 42 | 40 | 55 | 63 | 90 |

Assessment

River is in moderate condition. Chemical data indicates that this river is polluted.

Change from 2008

No significant change.

River: Cavan

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br nr Breffni Park | 0040 | NW_36_1654 | Poor | Good | 2021 |
| Br N of Killycannon | 0085 | NW_36_1654 | Poor | Good | 2021 |
| Br. D/s lisdarn | 0200 |  |  |  |  |
| Br. d/s St 0200 | 0300 | NW_36_189 | Poor | Good | 2021 |
| Br. u/s Annalee. | 0400 | NW_36_189 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br nr Breffni Park | 3 | 3-4 | 3 | 4 | 3 | 3 | - |
| 0085 | Br N of Killycannon | - | 3-4 | 4-5 | 4 | 4 | 4 | 3 |
| 0200 | Br. D/s lisdarn | 2 | 3 | - | - | - | - | 4 |
| 0300 | Br. d/s St 0200 | 2-3 | 3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |
| 0400 | Br. u/s Annalee. | 2-3 | 3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0002 | - | - | - | - | - | 52* | - | 134** |  |  |  |
| 0003 | - | - | - | - | - | 80 | 43** | 64** |  |  |  |
| 0004 | - | - | - | - | - | 60 | 69** | 80** |  |  |  |
| 0005 | - | - | - | - | - | 77 | 33** | 64** |  |  |  |
| 0030 | - | - | - | - | - | 63* |  | 55** |  |  |  |
| 0040 | - | - | 41 | 36 | 43 | 43 | 36 | 31 | 47 | 74 | 31 |
| 0085 | NM | NM | 70 | 42 | 60 | 69 | 37 | 61 | 71 | 63 | 46 |
| 0300 | NM | NM | 107 | 83 | 59 | 75 | 66 | 104 | 89 | 86 | 52 |
| 0400 | 99 | 68 | 72 | 36 | 58 | 79 | 75 | 77 | 60 | 53 | 63 |

*8 samples taken

** 6 samples taken

Assessment

Unsatisfactory. Overall this river is in poor condition.

Change from 2008

Some improvement in quality at stations 0040, 0085 and 0300.

River: Chapel Lake Stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br near Drumoosclin | 0400 | EA_07_931 | Good | Good | 2009 |
| Br U/S Blackwater Confl | 0700 | EA_07_931 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | Br near Drumoosclin | - | - | 4 | 4 | - | - | - |
| 0700 | Br U/S Blackwater Confl | 3 | 4 | 4 | 4 | 4-5 | 4 | - |

MRP results

| Station | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | NM | 19 | 17 | 22 | 16 | 23 | 16 | 16 | - |  |
| 0700 | NM | 25 | 19 | 14 | 14 | 20 | 27 | 18 | 20* | 39 |

* based on 3 samples

Assessment

Satisfactory

Change from 2008

There is an increase in phosphorous levels at station 0700

River: Cullies

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Gulladoo L | 0300 | NW_36_2032 | Poor | Good | 2021 |
| Br. U/s Drumhart L | 0550 | NW_36_2032 | Poor | Good | 2021 |
| Br. Nr Kilbracken House. | 0600 | NW_36_2032 | Poor | Good | 2021 |
| Br. D/s Laheen L | 0650 | NW_36_188 | Poor | Good | 2021 |
| Br. D/s Disert L | 0750 | NW_36_188 | Poor | Good | 2021 |
| New Br. | 0900 | NW_36_188 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Br u/s Gulladoo L | 3 | 3 | 3 | 3-4 | - | 4 | 3-4 |
| 0550 | Br. U/s Drumhart L | 2-3 | 3-4 | 3-4 | 3 | - | 3-4 | 3-4 |
| 0600 | Br. Nr Kilbracken House. | 1-2 | 3-4 | 3-4 | 3 | 3 | 3 | 3 |
| 0650 | Br. D/s Laheen L | 3 | 3-4 | 3-4 | 3-4 | 3 | 4 | 3-4 |
| 0750 | Br. D/s Disert L | 2 | 3-4 | 3-4 | 3 | 4 | 4 | - |
| 0900 | New Br. | 2 | 2-3 | 2-3 | 3 | 2-3 | 3 | 3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 11 | 14 | 17 | 13 | 10 | 10 | 10 | 10 | 22 | 13 | 10 |
| 0650 | NM | NM | 17 | 10 | 10 | 10 | 10 | 17 | 29 | 24 | 15 |
| 0750 | NM | NM | 18 | 10 | 10 | 17 | 13 | 11 | 24 | - | - |
| 0900 | 35 | 33 | 37 | 26 | 25 | 50 | 30 | 46 | 33 | 46 | 18 |

Assessment

River has a poor ecological status. In contrast the chemical data indicate that this river is unpolluted.

Change from 2008

Improvement at station 0900 downstream of Killeshandra.

River: Dee

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Ervy L | 0016 | NB_06_733 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0016 | Br u/s Ervy L | 4-5 | 4 | 4 | 3-4 | 4 | 3-4 | 3-4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0016 | 14 | 21 | 18 | 17 | 16 | 16 | 16 | 18 | 17 | 16 | 20 |

#### Assessment

This river has a poor ecological status but is satisfactory from a physiochemical element.

Change from 2008

No significant change.

River: Dromore

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| New Br. at Clementstown | 0600 | NW_36_237 | Poor | Good | 2021 |
| Old Bridge | 0700 | NW_36_237 | Poor | Good | 2021 |
| Killycreeney br | 0900 | NW_36_237 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Ballynascarva bridge | 3 | 3 | 3 | 3 | - | 3-4 | 3 |
| 0600 | New Br. at Clementstown | 3 | - | - | - | - | - | - |
| 0700 | Old Bridge | 3 | 3 | 3-4 | 3 | 3 | 3 | 3 |
| 0900 | Killycreeney br | 3-4 | 4 | 4 | 4-5 | 4 | 4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 36 | 38 | 35 | 26 | 33 | 30 | 38 | 31 | 49 | 32 | 22 |
| 0700 | 57 | 44 | 53 | 35 | 37 | 41 | 38 | 42 | 44 | 38 | 30 |
| 0900 | 58 | 48 | 53 | 40 | 39 | 45 | 40 | 40 | 65 | 68 | 44 |

Assessment

This river is in poor condition.

Change from 2008

There is a reduction in phosphorous levels at all stations

River: Derrada

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br West of Drumder | 0040 | - NW_36_1170 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br West of Drumder | - | - | - | - | - | - |

### MRP results

| Station | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- |
| 0016 | 12 | 11 | 12 |

#### Assessment

This river has a poor ecological status but is currently satisfactory from a physiochemical element.

Change from 2008

No significant change

River: Drumane

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br 3km W of Ballyconnell | 0200 | NW_36_1285 | Good | Good | 2009 |
| Br. u/s Woodford R | 0500 | NW_36_1285 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br 3km W of Ballyconnell | 5 | 5 | 5 | - | - | 4 |
| 0500 | Br. u/s Woodford R | 4-5 | - | 4 | 4 | - | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | NM | NM | 12 | 11 | 10 | 11 | 10 | 10 | 10 | 10 | 10 |
| 0500 | 10 | 10 | 12 | 11 | 10 | 10 | 10 | 12 | 10 | - | - |

Assessment

River is in good condition.

Change from 2008

No significant change.

River: Drumkeery Lough stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br d/s Skeagh L | 0100 | EA_07_581 | Poor | Good | 2015 |
| u/s Castle L | 0400 | EA_07_581 | Poor | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br d/s Skeagh L | 3 | 3 | 4 | - | - | - |
| 0400 | u/s Castle L | 3 | 3 | 2/3 | 2/3 | 2/3 | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | NM | NM | 18 | 11 | 14 | 15 | 15* | 14 |
| 0400 | 12 | 21 | 40 | 10 | 23 | 19 | 24 | 20 |

* Medians calculated over a two year period.

#### Assessment

Status is poor in this small stream.

Change from 2008

## Not monitored in 2009

River: Drumsallagh

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br W of Drumsallagh | 0056 | NB_06_484 | Good | Good | 2009 |
| Br ESE of Corrinshigo House | 0066 | NB_06_484 | Good | Good | 2009 |
| Br. U/s Magheracloone branch | 0070 | NB_06_602 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0056 | Br W Drumsallagh | 5 | 3 | 4-5 | 4-5 | 4-5 | 4 | 4 |
| 0066 | Br ESE of Corrinshigo House | 4-5 | 4-5 | 4-5 | 4 | 4 | - | - |
| 0070 | Br. U/s Magheracloone branch | 4-5 | 4-5 | 4-5 | 4-5 | 4-5 | 4 | 4-5 |

MRP results

| Station | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- |
| 0056 | 17 | 13 | 13 |

#### Assessment

Status is good to moderate however phosphorus results indicate good quality water.

Change from 2008

## No significant change

River: Erne

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. 3km S/W of Crosskeys | 0100 | NW_36_350 | Moderate | Good | 2021 |
| Derrin Bridge | 0200 | NW_36_199 | Poor | Good | 2021 |
| Br d/s Derrin Bridge | 0300 |  |  |  |  |
| Br at Carrigan | 0400 | NW_36_199 | Poor | Good | 2021 |
| Legwee bridge | 0500 | NW_36_199 | Poor | Good | 2021 |
| Kilsarn bridge | 0700 | NW_36_199 | Poor | Good | 2021 |
| Dernaferst Bridge | 0770 |  |  |  |  |
| Dingins br. | 0900 | NW_36_1746 | Moderate | Good | 2021 |
| Br east of Corlismore | 1000 | NW_36_1746 | Moderate | Good | 2021 |
| Bellaheelin Br | 1100 | NW_36_1746 | Moderate | Good | 2021 |
| 1 km u/s Carrs lake | 1200 | NW_36_1746 | Moderate | Good | 2021 |
| Drumulig Br near school | 1270 |  |  |  |  |
| Bakers bridge | 1300 | XB_36_east_4 | Poor | Good | 2021 |
| Br at Kilconny, Belturbet (RHS) | 1400 | XB_36_east_4 | Poor | Good | 2021 |
| Kilconny, Belturbet (LHS) | 1410 |  |  |  |  |
| D/s Marina LHS | 1430 |  |  |  |  |
| 2.5 km d/s Belturbet | 1500 |  |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. 3km S/W of Crosskeys | 4 | 4 | 4 | 4 | 4 | 3-4 |
| 0200 | Derrin Bridge | 4-5 | 4 | 3-4 | 3-4 | 4 | 3-4 |
| 0300 | Br d/s Derrin Bridge | - | - | - | - | - | - |
| 0400 | Br at Carrigan | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | - |
| 0500 | Legwee bridge | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3 |
| 0700 | Kilsarn bridge | 4 | 4 | 3-4 | 4 | 3-4 | 3-4 |
| 0770 | Dernaferst Bridge | - | - | - | - | - | - |
| 0900 | Dingins br. | 3 | 3 | 3 | 3 | 3-4 | 3-4 |
| 1000 | Br east of Corlismore | 4 | 4 | 4 | 4 | 4 | - |
| 1100 | Bellaheelin Br | 4 | 4 | 4 | 4 | 4 | 3-4 |
| 1200 | 1 km u/s Carrs lake | - | - | - | - | - | - |
| 1270 | Drumulig Br near school | - | - | - | - | - | - |
| 1300 | Bakers bridge | - | - | - | 3-4 | 3-4 | 3-4 |
| 1400 | Br at Kilconny, Belturbet (RHS) | 3-4 | 3 | 3 | 3 | 3-4 | 3-4 |
| 1410 | Kilconny, Belturbet (LHS) | - | - | 3 | 1 | 1 | - |
| 1430 | D/s Marina LHS | - | - | - | - | - | - |
| 1500 | 2.5 km d/s Belturbet | - | - | - | - | - | - |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 46 | 29 | 26 | 54 | 33 | 41 | 42 | 72 | 41 |
| 0200 | - | - | 29 | 39 | 28 | 62 | 43 | 35 | 56 | 86 | 53 |
| 0300 | 62 | 40 | - | - |  | - |  | - | - | - | - |
| 0400 | - | - | 70 | 70 | 50 | 91 | 60 | 55 | 140 | - | - |
| 0500 | - | - | 52 | 57 | 46 | 81 | 50 | 50 | 78 | 99 | 71 |
| 0700 | 63 | 53 | 60 | 61 | 47 | 70 | 61 | 45 | 73 | 75 | 79 |
| 0770 | 14 | 10 | - | - | - | - |  | - | - | - | - |
| 0900 | - | - | 35 | 22 | 16 | 19 | 15 | 13 | 52 | 52 | 21 |
| 1000 | - | - | 42 | 29 | 30 | 40 | 37 | 37 | 40 | - |  |
| 1100 | 41 | 41 | 37 | 32 | 33 | 47 | 37 | 41 | 61 | 65 | 34 |
| 1200 | 41 | 38 | 41 | 31 | 32 | 48 | 40* | 36 | 52 | 56 | 44 |
| 1300 | 40 | 23 | 27 | 26 | 23 | 29 | 39 | 51 | 63 | 42 | 38 |
| 1400 | 37 | 21 | 36 | 27 | 30 | 37 | 42 | 44 | 62 | 39** | 41 |

Assessment

## River is in unsatisfactory condition. It is classified as poor to moderate throughout

Change from 2008

Some reduction in phosphorous levels in the upper reaches of this river.

River: Glyde

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Cormey Br. | 0100 | NB_06_602 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Cormey Br. | 4-5 | 4-5 | 4-5 | 4-5 | 4 | 4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 46 | 41 | 46 | 30 | 36 | 34 | 47 | 43 | 52 | 58 | 54 |

Assessment

This river is classified as moderate. There is a disparity between the biological quality of this river which designates the river as unpolluted and the elevated phosphorous levels.

Change from 2008

No change

River: Inny

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Ballinrink br | 0300 | SH_26_2664 | Moderate | Good | 2021 |
| Ross bridge | 0400 | SH_26_2664 | Moderate | Good | 2021 |
| Finea Bridge | 0500 | SH_26_2660 | Bad | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’87 | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Ballinrink br | 4 | 3-4 | 4 | 3 | 3 | 3-4 | 3-4 |
| 0400 | Ross bridge | 4 | - | - | - | - | - | - |
| 0500 | Finea Bridge | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | 23 | 19 | 34 | 19 | 16 | 22 | 32 | 24 | 21 | 19 | 10 |
| 0500 | 10 | 10 | 10 | 10 | 10 | 16 | 10 | 10 | 10 | 10 | 10 |

Assessment

Moderate to bad. Station at finea may be bad as a result of lake effects, however physicochemical status is high.

Change from 2008.

## No change

River: Knappagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. U/s Annalee confl | 0700 | - NW_36_1068 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0700 | Br. U/s Annalee confl | 3-4 | 3-4 | 3 | 3-4 | 4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | 21 | 20 | - | - | - | - | - | - | - | - | - |
| 0600 | 30 | 27 | - | - | - | - | - | - | - | - | - |
| 0700 | - | - | 22 | 22 | 21 | 24 | 22 | 20 | 29 | 20 | 17 |

Assessment

Moderate. Phosphorous levels are low in this river.

Change from 2008

## No significant change.

River: Laragh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 1st bridge d/s L. Acanon | 0015 | - NW_36_1557 | Good | Good | 2009 |
| Br W/SW Knockataggert | 0030 | NW_36_228 | Moderate | Good | 2021 |
| Br 1.5 km E Laragh | 0050 | NW_36_228 | Moderate | Good | 2021 |
| Br. In Laragh | 0080 | NW_36_228 | Moderate | Good | 2021 |
| Shanns br Lisclone | 0200 | NW_36_228 | Moderate | Good | 2021 |
| U/s Annalee confl | 0400 | NW_36_228 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0015 | 1st bridge d/s L. Acanon | 3-4 | 3 | 3 | - | - | - |
| 0030 | Br W/SW Knockataggert | - | - | - | - | 4-5 | 4 |
| 0050 | Br 1.5 km E Laragh | 4-5 | 4-5 | 4 | 4 | 4 | - |
| 0080 | Br. In Laragh | 4-5 | 4-5 | 4-5 | 4 | 4 | 3-4 |
| 0200 | Shanns br Lisclone | 3-4 | 4-5 | 4-5 | 3-4 | 3-4 | - |
| 0400 | U/s Annalee confl | 4-5 | 4-5 | 4-5 | 3-4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0015 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 16 |  |  |
| 0030 | - | - |  |  |  |  |  |  | 35 | 25 | 12 |
| 0050 | - | - | 21 | 18 | 10 | 19 | 17 | 18 | 22 |  |  |
| 0080 | - | - | 31 | 30 | 18 | 30 | 26 | 30 | 31 | 31 | 22 |
| 0200 | - | - | 28 | 30 | 18 | 29 | 27 | 24 | 29 |  |  |
| 0400 | 35 | 29 | 28 | 31 | 22 | 32 | 30 | 31 | 35 | 22 | 17 |

Assessment

Most of the river stations are within a water body classified as moderate

Change from 2008

No significant change.

River: Lislea

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 1st bridge u/s L. Ramor | 0100 | EA_07_1368 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 1st bridge u/s L. Ramor | 4 | 4 | 4 | 4 | 4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 25 | 40 | 32 | 34 | 34 | 24 | 30 | 33 | 41 | 32 | 37 |

Assessment

Good status. Phosphorus results are elevated.

Change from 2008

No significant change

River: Maudabawn

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br E of Canningstown | 0070 | NW_36_2024 | Moderate | Good | 2021 |
| Br. 1 km S of Drumcondra | 0200 | NW_36_2024 | Moderate | Good | 2021 |
| bridge u/s Annalee river | 0600 | NW_36_2024 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0070 | Br E of Canningstown | 4 | 3-4 | 3-4 | 3 | 3-4 | 3-4 |
| 0200 | Br. 1 km S of Drumcondra | 4 | 4 | 3 | 4 | 4 | 4 |
| 0600 | bridge u/s Annalee river | 4 | 4 | 4 | 3-4 | 4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0070 | - | - | 25 | 27 | 24 | 35 | 23 | 26 | 42 | 53 | 38 |
| 0200 | - | - | 36 | 38 | 33 | 44 | 28 | 25 | 45 | 64 | 44 |
| 0600 | 33 | 26 | 20 | 30 | 26 | 40 | 23 | 19 | 37 | 60 | 41 |

Assessment

## Moderate

Change from 2008

Some reduction in phosphorous levels

River: Mountnugent

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Near Rockville | 0040 | SH_26_2264 | Poor | Good | 2021 |
| bridge se Rockville | 0120 | SH_26_2264 | Poor | Good | 2021 |
| Br.near Derrylea | 0200 | SH_26_2742 | Poor | Good | 2021 |
| Br. At Kildorra | 0300 | SH_26_2742 | Poor | Good | 2021 |
| Br E of Killnacrot House | 0350 | SH_26_2742 | Poor | Good | 2021 |
| Mountnugent br. | 0500 | SH_26_2742 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’87 | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br. Near Rockville | - | 4 | 4-5 | 3-4 | 4 | 4 | - |
| 0120 | bridge se Rockville | 4 | - | 3 | 1/0 | - | - | 2-3 |
| 0200 | Br.near Derrylea | 1 | 4 | 3-4 | 3 | 4-5 | 4 | 3-4 |
| 0300 | Br. At Kildorra | - | - | - | - | - | - | 3-4 |
| 0350 | Br E of Killnacrot House | 3 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | - |
| 0500 | Mountnugent br. | 3 | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3-4 |

MRP results

Assessment

Unsatisfactory, all stations are classified as poor status.

Change from 2008

Increase in phosphorous levels at stations 0120 and 0300.

River: Moynalty

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Near Lurganbane | 0050 | EA_07_1128 | Good | Good | 2009 |
| Cloggagh Br | 0070 | EA_07_940 | Poor | Good | 2021 |
| Skearke Br. | 0090 | EA_07_940 | Poor | Good | 2021 |
| br. D/s Rosehill Br | 0300 | EA_07_940 | Poor | Good | 2021 |
| bridge u/s main channel confl | 0400 | EA_07_940 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | Br. Near Lurganbane | 4 | 4 | 3 | 3-4 | 3-4 | - | - |
| 0070 | Cloggagh Br | 3 | 3 | 3-4 | 3 | 4 | 3-4 | 3-4 |
| 0090 | Skearke Br. | - | 3 | 4 | 3-4 | 4 | - | - |
| 0300 | br. D/s Rosehill Br | 3-4 | 3 | 3 | 3 | 3-4 | 3-4 | 3-4 |
| 0400 | bridge u/s main channel confl | 3 | 3 | 3 | 3 | 3 | - | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | - | - | 77 | 74 | 50 | 74 | 60 | 56 | 69 |  |  |
| 0070 | - | - | 43 | 45 | 32 | 30 | 38 | 36 | 55 | 61 | 69 |
| 0090 | - | - | 33 | 39 | 26 | 30 | 30 | 28 | 32 |  |  |
| 0200 | 27 | 51 | - | - | - | 25 | - | - | 30 |  |  |
| 0300 | 37 | 49 | 31 | 29 | 27 | 31 | 25 | 33 | 52 | 71 | 34 |
| 0400 | - | - | 134 | 70 | 94 | 72 | 68 | 36 | 114 | 96 | 118 |

Assessment

Unsatisfactory with the exception of station 0050 all stations are within a water body classified as having a poor ecological status.

Change from 2008

Some reduction in phosphorous levels at station 0300.

River: Nadregeel Lough stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Assan Br. | 0100 | EA_07_1221 | Good | Good | 2009 |
| Billis Br | 0200 | EA_07_1221 | Good | Good | 2009 |
| Footbridge d/s billis br | 0300 |  |  | Good |  |
| br. SW of Lisgrey X-rds | 0400 |  |  | Good |  |
| Bridge nr Cranadillon | 0500 | EA_07_1357 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Assan Br. | 3-4 | 3-4 | 4-5 | 3-4 | 3-4 | 4 | 3-4 |
| 0200 | Billis Br | 3 | 3 | 3 | 4 | 3-4 | 4 | - |
| 0300 | Footbridge d/s billis br | - | - | - | - | - | - | - |
| 0400 | br. SW of Lisgrey X-rds | 3 | - | - | - | - | - | - |
| 0500 | Bridge nr Cranadillon | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 23 | 29 | 23 | 26 | 18 | 19 | 26 | 24 | 16 |
| 0200 | 27 | 22 | 20 | 23 | 21 | 22 | 20 | 20 | 23 | 33 | 21 |
| 0500 | 14 | 18 | 16 | 15 | 11 | 16 | 17 | 13 | 25 | 22 | 12 |

Assessment

Generally satisfactory. River water bodies are classified as good to moderate

Change from 2008

## Some decrease in phosphorus levels.

River: Owenayle

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br NE of Aughrim | 0050 | SH_26_1095 | Moderate | Good | 2015 |
| Br D/S of Owenayle bridge | 0100 | SH_26_1095 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | Br NE of Aughrim | 4 | 4 | 4 | 4 | 3-4 | 3-4 |
| 0100 | Br D/S of Owenayle bridge | 4 | 4-5 | 4-5 | 4 | 4 | 4 |

MRP results

| Station | 1999 Med (µgP/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 11 | 12 |
| 0100 | 10 | 10 | 10 | 10 | 10 | 10 | 10* | 10 | 10 | 10 | 10 |

* Calculated over a two year period

Assessment

Moderate. However chemical results indicate good quality water

Change from 2008

## No change

River: Owenmore (Glangevlin)

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Carty’s Bridge | 0100 | SH_26_3419 | Moderate | Good | 2015 |
| Br. SE Carty’s Bridge | 0120 | SH_26_3419 | Moderate | Good | 2015 |
| U/s bellavally branch Confluence | 0140 | SH_26_3419 | Moderate | Good | 2015 |
| Br. NE Moneensauran | 0160 | SH_26_3273 | Good | Good | 2009 |
| B W of Legnacrow L | 0300 | SH_26_3539 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Carty’s Bridge | 4-5 | 4-5 | 5 | 4-5 | 4-5 | 3-4 |
| 0120 | Br. SE Carty’s Bridge | 4-5 | 5 | 4-5 | 5 | - | - |
| 0140 | U/s bellavally branch Confluence | 4 | 4-5 | 4-5 | 4-5 | - | - |
| 0160 | Br. NE Moneensauran | - | 4 | - | - | - | - |
| 0300 | B W of Legnacrow L | 4-5 | 5 | 4-5 | 4-5 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0120 | - | - | 10 | 10 | 10 | 11 | 10 | 10 | 10 | 10 | 10 |
| 0140 | - | - | 10 | 10 | 10 | 10 | 13 | 23 | 10 | 10 | 10 |
| 0160 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | - | - |
| 0300 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |

Assessment

River water bodies are generally moderate. Chemical results indicate low phosphorous levels.

Change from 2008

## No significant change.

River: Owensallagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br E of Drumcor | 0500 | - NW_36_1161 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Br E of Drumcor | 5 | 5 | 3-4 | 4-5 | 4-5 | 4 |

MRP results

| Station | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | 10 | 10 | 10 | 13 | 13 | 15 | 10 | 11 | 10 |

Assessment

River is in good condition.

Change from 2008

## No significant change.

River: Rag

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br D/S Clonty | 0200 | NW_36_550 | Good | Good | 2009 |
| Br D/S Togher | 0400 | NW_36_550 | Good | Good | 2009 |
| Br D/S Aghalane | 0500 | NW_36_550 | Good | Good | 2009 |
| Br. Nr Killywilly house | 0600 | - NW_36_1025 | Good | Good | 2009 |
| Br u/s Tee lough lr | 0800 | XB_36_east_4 | Poor | Good | 2021 |
| Bridge d/s Tee lough lr | 0900 |  |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br D/S Clonty | - | - | - | - | - | - | - |
| 0400 | Br D/S Togher | - | - | - | - | - | - | - |
| 0500 | Br D/S Aghalane | - | - | - | - | - | - | - |
| 0600 | Br. Nr Killywilly house | 3 | 3 | 3 | - | 3-4 | 3-4 | - |
| 0800 | Br u/s Tee lough lr | 2-3 | 3 | 3 | 3-4 | 4 | 4 | 4 |
| 0900 | Bridge d/s Tee lough lr | 3 | 3 | 3 | - | - | - | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | - | 24 | 34 | 27 | 38 | 31 | 28 | 27 | 41 | - |  |
| 0400 | - | 30 | 42 | 37 | 41 | 25 | 36* | 51 | 44 | - |  |
| 0500 | - | 27 | 34 | 16 | 38 | 23 | 25 | 28 | 42 | - |  |
| 0600 | 40 | 29 | 44 | 38 | 24 | 29 | 24 | 31 | 42 | - |  |
| 0800 | - | - | 40 | 36 | 22 | 20 | 38 | 43 | 43 | 29 | 34 |
| 0900 | - | - | 26 | 32 | 26 | 29 | 30 | 37 | 57 | - | - |

#### Assessment

Generally satisfactory, however station 0800 is in a poor quality water body

Change from 2008

No change

River: Roo

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. W Barrran | 0200 | NW_36_2114 | Good | Good | 2009 |
| Br SE of Thornhill house | 0400 | NW_36_2114 |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br. W Barrran | 4-5 | 4 | 4 | 5 | 5 |
| 0400 | Br SE of Thornhill house | - | - | - | 4 | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | 10 | 12 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |

Assessment

River is in good condition.

Change from 2008

## No significant change.

River: Shannon

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Shannon Br, Derrylahan | 0100 | SH_26_3539 | Moderate | Good | 2015 |
| Metal Br. S of Tullynafreave | 0200 | SH_26_3539 | Moderate | Good | 2015 |
| Dowra Br | 0300 | SH_26_3539 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Shannon Br, Derrylahan | 4 | 4-5 | 4 | 4 | 4 | 3-4 |
| 0200 | Metal Br. S of Tullynafreave | 5 | 5 | 4-5 | 4-5 | - | 4-5 |
| 0300 | Dowra Br | 5 | 4-5 | 4-5 | 5 | 4 | 4-5 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 10 | 13 | 10 | 10 | 11 | 10 | 10 | 10 | 10 |
| 0200 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0300 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |

Assessment

Moderate status, however chemical results indicate low phosphorous levels.

Change from 2008

## No significant change

River: Sheelin Feeder Streams

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status Station | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- | --- |
| Belsgrove bridge | 01 |  |  |  |  |  |
| Crover Bridge | 02 |  |  |  |  |  |
| Schoolhouse Bridge | 04 |  |  |  |  |  |
| Belsgrove(Ballynakerry) | 08 |  |  |  |  |  |

### No Biological Quality Ratings available

Sampling stations and Biological Quality Ratings

| Station Number | Station Location | Q’96 |
| --- | --- | --- |
| 01 | Belsgrove bridge | - |
| 02 | Crover Bridge | - |
| 04 | Schoolhouse Bridge | - |
| 08 | Belsgrove(Ballynakerry) | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 24 | 36 | 22 | 19 | 17 | 26 | 18 | 20 | 34 | 39 | 18 |
| 02 | 29 | 31 | 57 | 34 | 47 | 61 | 29 | 27 | 33 | 33 | 23 |
| 04 | 34 | 32 | 37 | 27 | 17 | 21 | 25 | 15 | 29 | 30 | 13 |
| 08 | - | - | - | - | 21 | 31 | 18 | 31 | 26 | 10 | 24 |

Assessment

Satisfactory chemically.

Change from 2008

## Some improvement at Schoolhouse bridge

Note : These rivers are monitored as part of the councils monitoring programme for Lough Sheelin

River: Stradone

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 2nd Br. D/s Lisnanagh L | 0075 | NW_36_245 | Good | Good | 2009 |
| Br u/s Laragh R confl | 0200 | NW_36_245 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0075 | 2nd Br. D/s Lisnanagh L | 3-4 | 4 | 3-4 | 4 | 4 | - |
| 0200 | Br u/s Laragh R confl | 4-5 | 4 | 3-4 | 4 | 4 | 4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0075 | - | - | 40 | 35 | 26 | 29 | 27 | 23 | 46 | 37 | 10 |
| 0200 | 38 | 38 | 33 | 32 | 27 | 25 | 25 | 23 | 40 | 32 | 17 |

Assessment

Satisfactory

Change from 2008

Reduction in phosphorous levels

River: Swanlinbar

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Nr Altbrean | 0100 | NW_36_18 | Moderate | Good | 2021 |
| 0.6km d/s Swanlinbar br | 0300 | NW_36_18 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. Nr Altbrean | 5 | 5 | 5 | - | 4-5 | 4-5 |
| 0300 | 0.6km d/s Swanlinbar br | 4-5 | 4-5 | 4-5 | - | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 |  | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0300 | 14 | 13 | 14 | 13 | 11 | 12 | 14 | 10 | 11 | 10 | 10 |

Assessment

Moderate. Chemical results indicate low phosphorous levels.

Change from 2008

No significant change

River: Templeport Lough Stream

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Templeport Lough | 0600 | NW_36_1177 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 |
| --- | --- | --- | --- | --- | --- | --- |
| 0600 | Br u/s Templeport Lough | 3 | 3 | 4 | 3 | 3 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l | 2007 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 16 | 15 | 14 | 20 | 12 | 20 | 10 | 16 | 13 |

Assessment

## Satisfactory.

Change from 2008

Not monitored chemically in 08

River: Woodford

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status Station | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- | --- |
| Ballyconnell Quay | 0400 |  |  |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’85 | Q’89 |
| --- | --- | --- | --- |
| 0400 | Ballyconnell Quay | 4-5 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | 17 | 12 | 15 | 17 | 11 | 20 | 14 | 12 | 13 | 10 | 10 |

* Baseline Q rating on this station based on ‘89 Q rating

Assessment

## Satisfactory

Change from 2008

No significant change

4	Lake Water Quality

Lakes are listed in alphabetical order.

#### Summary Data

Lake name and catchment:

River name and catchment: Included are the lake name and the catchment name (namely: the Erne, the Boyne, the Shannon and the Glyde). Sub-catchments and water management units are not listed.

The Lake water body code is given for each lake. The Current Ecological status as defined is given with target ecological status. The target ecological status cannot be lower than Good. A target date for reaching the target ecological status is included for each station.

#### Annual Status

This table indicates the annual status of the lake in question and includes

- Average annual Total P (g/l),

- Maximum Chlorophyll,

#### Assessment

A summary assessment of the water quality is given which is based on the scientific data.

Change from 2008

Where applicable any significant changes in quality from 2008 are also given.

List of  Lakes Monitored

Lake 						Catchment

Acurry	Boyne

Annagh	Erne

Ardan	Erne

Bawn	Erne

Bunerky	Erne

Culliaghan	Erne

Derrycassan	Erne

Drumkeery	Boyne

Garty	Erne

Glasshouse	Erne

Gowna	Erne

Kinale	Shannon

Mill Lough	Erne

Nadrageel	Boyne

Oughter	Erne

Ramor	Boyne

Lough Sheelin 	Shannon

Sillan	Erne

Lough Tacker 	Erne

Lake: Accury

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_242 | Good | Good | 2009 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 10 | 12 |
| 2008 | 21 | 10 |
| 2009 | 44 | 30 |

Assessment

## Good status, under the new RBD classification. Generally good quality trout lake.

Change from 2008

Some increase in both phosphorous levels and maximum chlorphyll levels.

Lake: Annagh

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_517 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 9 | 24 |
| 2008 | 29 | 18 |
| 2009 | 44 | 24 |

Assessment

Moderate status, under the new RBD classification. Generally good water abstraction lake

Change from 2008

Increase in both phosphorous levels and maximum chlorphyll levels.

Lake: Ardan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_432 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 10 | 12 |
| 2008 | 21 | 10 |
| 2009 | 44 | 30 |

Assessment

Moderate status, under the new RBD classification

Change from 2008

Some increase in both phosphorous levels and maximum chlorphyll levels.

Lake: Bawn

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_573 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 13 | 10 |
| 2008 | 41 | 4 |
| 2009 | 79 | 6 |

Assessment

Moderate status, under the new RBD classification

Change from 2008

Significant increase in phosphorous levels

Lake: Bunerky

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_624 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 29 | 22 |
| 2008 | 26 | 10 |
| 2009 | 37 | 12 |

Assessment

Moderate status, under the new RBD classification

Change from 2008

No significant change

Lake: Culliaghan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_385 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 22 | 8 |
| 2008 | 51 | 6 |
| 2009 | 46 | 6 |

Assessment

Moderate status, under the new RBD classification

Change from 2008

No significant change

Lake: Derrycasson

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_514 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 14 | 12 |
| 2008 | 34 | 12 |
| 2009 | 45 | 10 |

Assessment

Moderate status, under the new RBD classification

Change from 2008

No significant change

Lake: Drumkeery

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_268 | Poor | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 30 | 34 |
| 2008 | 65 | 40 |
| 2009 | 48 | 16 |

Assessment

## Unsatisfactory.

Change from 2008

Reduction in maximum chlorophyll levels recorded.

Lake: Garty

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_430 | Poor | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 21 | 10 |
| 2008 | 39 | 8 |
| 2009 | 44 | 16 |

Assessment

## Unsatisfactory.

Change from 2008

No significant change

Lake: Glasshouse

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_615 | Bad | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 15 | 10 |
| 2008 | 38 | 22 |
| 2009 | 70 | 20 |

Assessment

## Unsatisfactory

Change from 2008

Significant increase in phosphorous levels.

Lake: Gowna

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_524 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 61 | 121 |
| 2001 | 47 | 107 |
| 2002 | 72 | 56 |
| 2003 | 76 | 52 |
| 2004* | 72 | 125 |
| 2005** | 68 | 125 |
| 2006 | 60 | 56 |
| 2007 | 31 | 48 |
| 2008 | 42 | 30 |
| 2009 | 53 | 99 |

* Based on 9 samples

** Average and maximum values calculated over a two year period

Assessment

## Unsatisfactory.

Change from 2008

Significant increase in maximum chlorophyll levels.

Lake: Kinale

Catchment: Shannon

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| SH_26_430 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 28 | 77 |
| 2001 | 24 | 56 |
| 2002 | 27 | 26 |
| 2003 | 28 | 18 |
| 2004 | 25 | 6 |
| 2005 | 26 | 24 |
| 2006 | 17 | 10 |
| 2007 | 10 | 10 |
| 2008 | 17 | 4 |
| 2009 | 20 | 15 |

Assessment

Moderate status, under the new RBD classification, however physicochemical data is good. The water quality of this lake is inextricabily linked to the quality of water in Lough Sheelin.

Change from 2008

No change

Lake: Mill lough

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_597 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 7 | 18 |
| 2008 | 22 | 28 |
| 2009 | 35 | 16 |

Assessment

Moderate status, under the new RBD classification

Change from 2008

Some increase in phosphorous levels.

Lake: Nadrageel

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_273 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 47 | 103 |
| 2001 | 47 | 85 |
| 2002 | 54 | 118 |
| 2003 | 59 | 111 |
| 2004 | 64 | 71 |
| 2005 | 49 | 50 |
| 2006 | 44 | 92 |
| 2007 | 31 | 38 |
| 2008 | 44 | 40 |
| 2009 | 55 | 28 |

Assessment

Moderate status, under the new RBD classification

Change from 2008

No significant change

Lake: Oughter

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_657 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 73 | 183 |
| 2001 | 64 | 129 |
| 2002 | 70 | 494 |
| 2003 | 94 | 187 |
| 2004 | 101 | 232 |
| 2005** | 90 | 232 |
| 2006 | 82 | 86 |
| 2007 | 54 | 92 |
| 2008 | 51 | 244# |
| 2009 | 76 | 62 |

** Average and Maximum values calculated over a two year period

# next nearest value was 38

Assessment

Moderate status, under the new RBD classification

Change from 2008

Zebra mussels have colonised Lough Oughter. Clarity has improved in the last three years. Maximum chlorophyll result shows some improvement

Lake: Ramor

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_275 | Bad | Good | 2027 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 67 | 90 |
| 2001 | 59 | 92 |
| 2002 | 73 | 80 |
| 2003 | 100 | 165 |
| 2004 | 68 | 75 |
| 2005 | 74 | 80 |
| 2006 | 92 | 102 |
| 2007 | 58 | 83 |
| 2008 | 52 | 110 |
| 2009 | 70 | 69 |

Assessment

## Unsatisfactory.

Change from 2008

Decrease in maximum chlorophyll levels

Lake: Lough Sheelin

Catchment: Shannon

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| SH_26_709 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 32 | 36 |
| 2001 | 23 | 34 |
| 2002 | 28 | 32 |
| 2003 | 31 | 46 |
| 2004 | 32 | 62 |
| 2005* | 25 | 14 |
| 2006 | 25 | 8 |
| 2007 | 18 | 24 |
| 2008 | 28 | 14 |
| 2009 | 26 | 12 |

*includes one sample from January 2006

Assessment

Moderate status, under the new RBD classification.

Change from 2008

No significant change

Lake: Sillan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_528 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 77 | 73 |
| 2001 | 57 | 50 |
| 2002 | 56 | 73 |
| 2003 | 66 | 52 |
| 2004 | 97 | 62 |
| 2005** | 103 | 74 |
| 2006 | 117 | 68 |
| 2007 | 46 | 66 |
| 2008 | 79 | 121 |
| 2009 | 73 | 54 |

** Average and Maximum values calculated over a two year period

Assessment

Moderate status, under the new RBD classification

Change from 2008

Significant reduction in maximum chlorophyll levels

Lake: Lough Tacker

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_363 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 47 | 32 |
| 2008 | 69 | 22 |
| 2009 | 69 | 16 |

Assessment

Moderate status, under the new RBD classification

Change from 2008

No significant change