Comhairle Chontae an Cabhain

CAVAN  COUNTY  COUNCIL

THE QUALITY OF RIVER

& LAKE WATER IN COUNTY CAVAN

###### A Report for the Year 2011

J Keyes			E Doyle			Environment Section

County Manager		Director of Services	 	Cavan County Council

Environment			June 2012

Index

Page

- Introduction	1

- River Water Quality	11

- Lake Water Quality	50

2.	Introduction

This report presents the summary monitoring data for 2011 for the rivers and lakes in County Cavan along with historical data; it can be viewed as a companion volume to those previously produced.

All chemical monitoring was undertaken by the Council’s Environment Section. Monitoring complies with the requirements of the monitoring programme prescribed by the EPA in compliance with the requirements of the Water Framework Directive(WFD). Approximately 99 river stations and 21 lakes are monitored. The monitoring programme allows comparison, on an annual basis, of the chemical quality of the rivers and lakes.

The Biological monitoring referred to in this report on river stations was undertaken by the EPA as part of its National Programme of Biological Assessment of River Water Quality.The EPA’s contribution is gratefully acknowledged.

The lake monitoring programme involves the monitoring of 21 lakes designated by the EPA. The number of stations monitored on each lake varies depending on a number of factors including lake size, historical data and uniformity of water body. Samples are taken at time intervals specified by the EPA. This involves a minimum of four samples taken once in April and three times between July and October.

The new classification system of quality standards introduced in 2009 is defined in S.I. No. 272 of 2009 the EUROPEAN COMMUNITIES ENVIRONMENTAL OBJECTIVES (SURFACE WATERS) REGULATIONS 2009. The Classification tables are defined in schedules 4 , 5 and 6 over 13 pages of these regulations.

In order to allow the reader assess the information regarding the quality of river water three tables are shown below.

Tables A and B set out the standards specified in the Local Government (Water Pollution) Act, 1977 (Water Quality Standards for Phosphorus), 1998 for rivers and lakes respectively. These regulations identify biological ratings and Molybdate-Reactive Phosphorus Annual Median concentrations for the rivers, and Chlorophyll and Total Phosphorus levels for the lakes, either of which must be at least maintained or improved in order to meet the relevant specified target standard.

Note: The Phosphorus regulations have been revoked however this information is provided as an aide to interpretation of the results

Table C sets out the standard specified in EC directive 78/659/EEC (Freshwater Fish).

Table A: ‘Phosphorus Regs’, Local Government (Water Pollution) Act, 1977 (Water Quality  Standards for Rivers), 1998

| Existing Biological Quality Q Rating | Molybdate -Reactive Phosphorus annual Median concentration (mg/l P) | Target Biological Quality (Q) Rating |
| --- | --- | --- |
| 5 } 4-5 } Unpolluted 4 } | 0.015 0.020 0.030 | 5 4-5 4 |
| 3-4 }Slightly Polluted | 0.030 | 4 |
| 3 } Moderately 2-3 } Polluted | 0.05 0.070 | 3-4 3 |
| < 2 } Seriously Polluted | 0.070 | 3 |

Table B: ‘Phosphorus Regs’,Trophic Classification System for Lakes

| Lake Trophic Category | Target Trophic Status | Total Phosphorus Average Concentration (g P/l) | Annual Max. Chlorophyll (mg/m3) |
| --- | --- | --- | --- |
| Ultra-Oligotrophic | Ultra-Oligotrophic |  5 | < 2.5 |
| ### Oligotrophic | ### Oligotrophic | > 5  10 | > 2.5 < 8 |
| Mesotrophic | Mesotrophic | > 10  20 |  8 < 25 |
| Eutrophic | Mesotrophic | > 10  20 |  8 < 25 |
| Hypertrophic | Eutrophic | > 20  50 |  25 < 75 |

Table C: EC Directive 78/659/EEC (Freshwater Fish)

| Parameter | ### Units | Salmonid | ### Waters | ### Cyprinid | Waters |
| --- | --- | --- | --- | --- | --- |
|  |  | Guide | #### Mandatory | Guide | #### Mandatory |
| Total Ammonia | mg/l N | < 0.03 | < 0.78 | < 0.16 | < 0.78 |
| B.O.D. | mg/l O2 | 100% < 3 |  | 100% < 6 |  |
| D.O. | mg/l O2 | 50% > 9 | 50% > 9 | 50% > 8 | 50% > 7 |
| Total Phosphorus | mg/l P |  | 0.065 |  | 0.13 |
| PH | pH units |  | 6-9 |  | 6-9 |

3.	River Water Quality

Rivers are listed in alphabetical order.

River name and catchment: Included are the river name and the catchment name (namely: the Erne, the Boyne, the Shannon and the Glyde). Sub-catchments and water management units are not listed.

Sampling stations: Sampling stations are numbered according to the EPA assignment. In the case where the EPA has not assigned a sampling station number, no number is included. Stations are listed  starting upstream and leading downstream and include stations that are biologically assessed. It is not always possible to use the same location for both chemical and biological assessment, as the requirements for both are different. A brief location description for each station is also given. The water body code is given for each station, and there may be more than one individual station in a single water body. The current ecological status as defined is given with target ecological status. The target ecological status cannot be lower than Good. A target date for reaching the target ecological status is included for each station.

Biological Quality Ratings: This report includes recent and historical data from the EPA’s National Biological River Monitoring Programme. The Q ratings for 2011 are provisional. The biotic index classification is listed in Table A.

MRP results

Median annual MRP results from 1999-2010 are included in the summary data.

#### Assessment

A summary assessment of the water quality is given.

Change from 2010

Any significant changes in quality from 2010 are also given.

Note: The analysis of MRP involves a specific procedure. Analytical procedures are designed to accurately detect concentration levels over a specific range. The upper and lower limits of this range are assigned as limits of quantification. The lower limit of quantification for MRP is assigned at 10µg P/l. In this section, if a result of  10µg P/l is recorded, this is reported as 10 µg P/l.

##### List of  Rivers Monitored

River 						Catchment

Annalee						Erne

Ballinagh						Erne

Bawnboy 						Erne

Blackwater (Kells)					Boyne

Blackwater (Newtowngore)			Erne

Blackwater (Swanlinbar)				Erne

Bunnoe						Erne

Cavan							Erne

Chapel Lake Stream				Boyne

Cullies						Erne

Dee							Glyde

Dromore						Erne

Derrada	Erne

Drumane						Erne

Drumkeery Lough stream			Boyne

Drumsallagh					Glyde

Erne							Erne

Glyde							Glyde

Inny							Shannon

Knappagh						Erne

Laragh						Erne

Lislea							Boyne

Maudabawn						Erne

Mountnugent					Shannon

Moynalty						Boyne

Nadregeel Lough Stream				Boyne

Owenayle						Shannon

Owenmore (Glangevlin)				Shannon

Owensallagh					Erne

Rag							Erne

Roo							Erne

Shannon						Shannon

Sheelin Feeder Streams				Shannon

Stradone						Erne

Swanlinbar						Erne

Templeport Lough Stream			Erne

Woodford						Erne

River: Annalee

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 2nd br. u/s L.Sillan | 0080 | NW_36_1947 | Moderate | Good | 2021 |
| 2nd br. d/s L. Sillan | 0150 | NW_36_1181 | Poor | Good | 2021 |
| 1st br. d/s L. Tacker | 0250 | NW_36_1072 | Good | Good | 2009 |
| Br. Nr Anns fort | 0350 | - NW_36_1102 | Moderate | Good | 2021 |
| New Grove Br. | 0500 | - NW_36_1102 | Moderate | Good | 2021 |
| Br SE of Fort William | 0600 | - NW_36_1102 | Moderate | Good | 2021 |
| Br.NW of Rakenny Ho | 0800 | NW_36_2417 | Poor | Good | 2021 |
| Ballynallon Br | 0900 | NW_36_2417 | Poor | Good | 2021 |
| Ballyhaise Br. | 1000 | NW_36_2417 | Poor | Good | 2021 |
| Br. Nr Curraghanoe | 1150 | NW_36_2417 | Poor | Good | 2021 |
| 0.2Km u/s Cavan R confl | 1350 | NW_36_2417 | Poor | Good | 2021 |
| 0.2km d/s Cavan R confl | 1400 | NW_36_2417 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0080 | 2nd br. u/s L.Sillan | 3-4 | 3-4 | 3-4 | 4-5 | 3-4 | 4 | 3-4 | 4 |
| 0150 | 2nd br. d/s L. Sillan | - | 3-4 | 3 | 3 | 3 | 3 | 3 | 3 |
| 0250 | 1st br. d/s L. Tacker | 3-4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 | - | - |
| 0350 | Br. Nr Anns fort | 4 | 4 | 4 | 3-4 | 4 | 4 | 4 | 4 |
| 0500 | New Grove Br. | 4-5 | 4-5 | 4-5 | 3 | 4 | 4 | 4 | 4 |
| 0600 | Br SE of Fort William | 2-3 | 4 | 4-5 | 3-4 | 3-4 | 4 | 3-4 | 3-4 |
| 0800 | Br.NW of Rakenny Ho | 3-4 | 4-5 | 4-5 | 4 | 4-5 | 4 | 4 | 4 |
| 0900 | Ballynallon Br | 4 | 4 | 4 | 4 | 3-4 | 4 | 3-4 | 4 |
| 1000 | Ballyhaise Br. | 3-4 | 4 | 4 | 4 | 3-4 | 3-4 | 3-4 | 3-4 |
| 1150 | Br. Nr Curraghanoe | 3-4 | 4-5 | 4-5 | 4 | 3-4 | 4-5 | - | 4 |
| 1350 | 0.2Km u/s Cavan R confl | 4-5 | 4 | 3-4 | 4 | 3-4 | 3-4 | - | 3-4 |
| 1400 | 0.2km d/s Cavan R confl | 3-4 | 4 | 3-4 | 3-4 | 3 | 3-4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0080 | NM* | NM | 24 | 17 | 17 | 17 | 19 | 15 | 20 | 30 | 16 | 12 | 16 |
| 0150 | 60 | 50 | 47 | 30 | 36 | 34 | 61 | 62 | 61 | 63 | 24 | 40 | 45 |
| 0250 | NM | NM | 41 | 24 | 24 | 30 | 25 | 60 | 43 | - | - | - | - |
| 0350 | NM | NM | 35 | 21 | 24 | 35 | 33 | 23 | 44 | 30 | 24 | 30 | 21 |
| 0500 | 38 | 31 | 36 | 27 | 28 | 38 | 24 | 35 | 41 | 49 | 31 | 28 | 30 |
| 0600 | NM | NM | 32 | 30 | 23 | 35 | 34 | 35 | 49 | 50 | 56 | 33 | 33 |
| 0800 |  |  |  |  |  |  |  |  |  | 47 | 53 | 57 | 36 |
| 0900 | 52 | 42 | 40 | 40 | 24 | 42 | 41 | 35 | 44 | 45 | 56 | 54 | 38 |
| 1000 | NM | NM | 39 | 41 | 36 | 41 | 38 | 31 | 50 | 46 | 53 | 52 | 39 |
| 1150 | NM | NM | 39 | 41 | 35 | 40 | 36 | 34 | 33 | - | - | - | - |
| 1350 | 41 | 40 | 41 | 41 | 35 | 40 | 38 | 38 | 47 | 41 | 36 | 51 | 39 |
| 1400 | NM | NM | 55 | 47 | 43 | 45 | 40 | 37 | 53 | 29 | 40 | 67 | 44 |

*Not Measured

Assessment

This river ranges from poor through to good ecological status. The majority of stations are poor or moderate.

Change from 2010

Some improvement in median ortho-phosphate levels in the lower reaches of this river.

River: Ballinagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Ballinagh bridge | 0200 | NW_36_2100 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | ### Station Location | Q’86 | Q’89 |
| --- | --- | --- | --- |
| 0200 | Ballinagh bridge | 3 | 2-3 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | 114 | 109 | 128 | 108 | 152 | 117 | 96 | 87 | 112 | 81 | 127 | 87 | 77 |

Assessment

The current ecological status is good. The water quality is poor according to chemical data.

Change from 2010

No change.

River: Bawnboy

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br U/S Brackley Lough | 0300 | - NW_36_1034 | Good | Good | 2009 |
| Br U/S Bellaboy Lough | 0500 | - NW_36_1034 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Br U/S Brackley Lough | 4-5 | 4-5 | 5 | NM | 4-5 | 4 | 4 |
| 0500 | Br U/S Bellaboy Lough | 4-5 | 4 | 4 | 4 | 4 | 4 | 3 |

MRP results

| Station | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | NM | 13 | 10 | 10 | 10 | 11 | 11 | 10 | 10 | 12 | 10 | 10 |
| 0500 | NM | 19 | 13 | 10 | 12 | 15 | 14 | 10 | 15 | 10 | 10 | 12 |

Assessment

The ecological status is good and the chemical results correspond with this status

Change from 2010

No significant change

River: Blackwater (Kells)

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. 2km NE of Balieborough | 0100 | EA_07_581 | Poor | Good | 2015 |
| U/S discharge pipe at Drumbannon | 0160 |  |  | Good |  |
| Br.at Drumbannon | 0170 | EA_07_581 | Poor | Good | 2015 |
| Footbridge u/s Castle lake | 0200 | EA_07_581 | Poor | Good | 2015 |
| 3rd bridge d/s castle Lake | 0280 | EA_07_581 | Poor | Good | 2015 |
| Br. D/s parkers br | 0420 | EA_07_581 | Poor | Good | 2015 |
| Br in Killenkere | 0500 | EA_07_1035 | Moderate | Good | 2009 |
| Stramaquerty Br (nr Drumagolan) | 0600 | EA_07_1035 | Moderate | Good | 2009 |
| Br. u/s L. Ramor | 0800 | EA_07_1035 | Moderate | Good | 2009 |
| Nine eyes Br | 0900 | EA_07_1536_1 | Poor | Good | 2027 |
| Daly’s bridge | 1000 | EA_07_1536_1 | Poor | Good | 2027 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. 2km NE of Balieborough | - | 4 | 4 | 4-5 | 4-5 | 4 | 4 |
| 0160 | Upstream discharge pipe at Drumbannon | - | - | - | - | - | - | - |
| 0170 | Br.at Drumbannon | 1 | 2 | 2-3 | 2 | 2-3 | 2-3 | 3 |
| 0200 | Footbridge u/s Castle lake | 1 | 2 | - | - |  | - | - |
| 0280 | 3rd bridge d/s castle Lake | 3 | 3 | 3 | 3 | 3 | 3 | - |
| 0420 | Br. D/s parkers bridge | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 |
| 0500 | Br in Killenkere | 4 | 3-4 | 3-4 | 3-4 | 4 | 4 | 4 |
| 0600 | Stramaquerty Br (nr Drumagolan) | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 | 4 |
| 0800 | Br. u/s L. Ramor | 4 | 3-4 | 4 | 4 | 4 | 4 | 4 |
| 0900 | Nine eyes Br | 3 | 3 | 3 | 3 | 3 | 3 | - |
| 1000 | Daly’s bridge | 3-4 | 3 | 3 | 3-4 | 3 | 3 | 3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 32 | 18 | 19 | 17 | 10 | 10 | 23 | 17 | 13 | 19 | 15 | 12 | 20 |
| 0160 | - | - | - | - | 69 | 51 | 71 | 82 | 90 | 105 | 158 | 42 | 34 |
| 0170 | NM | NM | 40 | 53 | 65 | 59 | 47 | 119 | 99 | 104 | 91 | 42 | 35 |
| 0200 | 55 | 38 | - | - | - | - | - | - | - | - | - |  | - |
| 0280 | NM | NM | 26 | 24 | 17 | 24 | 29 | 38 | 55 | 101 | - |  | - |
| 0300 |  |  |  |  |  |  |  |  |  |  |  | 46 | 30 |
| 0400 | NM | NM | 16 | 15 | 20 | 26 | 22 | 30 | 49 | 26 | 17 | 16 | 26 |
| 0450 |  |  |  |  |  |  |  |  |  | 30 | 23 | 17 | 27 |
| 0500 | 23 | 23 | 34 | 20 | 15 | 27 | 23 | 30 | 48 | 24 | 30 | 22 | 28 |
| 0600 | NM | NM | 16 | 21 | 16 | 19 | 25 | 26 | 42 | 20 | 27 | 23 | 27 |
| 0800 | 29 | 28 | 19 | 25 | 15 | 19 | 20 | 20 | 38 | 19 | 29 | 23 | 32 |
| 0900 | NM | NM | 17 | 15 | 10 | 10 | 32 | 25 | 30 | - | - |  | - |
| 1000 | 10 | 23 | 22 | 15 | 14 | 10 | 23 | 17 | 45 | 35 | 23 | 15 | 16 |

Assessment

The river is in general poor based on the ecological status at the upper reaches near Baileborough and is classified as poor until Killenkere. The stretch from Killenkere to Lough Ramor is classified as moderate. Two stations in the lower reaches downstream of Lough Ramor are poor. This is probably as a result of lake effects.

Change from 2010

No significant change

River: Blackwater (Newtowngore)

Catchment: Erne

Sampling Stations and Biological Quality Ratings

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br west of Knockmore | 0180 | NW_36_323 | Good | Good | 2009 |
| Br. NE of Stadiran | 0200 | NW_36_323 | Good | Good | 2009 |
| Br. D/s Blackwater Br. | 0400 | NW_36_148 | Moderate | Good | 2021 |
| Br. U/s Ballymagauran Lough | 0600 | - | - | - | - |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0180 | Br west of Knockmore | 5 | 5 | 4-5 | 5 | 4-5 | 4 | 4 |
| 0200 | Br. NE of Stadiran | 4-5 | 4-5 | 4 | 5 | 4 | 4 | 4-5 |
| 0400 | Br. D/s Blackwater Br. | 5 | 5 | 4 | - | 4 | 3-4 | 4 |
| 0600 | Br. U/s Ballymagauran L | 4 | 4 | 4-5 | - | 4 | - | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg p/l) | 2006 Med (µg p/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0180 |  |  |  |  |  |  |  |  | 10 | 10 | 10 | 10 | 18 |
| 0200 | NM | NM | 13 | 11 | 10 | 12 | 10 | 10 | 10 | 15 | 12 | 10 | 10 |
| 0400 | 12 | 11 | 13 | 14 | 10 | 12 | 10 | 10 | 14 | 16 | 10 | 10 | 10 |
| 0600 | NM | NM | 15 | 14 | 10 | 12 | 11 | 10 | 15 | - | - | - | - |

Assessment

The two stations in the upper reaches are classified as good. The bridge downstream of the blackwater bridge is classified as moderate. Chemical results indicate that river is in satisfactory condition.

Change from 2010

## No significant change.

River: Blackwater (Swanlinbar)

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. D/s Blackwater Br. | 0400 | - NW_36_1161 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | Br. D/s Blackwater Br. | 4-5 | 4 | 4-5 | 5 | 5 | 4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | NM | NM | 12 | 13 | 10 | 10 | 14 | 14 | 12 | 10 | 16 | 10 | 11 |

Assessment

Good.

Change from 2010

No significant change

River: Bunnoe

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. W of Killynenagh L | 0500 | NW_36_596 | Moderate | Good | 2021 |
| Br. U/s Annalee R confl | 0700 | NW_36_596 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Br. W of Killynenagh L | 3-4 | 3-4 | 3-4 | 3-4 | 3 | 3-4 | - | 3-4 |
| 0700 | Br. U/s Annalee R confl | 3-4 | 4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | NM | NM | 39 | 61 | 29 | 55 | 38 | 28 | 56 | 80 | 60 | 29 | 40 |
| 0700 | 39 | 50 | 47 | 63 | 37 | 54 | 42 | 40 | 55 | 63 | 90 | 33 | 50 |

Assessment

River is in moderate condition.

Change from 2010

Increase in median ortho-phosphate levels.

River: Cavan

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br nr Breffni Park | 0040 | NW_36_1654 | Good | Good | 2021 |
| Br N of Killycannon | 0085 | NW_36_1654 | Good | Good | 2021 |
| Br. D/s lisdarn | 0200 |  |  |  |  |
| Br. d/s St 0200 | 0300 | NW_36_189 | Poor | Good | 2021 |
| Br. u/s Annalee. | 0400 | NW_36_189 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br nr Breffni Park | 3 | 3-4 | 3 | 4 | 3 | 3 | 3 | 3 |
| 0085 | Br N of Killycannon | - | 3-4 | 4-5 | 4 | 4 | 4 | 4 | 4 |
| 0200 | Br. D/s lisdarn | 2 | 3 | - | - | - | - |  |  |
| 0300 | Br. d/s St 0200 | 2-3 | 3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2 |
| 0400 | Br. u/s Annalee. | 2-3 | 3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0002 | - | - | - | - | - | 52* | - | 134** |  |  |  |  |  |
| 0003 | - | - | - | - | - | 80 | 43** | 64** |  |  |  |  |  |
| 0004 | - | - | - | - | - | 60 | 69** | 80** |  |  |  |  |  |
| 0005 | - | - | - | - | - | 77 | 33** | 64** |  |  |  |  |  |
| 0030 | - | - | - | - | - | 63* |  | 55** |  |  |  |  |  |
| 0040 | - | - | 41 | 36 | 43 | 43 | 36 | 31 | 47 | 74 | 31 | 47 | 49 |
| 0085 | NM | NM | 70 | 42 | 60 | 69 | 37 | 61 | 71 | 63 | 46 | 36 | 53 |
| 0300 | NM | NM | 107 | 83 | 59 | 75 | 66 | 104 | 89 | 86 | 52 | 60 | 66 |
| 0400 | 99 | 68 | 72 | 36 | 58 | 79 | 75 | 77 | 60 | 53 | 63 | 100 | 99 |

*8 samples taken

** 6 samples taken

Assessment

Overall this river is classified as poor status downstream of the town and good status upstream of the town.

Change from 2010

Some increase in median ortho-phosphate levels at station 0400.

River: Chapel Lake Stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br near Drumoosclin | 0400 | EA_07_931 | Good | Good | 2009 |
| Br U/S Blackwater Confl | 0700 | EA_07_931 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | Br near Drumoosclin | - | - | 4 | 4 | - | - | - |
| 0700 | Br U/S Blackwater Confl | 3 | 4 | 4 | 4 | 4-5 | 4 | - |

MRP results

| Station | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | NM | 19 | 17 | 22 | 16 | 23 | 16 | 16 | - | - | - | - |
| 0700 | NM | 25 | 19 | 14 | 14 | 20 | 27 | 18 | 20* | 39 | 30 | 25 |

* based on 3 samples

Assessment

Good

Change from 2009

No significant change.

River: Cullies

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Gulladoo L | 0300 | NW_36_2032 | Poor | Good | 2021 |
| Br. U/s Drumhart L | 0550 | NW_36_2032 | Poor | Good | 2021 |
| Br. Nr Kilbracken House. | 0600 | NW_36_2032 | Poor | Good | 2021 |
| Br. D/s Laheen L | 0650 | NW_36_188 | Poor | Good | 2021 |
| Br. D/s Disert L | 0750 | NW_36_188 | Poor | Good | 2021 |
| New Br. | 0900 | NW_36_188 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Br u/s Gulladoo L | 3 | 3 | 3 | 3-4 | - | 4 | 3-4 | 3-4 |
| 0550 | Br. U/s Drumhart L | 2-3 | 3-4 | 3-4 | 3 | - | 3-4 | 3-4 | 3-4 |
| 0600 | Br. Nr Kilbracken House. | 1-2 | 3-4 | 3-4 | 3 | 3 | 3 | 3 | 3 |
| 0650 | Br. D/s Laheen L | 3 | 3-4 | 3-4 | 3-4 | 3 | 4 | 3-4 | 3-4 |
| 0750 | Br. D/s Disert L | 2 | 3-4 | 3-4 | 3 | 4 | 4 | - | 3-4 |
| 0900 | New Br. | 2 | 2-3 | 2-3 | 3 | 2-3 | 3 | 3 | 3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 11 | 14 | 17 | 13 | 10 | 10 | 10 | 10 | 22 | 13 | 10 | 14 | 12 |
| 0650 | NM | NM | 17 | 10 | 10 | 10 | 10 | 17 | 29 | 24 | 15 | 15 | 18 |
| 0750 | NM | NM | 18 | 10 | 10 | 17 | 13 | 11 | 24 | - | - | - | - |
| 0900 | 35 | 33 | 37 | 26 | 25 | 50 | 30 | 46 | 33 | 46 | 18 | 27 | 35 |

Assessment

River has a poor ecological status. In contrast the chemical data indicate that this river is relatively unpolluted.

Change from 2010

No significant change.

River: Dee

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Ervy L | 0016 | NB_06_733 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0016 | Br u/s Ervy L | 4-5 | 4 | 4 | 3-4 | 4 | 3-4 | 3-4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0016 | 14 | 21 | 18 | 17 | 16 | 16 | 16 | 18 | 17 | 16 | 20 | 20 | 22 |

#### Assessment

This river has a poor ecological status but has low levels of median ortho-Phosphorus.

Change from 2010

No significant change.

River: Dromore

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| New Br. at Clementstown | 0600 | NW_36_237 | Poor | Good | 2021 |
| Old Bridge | 0700 | NW_36_237 | Poor | Good | 2021 |
| Killycreeney br | 0900 | NW_36_237 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Ballynascarva bridge | 3 | 3 | 3 | 3 | - | 3-4 | 3 | 3 |
| 0600 | New Br. at Clementstown | 3 | - | - | - | - | - | - | - |
| 0700 | Old Bridge | 3 | 3 | 3-4 | 3 | 3 | 3 | 3 | 3 |
| 0900 | Killycreeney br | 3-4 | 4 | 4 | 4-5 | 4 | 4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 36 | 38 | 35 | 26 | 33 | 30 | 38 | 31 | 49 | 32 | 22 | 30 | 24 |
| 0700 | 57 | 44 | 53 | 35 | 37 | 41 | 38 | 42 | 44 | 38 | 30 | 55 | 37 |
| 0900 | 58 | 48 | 53 | 40 | 39 | 45 | 40 | 40 | 65 | 68 | 44 | 57 | 43 |

Assessment

This river is in poor condition.

Change from 2009

There is a reduction in Phosphorus levels at all stations

River: Derrada

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br West of Drumder | 0040 | - NW_36_1170 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’97 | Q’98 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br West of Drumderg | - | 4-5 | 4-5 | 4 | 4 | 4 |

### MRP results

| Station | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- |
| 0016 | 12 | 11 | 12 | 10 | 10 |

#### Assessment

This river has a poor ecological status but is currently satisfactory from a physiochemical and biological quality element. Ecological status is defined as poor because of macroinvertebrate monitoring in this waterbody, this would appear to be contrary to the actual Q result for this station.

Change from 2010

No significant change

River: Drumane

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br 3km W of Ballyconnell | 0200 | NW_36_1285 | Good | Good | 2009 |
| Br. u/s Woodford R | 0500 | NW_36_1285 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br 3km W of Ballyconnell | 5 | 5 | 5 | - | - | 4 | 4-5 |
| 0500 | Br. u/s Woodford R | 4-5 | - | 4 | 4 | - | - | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | NM | NM | 12 | 11 | 10 | 11 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0500 | 10 | 10 | 12 | 11 | 10 | 10 | 10 | 12 | 10 | - | - | - | - |

Assessment

River is in good condition.

Change from 2010

No change.

River: Drumkeery Lough stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br d/s Skeagh L | 0100 | EA_07_581 | Poor | Good | 2015 |
| u/s Castle L | 0400 | EA_07_581 | Poor | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br d/s Skeagh L | 3 | 3 | 4 | - | - | - |
| 0400 | u/s Castle L | 3 | 3 | 2/3 | 2/3 | 2/3 | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | NM | NM | 18 | 11 | 14 | 15 | 15* | 14 |
| 0400 | 12 | 21 | 40 | 10 | 23 | 19 | 24 | 20 |

* Medians calculated over a two year period.

#### Assessment

Status is poor in this small stream.

Change from 2010

## Not designated by EPA for monitoring

River: Drumsallagh

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br W of Drumsallagh | 0056 | NB_06_484 | Good | Good | 2009 |
| Br ESE of Corrinshigo House | 0066 | NB_06_484 | Good | Good | 2009 |
| Br. U/s Magheracloone branch | 0070 | NB_06_602 | Good | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0056 | Br W Drumsallagh | 5 | 3 | 4-5 | 4-5 | 4-5 | 4 | 4 |
| 0066 | Br ESE of Corrinshigo House | 4-5 | 4-5 | 4-5 | 4 | 4 | - | - |
| 0070 | Br. U/s Magheracloone branch | 4-5 | 4-5 | 4-5 | 4-5 | 4-5 | 4 | 4-5 |

MRP results

| Station | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- |
| 0056 | 17 | 13 | 13 | 16 | 19 |

#### Assessment

Status is good.

Change from 2010

## No significant change

River: Erne

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. 3km S/W of Crosskeys | 0100 | NW_36_350 | Moderate | Good | 2021 |
| Derrin Bridge | 0200 | NW_36_199 | Poor | Good | 2021 |
| Br d/s Derrin Bridge | 0300 |  |  |  |  |
| Br at Carrigan | 0400 | NW_36_199 | Poor | Good | 2021 |
| Legwee bridge | 0500 | NW_36_199 | Poor | Good | 2021 |
| Kilsarn bridge | 0700 | NW_36_199 | Poor | Good | 2021 |
| Dernaferst Bridge | 0770 |  |  |  |  |
| Dingins br. | 0900 | NW_36_1746 | Moderate | Good | 2021 |
| Br east of Corlismore | 1000 | NW_36_1746 | Moderate | Good | 2021 |
| Bellaheelin Br | 1100 | NW_36_1746 | Moderate | Good | 2021 |
| 1 km u/s Carrs lake | 1200 | NW_36_1746 | Moderate | Good | 2021 |
| Drumulig Br near school | 1270 |  |  |  |  |
| Bakers bridge | 1300 | XB_36_east_4 | Poor | Good | 2021 |
| Br at Kilconny, Belturbet (RHS) | 1400 | XB_36_east_4 | Poor | Good | 2021 |
| Kilconny, Belturbet (LHS) | 1410 |  |  |  |  |
| D/s Marina LHS | 1430 |  |  |  |  |
| 2.5 km d/s Belturbet | 1500 |  |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. 3km S/W of Crosskeys | 4 | 4 | 4 | 4 | 4 | 3-4 | 3-4 |
| 0200 | Derrin Bridge | 4-5 | 4 | 3-4 | 3-4 | 4 | 3-4 | 3-4 |
| 0300 | Br d/s Derrin Bridge | - | - | - | - | - | - | - |
| 0400 | Br at Carrigan | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | - | 3 |
| 0500 | Legwee bridge | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3 | 3-4 |
| 0700 | Kilsarn bridge | 4 | 4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 |
| 0770 | Dernaferst Bridge | - | - | - | - | - | - | - |
| 0900 | Dingins br. | 3 | 3 | 3 | 3 | 3-4 | 3-4 | 3-4 |
| 1000 | Br east of Corlismore | 4 | 4 | 4 | 4 | 4 | - | 4 |
| 1100 | Bellaheelin Br | 4 | 4 | 4 | 4 | 4 | 3-4 | 3-4 |
| 1200 | 1 km u/s Carrs lake | - | - | - | - | - | - | - |
| 1270 | Drumulig Br near school | - | - | - | - | - | - | - |
| 1300 | Bakers bridge | - | - | - | 3-4 | 3-4 | 3-4 | 3-4 |
| 1400 | Br at Kilconny, Belturbet (RHS) | 3-4 | 3 | 3 | 3 | 3-4 | 3-4 | 3-4 |
| 1410 | Kilconny, Belturbet (LHS) | - | - | 3 | 1 | 1 | - | 3-4 |
| 1430 | D/s Marina LHS | - | - | - | - | - | - | - |
| 1500 | 2.5 km d/s Belturbet | - | - | - | - | - | - | - |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 46 | 29 | 26 | 54 | 33 | 41 | 42 | 72 | 41 | 33 | 41 |
| 0200 | - | - | 29 | 39 | 28 | 62 | 43 | 35 | 56 | 86 | 53 | 42 | 58 |
| 0300 | 62 | 40 | - | - |  | - |  | - | - | - | - | - | - |
| 0400 | - | - | 70 | 70 | 50 | 91 | 60 | 55 | 140 | - | - | - | - |
| 0500 | - | - | 52 | 57 | 46 | 81 | 50 | 50 | 78 | 99 | 71 | 50 | 58 |
| 0700 | 63 | 53 | 60 | 61 | 47 | 70 | 61 | 45 | 73 | 75 | 79 | 40 | 63 |
| 0770 | 14 | 10 | - | - | - | - |  | - | - | - | - | - | - |
| 0900 | - | - | 35 | 22 | 16 | 19 | 15 | 13 | 52 | 52 | 21 | 50 | 53 |
| 1000 | - | - | 42 | 29 | 30 | 40 | 37 | 37 | 40 | - |  | - | - |
| 1100 | 41 | 41 | 37 | 32 | 33 | 47 | 37 | 41 | 61 | 65 | 34 | 48 | 44 |
| 1200 | 41 | 38 | 41 | 31 | 32 | 48 | 40* | 36 | 52 | 56 | 44 | 49 | 30 |
| 1300 | 40 | 23 | 27 | 26 | 23 | 29 | 39 | 51 | 63 | 42 | 38 | 36 | 50 |
| 1400 | 37 | 21 | 36 | 27 | 30 | 37 | 42 | 44 | 62 | 39** | 41 | 31 | 46 |

Assessment

## River is classified as poor to moderate throughout

Change from 2010

Some increase in Phosphorus levels in the upper reaches of this river.

River: Glyde

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Cormey Br. | 0100 | NB_06_602 | Good | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Cormey Br. | 4-5 | 4-5 | 4-5 | 4-5 | 4 | 4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 46 | 41 | 46 | 30 | 36 | 34 | 47 | 43 | 52 | 58 | 54 | 50 | 38 |

Assessment

This river is classified as moderate.

Change from 2010

Some reduction in Phosphorus levels

River: Inny

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Ballinrink br | 0300 | SH_26_2664 | Poor | Good | 2021 |
| Ross bridge | 0400 | SH_26_2664 | Poor | Good | 2021 |
| Finea Bridge | 0500 | SH_26_2660 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’87 | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Ballinrink br | 4 | 3-4 | 4 | 3 | 3 | 3-4 | 3-4 | 4 |
| 0400 | Ross bridge | 4 | - | - | - | - | - | - | - |
| 0500 | Finea Bridge | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | 23 | 19 | 34 | 19 | 16 | 22 | 32 | 24 | 21 | 19 | 10 | 17 | 19 |
| 0500 | 10 | 10 | 10 | 10 | 10 | 16 | 10 | 10 | 10 | 10 | 10 | 10 | 11 |

Assessment

Poor to Moderate. Low levels of Phosphorus at both stations

Change from 2010.

## An improvement in biological quality at 0300 (Ballinrink Br) is followed by unsatisfactory conditions at Finnea

River: Knappagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. U/s Annalee confl | 0700 | - NW_36_1068 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0700 | Br. U/s Annalee confl | 3-4 | 3-4 | 3 | 3-4 | 4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | 21 | 20 | - | - | - | - | - | - | - | - | - |  |  |
| 0600 | 30 | 27 | - | - | - | - | - | - | - | - | - |  |  |
| 0700 | - | - | 22 | 22 | 21 | 24 | 22 | 20 | 29 | 20 | 17 | 21 | 22 |

Assessment

Poor status however Phosphorus levels are low in this river.

Change from 2010

## No significant change.

River: Laragh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 1st bridge d/s L. Acanon | 0015 | - NW_36_1557 | Good | Good | 2009 |
| Br W/SW Knockataggert | 0030 | NW_36_228 | Moderate | Good | 2021 |
| Br 1.5 km E Laragh | 0050 | NW_36_228 | Moderate | Good | 2021 |
| Br. In Laragh | 0080 | NW_36_228 | Moderate | Good | 2021 |
| Shanns br Lisclone | 0200 | NW_36_228 | Moderate | Good | 2021 |
| U/s Annalee confl | 0400 | NW_36_228 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0015 | 1st bridge d/s L. Acanon | 3-4 | 3 | 3 | - | - | - |  |
| 0030 | Br W/SW Knockataggert | - | - | - | - | 4-5 | 4 | 4 |
| 0050 | Br 1.5 km E Laragh | 4-5 | 4-5 | 4 | 4 | 4 | - |  |
| 0080 | Br. In Laragh | 4-5 | 4-5 | 4-5 | 4 | 4 | 3-4 | 4 |
| 0200 | Shanns br Lisclone | 3-4 | 4-5 | 4-5 | 3-4 | 3-4 | - |  |
| 0400 | U/s Annalee confl | 4-5 | 4-5 | 4-5 | 3-4 | 3-4 | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0015 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 16 |  |  |  |  |
| 0030 | - | - |  |  |  |  |  |  | 35 | 25 | 12 | 13 | 19 |
| 0050 | - | - | 21 | 18 | 10 | 19 | 17 | 18 | 22 |  |  |  |  |
| 0080 | - | - | 31 | 30 | 18 | 30 | 26 | 30 | 31 | 31 | 22 | 17 | 28 |
| 0200 | - | - | 28 | 30 | 18 | 29 | 27 | 24 | 29 |  |  |  |  |
| 0400 | 35 | 29 | 28 | 31 | 22 | 32 | 30 | 31 | 35 | 22 | 17 | 15 | 33 |

Assessment

Most of the river stations are within a water body classified as moderate

Change from 2010

Some increase in median orthophosphate levels.

River: Lislea

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 1st bridge u/s L. Ramor | 0100 | EA_07_1368 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 1st bridge u/s L. Ramor | 4 | 4 | 4 | 4 | 4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 25 | 40 | 32 | 34 | 34 | 24 | 30 | 33 | 41 | 32 | 37 | 38 | 37 |

Assessment

Good status.

Change from 2010

No significant change

River: Maudabawn

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br E of Canningstown | 0070 | NW_36_2024 | Moderate | Good | 2021 |
| Br. 1 km S of Drumcondra | 0200 | NW_36_2024 | Moderate | Good | 2021 |
| bridge u/s Annalee river | 0600 | NW_36_2024 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0070 | Br E of Canningstown | 4 | 3-4 | 3-4 | 3 | 3-4 | 3-4 | 3-4 |
| 0200 | Br. 1 km S of Drumcondra | 4 | 4 | 3 | 4 | 4 | 4 | 4 |
| 0600 | bridge u/s Annalee river | 4 | 4 | 4 | 3-4 | 4 | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0070 | - | - | 25 | 27 | 24 | 35 | 23 | 26 | 42 | 53 | 38 | 28 | 32 |
| 0200 | - | - | 36 | 38 | 33 | 44 | 28 | 25 | 45 | 64 | 44 | 41 | 37 |
| 0600 | 33 | 26 | 20 | 30 | 26 | 40 | 23 | 19 | 37 | 60 | 41 | 28 | 35 |

Assessment

## Moderate

Change from 2010

No significant change

River: Mountnugent

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Near Rockville | 0040 | SH_26_2264 | Poor | Good | 2021 |
| bridge se Rockville | 0120 | SH_26_2264 | Poor | Good | 2021 |
| Br.near Derrylea | 0200 | SH_26_2742 | Moderate | Good | 2021 |
| Br. At Kildorra | 0300 | SH_26_2742 | Moderate | Good | 2021 |
| Br E of Killnacrot House | 0350 | SH_26_2742 | Moderate | Good | 2021 |
| Mountnugent br. | 0500 | SH_26_2742 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’87 | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br. Near Rockville | - | 4 | 4-5 | 3-4 | 4 | 4 | - |  |
| 0120 | bridge se Rockville | 4 | - | 3 | 1/0 | - | - | 2-3 | 3 |
| 0200 | Br.near Derrylea | 1 | 4 | 3-4 | 3 | 4-5 | 4 | 3-4 | 3 |
| 0300 | Br. At Kildorra | - | - | - | - | - | - | 3-4 | 3-4 |
| 0350 | Br E of Killnacrot House | 3 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | - |  |
| 0500 | Mountnugent br. | 3 | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | - | - | 39 | 24 | 16 | 23 | 14 | 11 | 21 | - | - | - |  |
| 0120 | - | - | 32 | 28 | 35 | 32 | 25 | 22 | 38 | 78 | 98 | 41 | 67 |
| 0200 | 40 | 33 | 33 | 24 | 29 | 24 | 18* | 11 | 43 | 45 | 43 | 25 | 42 |
| 0300 | 28 | 24 | 29 | 33 | 43 | 48 | 32 | 21 | 57 | 103 | 192 | 146 | 176 |
| 0350 | - | - | 26 | 19 | 19 | 22 | 16 | 13 | 36 | 39 | 54 | 48 | 44 |
| 0400 | 23 | 21 | - | - | - | - | - | - | - | - | - | - | - |
| 0500 | 22 | 24 | 35 | 20 | 18 | 21 | 17 | 12 | 33 | 40 | 46 | 42 | 46 |

Assessment

Unsatisfactory, all stations are classified as poor status

Change from 2010

All biological status station results indicate a polluted river. Increase in Phosphorus levels. Elevated Phosphorus levels at stations 0300 are possibly as a result of licenced activities.

River: Moynalty

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Near Lurganbane | 0050 | EA_07_1128 | Good | Good | 2009 |
| Cloggagh Br | 0070 | EA_07_940 | Moderate | Good | 2027 |
| Skearke Br. | 0090 | EA_07_940 | Moderate | Good | 2027 |
| br. D/s Rosehill Br | 0300 | EA_07_940 | Moderate | Good | 2027 |
| bridge u/s main channel confl | 0400 | EA_07_940 | Moderate | Good | 2027 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | Br. Near Lurganbane | 4 | 4 | 3 | 3-4 | 3-4 | - | - |
| 0070 | Cloggagh Br | 3 | 3 | 3-4 | 3 | 4 | 3-4 | 3-4 |
| 0090 | Skearke Br. | - | 3 | 4 | 3-4 | 4 | - | - |
| 0300 | br. D/s Rosehill Br | 3-4 | 3 | 3 | 3 | 3-4 | 3-4 | 3-4 |
| 0400 | bridge u/s main channel confl | 3 | 3 | 3 | 3 | 3 | - | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | - | - | 77 | 74 | 50 | 74 | 60 | 56 | 69 | - | - |  |  |
| 0070 | - | - | 43 | 45 | 32 | 30 | 38 | 36 | 55 | 61 | 69 | 40 | 47 |
| 0090 | - | - | 33 | 39 | 26 | 30 | 30 | 28 | 32 | - | - |  |  |
| 0200 | 27 | 51 | - | - | - | 25 | - | - | 30 | - | - |  |  |
| 0300 | 37 | 49 | 31 | 29 | 27 | 31 | 25 | 33 | 52 | 71 | 34 | 32 | 31 |
| 0400 | - | - | 134 | 70 | 94 | 72 | 68 | 36 | 114 | 96 | 118 | 146 | 97 |

Assessment

Poor, with the exception of station 0050 all stations are within a water body classified as having a poor ecological status.

Change from 2010

Reduction in Phosphorus levels at station 0400.

River: Nadregeel Lough stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Assan Br. | 0100 | EA_07_1221 | Moderate | Good | 2009 |
| Billis Br | 0200 | EA_07_1221 | Moderate | Good | 2009 |
| Footbridge d/s billis br | 0300 |  |  | Good |  |
| br. SW of Lisgrey X-rds | 0400 |  |  | Good |  |
| Bridge nr Cranadillon | 0500 | EA_07_1357 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Assan Br. | 3-4 | 3-4 | 4-5 | 3-4 | 3-4 | 4 | 3-4 |
| 0200 | Billis Br | 3 | 3 | 3 | 4 | 3-4 | 4 | - |
| 0300 | Footbridge d/s billis br | - | - | - | - | - | - | - |
| 0400 | br. SW of Lisgrey X-rds | 3 | - | - | - | - | - | - |
| 0500 | Bridge nr Cranadillon | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 23 | 29 | 23 | 26 | 18 | 19 | 26 | 24 | 16 | 12 | 27 |
| 0200 | 27 | 22 | 20 | 23 | 21 | 22 | 20 | 20 | 23 | 33 | 21 | 13 | 20 |
| 0500 | 14 | 18 | 16 | 15 | 11 | 16 | 17 | 13 | 25 | 22 | 12 | 16 | 19 |

Assessment

River water bodies are classified as moderate

Change from 2010

Increase in Phosphorus levels at all stations.

River: Owenayle

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br NE of Aughrim | 0050 | SH_26_1095 | Moderate | Good | 2015 |
| Br D/S of Owenayle bridge | 0100 | SH_26_1095 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | Br NE of Aughrim | 4 | 4 | 4 | 4 | 3-4 | 3-4 | 3-4 |
| 0100 | Br D/S of Owenayle bridge | 4 | 4-5 | 4-5 | 4 | 4 | 4 | 4 |

MRP results

| Station | 1999 Med (µgP/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 11 | 12 | 10 | 10 |
| 0100 | 10 | 10 | 10 | 10 | 10 | 10 | 10* | 10 | 10 | 10 | 10 | 10 | 15 |

* Calculated over a two year period

Assessment

Moderate. However chemical results indicate good quality water

Change from 2010

## No change

River: Owenmore (Glangevlin)

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Carty’s Bridge | 0100 | SH_26_3419 | Moderate | Good | 2015 |
| Br. SE Carty’s Bridge | 0120 | SH_26_3419 | Moderate | Good | 2015 |
| U/s bellavally branch Confluence | 0140 | SH_26_3419 | Moderate | Good | 2015 |
| Br. NE Moneensauran | 0160 | SH_26_3273 | Good | Good | 2009 |
| B W of Legnacrow L | 0300 | SH_26_3539 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Carty’s Bridge | 4-5 | 4-5 | 5 | 4-5 | 4-5 | 3-4 | 4 |
| 0120 | Br. SE Carty’s Bridge | 4-5 | 5 | 4-5 | 5 | - | - | - |
| 0140 | U/s bellavally branch Confluence | 4 | 4-5 | 4-5 | 4-5 | - | - | - |
| 0160 | Br. NE Moneensauran | - | 4 | - | - | - | - | - |
| 0300 | B W of Legnacrow L | 4-5 | 5 | 4-5 | 4-5 | 3-4 | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0120 | - | - | 10 | 10 | 10 | 11 | 10 | 10 | 10 | 10 | 10 | 10 | 12 |
| 0140 | - | - | 10 | 10 | 10 | 10 | 13 | 23 | 10 | 10 | 10 | 10 | 10 |
| 0160 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | - | - | - | - |
| 0300 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |

Assessment

River water bodies are generally moderate. Chemical results indicate low Phosphorus levels.

Change from 2010

## Welcome increase in Q values at both stations.

River: Owensallagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br E of Drumcor | 0500 | - NW_36_1161 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Br E of Drumcor | 5 | 5 | 3-4 | 4-5 | 4-5 | 4 | 4 |

MRP results

| Station | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | 10 | 10 | 10 | 13 | 13 | 15 | 10 | 11 | 10 | 10 | 10 |

Assessment

River is in good condition.

Change from 2010

## No significant change.

River: Rag

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br D/S Clonty | 0200 | NW_36_550 | Good | Good | 2009 |
| Br D/S Togher | 0400 | NW_36_550 | Good | Good | 2009 |
| Br D/S Aghalane | 0500 | NW_36_550 | Good | Good | 2009 |
| Br. Nr Killywilly house | 0600 | - NW_36_1025 | Good | Good | 2009 |
| Br u/s Tee lough lr | 0800 | XB_36_east_4 | Poor | Good | 2021 |
| Bridge d/s Tee lough lr | 0900 |  |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br D/S Clonty | - | - | - | - | - | - | - | - |
| 0400 | Br D/S Togher | - | - | - | - | - | - | - | - |
| 0500 | Br D/S Aghalane | - | - | - | - | - | - | - | - |
| 0600 | Br. Nr Killywilly house | 3 | 3 | 3 | - | 3-4 | 3-4 | - | 3 |
| 0800 | Br u/s Tee lough lr | 2-3 | 3 | 3 | 3-4 | 4 | 4 | 4 | 4 |
| 0900 | Bridge d/s Tee lough lr | 3 | 3 | 3 | - | - | - | - | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | - | 24 | 34 | 27 | 38 | 31 | 28 | 27 | 41 | - | - | - |  |
| 0400 | - | 30 | 42 | 37 | 41 | 25 | 36* | 51 | 44 | - | - | - |  |
| 0500 | - | 27 | 34 | 16 | 38 | 23 | 25 | 28 | 42 | - | - | - |  |
| 0600 | 40 | 29 | 44 | 38 | 24 | 29 | 24 | 31 | 42 | - | - | - |  |
| 0800 | - | - | 40 | 36 | 22 | 20 | 38 | 43 | 43 | 29 | 34 | 35 | 28 |
| 0900 | - | - | 26 | 32 | 26 | 29 | 30 | 37 | 57 | - | - | - |  |

#### Assessment

Generally good, however station 0800 is in a poor quality water body

Change from 2010

No significant change

River: Roo

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. W Barrran | 0200 | NW_36_2114 | High | Good | 2009 |
| Br SE of Thornhill house | 0400 | NW_36_2114 |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br. W Barrran | 4-5 | 4 | 4 | 5 | 5 | 5 |
| 0400 | Br SE of Thornhill house | - | - | 3-4 | - | - |  |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | 10 | 12 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 14 |

Assessment

River is in good condition.

Change from 2010

## No significant change.

River: Shannon

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Shannon Br, Derrylahan | 0100 | SH_26_3539 | Moderate | Good | 2015 |
| Metal Br. S of Tullynafreave | 0200 | SH_26_3539 | Moderate | Good | 2015 |
| Dowra Br | 0300 | SH_26_3539 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Shannon Br, Derrylahan | 4 | 4-5 | 4 | 4 | 4 | 3-4 | 4-5 |
| 0200 | Metal Br. S of Tullynafreave | 5 | 5 | 4-5 | 4-5 | - | 4-5 | 4-5 |
| 0300 | Dowra Br | 5 | 4-5 | 4-5 | 5 | 4 | 4-5 | 4-5 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 10 | 13 | 10 | 10 | 11 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0200 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0300 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |

Assessment

Moderate status, however chemical results indicate low Phosphorus levels.

Change from 2010

## A return to highly satisfactory ecological conditions in the three stations

River: Sheelin Feeder Streams

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status Station | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- | --- |
| Belsgrove bridge | 01 |  |  |  |  |  |
| Crover Bridge | 02 |  |  |  |  |  |
| Schoolhouse Bridge | 04 |  |  |  |  |  |
| Belsgrove(Ballynakerry) | 08 |  |  |  |  |  |

### No Biological Quality Ratings available

Sampling stations and Biological Quality Ratings

| Station Number | Station Location | Q’96 |
| --- | --- | --- |
| 01 | Belsgrove bridge | - |
| 02 | Crover Bridge | - |
| 04 | Schoolhouse Bridge | - |
| 08 | Belsgrove(Ballynakerry) | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 24 | 36 | 22 | 19 | 17 | 26 | 18 | 20 | 34 | 39 | 18 | 19 | 10 |
| 02 | 29 | 31 | 57 | 34 | 47 | 61 | 29 | 27 | 33 | 33 | 23 | 26 | 36 |
| 04 | 34 | 32 | 37 | 27 | 17 | 21 | 25 | 15 | 29 | 30 | 13 | 15 | 22 |
| 08 | - | - | - | - | 21 | 31 | 18 | 31 | 26 | 10 | 24 | 33 | 45 |

Assessment

Status has not been assigned under the WFD.

Change from 2010

## No significant change

Note : These rivers are monitored as part of the councils monitoring programme for Lough Sheelin

River: Stradone

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 2nd Br. D/s Lisnanagh L | 0075 | NW_36_245 | Good | Good | 2009 |
| Br u/s Laragh R confl | 0200 | NW_36_245 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0075 | 2nd Br. D/s Lisnanagh L | 3-4 | 4 | 3-4 | 4 | 4 | - | 4 |
| 0200 | Br u/s Laragh R confl | 4-5 | 4 | 3-4 | 4 | 4 | 4 | 4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0075 | - | - | 40 | 35 | 26 | 29 | 27 | 23 | 46 | 37 | 10 | 25 | 32 |
| 0200 | 38 | 38 | 33 | 32 | 27 | 25 | 25 | 23 | 40 | 32 | 17 | 25 | 30 |

Assessment

Good

Change from 2010

Slight increase in Phosphorus levels.

River: Swanlinbar

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Nr Altbrean | 0100 | NW_36_18 | Good | Good | 2015 |
| 0.6km d/s Swanlinbar br | 0300 | NW_36_18 | Good | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. Nr Altbrean | 5 | 5 | 5 | - | 4-5 | 4-5 | 4-5 |
| 0300 | 0.6km d/s Swanlinbar br | 4-5 | 4-5 | 4-5 | - | 3-4 | 4 | 4-5 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 |  | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0300 | 14 | 13 | 14 | 13 | 11 | 12 | 14 | 10 | 11 | 10 | 10 | 10 | 10 |

Assessment

Good. Chemical results indicate low Phosphorus levels.

Change from 2010

No change

River: Templeport Lough Stream

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Templeport Lough | 0600 | NW_36_1177 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | Br u/s Templeport Lough | 3 | 3 | 4 | 3 | 3 | 4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l | 2007 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 16 | 15 | 14 | 20 | 12 | 20 | 10 | 16 | 13 |

Assessment

## Good.

Change from 2010

Not monitored chemically in 2011.

River: Woodford

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Ballyconnell Quay | 0400 | XB_36_east_5 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’85 | Q’89 |
| --- | --- | --- | --- |
| 0400 | Ballyconnell Quay | 4-5 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | 17 | 12 | 15 | 17 | 11 | 20 | 14 | 12 | 13 | 10 | 10 | 10 | 15 |

* Baseline Q rating on this station based on ‘89 Q rating

Assessment

## Good

Change from 2010

No significant change

4	Lake Water Quality

Lakes are listed in alphabetical order.

#### Summary Data

Lake name and catchment:

River name and catchment: Included are the lake name and the catchment name (namely: the Erne, the Boyne, the Shannon and the Glyde). Sub-catchments and water management units are not listed.

The Lake water body code is given for each lake. The Current Ecological status as defined is given with target ecological status. The target ecological status cannot be lower than Good. A target date for reaching the target ecological status is included for each station.

#### Annual Status

This table indicates the annual status of the lake in question and includes

- Average annual Total P (g/l),

- Maximum Chlorophyll,

#### Assessment

A summary assessment of the water quality is given which is based on the scientific data.

Change from 2010

Where applicable any significant changes in quality from 2010 are also given.

List of  Lakes Monitored

Lake 						Catchment

Acurry	Boyne

Annagh	Erne

Ardan	Erne

Atrain	Erne

Bawn	Erne

Bunerky	Erne

Culliaghan	Erne

Derrycassan	Erne

Drumkeery	Boyne

Farnham	Erne

Garty	Erne

Glasshouse	Erne

Gowna	Erne

Kinale	Shannon

Mill Lough	Erne

Nadrageel	Boyne

Oughter	Erne

Ramor	Boyne

Lough Sheelin 	Shannon

Sillan	Erne

Lough Tacker 	Erne

Lake: Accury

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_242 | Good | Good | 2009 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 10 | 12 |
| 2008 | 21 | 10 |
| 2009 | 44 | 30 |
| 2010 | 31 | 22 |
| 2011 | 35 | 233 |

Assessment

## Good status, under the new RBD classification. Generally good quality trout lake.

Change from 2010

Unusually high maximum chlorphyll levels in June 2011.

Lake: Annagh

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_517 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 9 | 24 |
| 2008 | 29 | 18 |
| 2009 | 44 | 24 |
| 2010 | 40 | 14 |
| 2011 | 32 | 12 |

Assessment

Moderate status, under the RBD classification. Generally good water abstraction lake

Change from 2010

Slight reduction in both Phosphorus levels and maximum chlorphyll levels.

Lake: Ardan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_432 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 10 | 12 |
| 2008 | 21 | 10 |
| 2009 | 44 | 30 |
| 2010 | 71 | 18 |
| 2011 | 39 | 10 |

Assessment

Moderate status, under the new RBD classification

Change from 2010

Welcome decrease in Phosphorus levels, with a reduction in the Maximum Chlorophyll level recorded

Lake: Atrain

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_618 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2010 | 55 | 24 |
| 2011 | 63 | 28 |

Assessment

Moderate status, under the RBD classification

Change from 2010

No significant change

Lake: Bawn

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_573 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 13 | 10 |
| 2008 | 41 | 4 |
| 2009 | 79 | 6 |
| 2010 | 63 | 16 |
| 2011 | 53 | 24 |

Assessment

Moderate status, under the RBD classification

Change from 2010

Slight increase in Maximum chlorophyll levels

Lake: Bunerky

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_624 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 29 | 22 |
| 2008 | 26 | 10 |
| 2009 | 37 | 12 |
| 2010 | 34 | 10 |
| 2011 | 53 | 12 |

Assessment

Moderate status, under the RBD classification

Change from 2010

Increase in Phosphorus and Maximum Chlorophyll.

Lake: Culliaghan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_385 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 22 | 8 |
| 2008 | 51 | 6 |
| 2009 | 46 | 6 |
| 2010 | 87 | 44 |
| 2011 | 49 | 8 |

Assessment

Moderate status, under the RBD classification

Change from 2009

Significant reduction in total Phosphorus levels and maximum chlorophyll levels

Lake: Derrycasson

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_514 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 14 | 12 |
| 2008 | 34 | 12 |
| 2009 | 45 | 10 |
| 2010 | 34 | 24 |
| 2011 | 40 | 12 |

Assessment

Moderate status, under the RBD classification

Change from 2010

Some reduction in Chlorophyll level.

Lake: Drumkeery

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_268 | Poor | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 30 | 34 |
| 2008 | 65 | 40 |
| 2009 | 48 | 16 |
| 2010 | 44 | 30 |
| 2011 | 100 | 207 |

Assessment

## Poor.

Change from 2010

Significant increase in Phosphorus and Chlorophyll levels. High chlorophyll result  occurred in September 2011.

Lake: Farnham

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_618 | High | High | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2010 | 265 | 260 |
| 2011 | 371 | 191 |

Assessment

High status, under the new RBD classification, however this status is based on extrapolated results and does not appear to accurately reflect the water quality of the lake. Current data indicates that this lake is polluted.

Change from 2010

Increase in Phosphorus levels.

Lake: Garty

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_430 | Poor | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 21 | 10 |
| 2008 | 39 | 8 |
| 2009 | 44 | 16 |
| 2010 | 50 | 181 |
| 2011 | 63 | 40 |

Assessment

## Poor.

Change from 2010

Increase in Phosphorus levels

Lake: Glasshouse

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_615 | Bad | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 15 | 10 |
| 2008 | 38 | 22 |
| 2009 | 70 | 20 |
| 2010 | 42 | 26 |
| 2011 | 52 | 36 |

Assessment

## Current Bad status is as a result of the macrophyte classification.

Change from 2009

Increase in Phosphorus and Total Chlorophyll

Lake: Gowna

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_524 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 61 | 121 |
| 2001 | 47 | 107 |
| 2002 | 72 | 56 |
| 2003 | 76 | 52 |
| 2004* | 72 | 125 |
| 2005** | 68 | 125 |
| 2006 | 60 | 56 |
| 2007 | 31 | 48 |
| 2008 | 42 | 30 |
| 2009 | 53 | 99 |
| 2010 | 61 | 137 |
| 2011 | 54 | 93 |

* Based on 9 samples

** Average and maximum values calculated over a two year period

Assessment

## Moderate.

Change from 2010

Reduction in maximum chlorophyll levels and Phosphorus.

Lake: Kinale

Catchment: Shannon

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| SH_26_430 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 28 | 77 |
| 2001 | 24 | 56 |
| 2002 | 27 | 26 |
| 2003 | 28 | 18 |
| 2004 | 25 | 6 |
| 2005 | 26 | 24 |
| 2006 | 17 | 10 |
| 2007 | 10 | 10 |
| 2008 | 17 | 4 |
| 2009 | 20 | 15 |
| 2010 | 33 | 8 |
| 2011 | 32 | 6 |

Assessment

Moderate status, under the new RBD classification, however physicochemical data is good. The water quality of this lake is inextricably linked to the quality of water in Lough Sheelin.

Change from 2010

No change

Lake: Mill lough

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_597 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 7 | 18 |
| 2008 | 22 | 28 |
| 2009 | 35 | 16 |
| 2010 | 62 | 57 |
| 2011 | 44 | 12 |

Assessment

Moderate status.

Change from 2010

Reduction in Phosphorus levels and maximum chlorophyll

Lake: Nadrageel

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_273 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 47 | 103 |
| 2001 | 47 | 85 |
| 2002 | 54 | 118 |
| 2003 | 59 | 111 |
| 2004 | 64 | 71 |
| 2005 | 49 | 50 |
| 2006 | 44 | 92 |
| 2007 | 31 | 38 |
| 2008 | 44 | 40 |
| 2009 | 55 | 28 |
| 2010 | 61 | 34 |
| 2011 | 41 | 22 |

Assessment

Moderate status.

Change from 2010

Reduction in Phosphorus and Chlorophyll levels

Lake: Oughter

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_657 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 73 | 183 |
| 2001 | 64 | 129 |
| 2002 | 70 | 494 |
| 2003 | 94 | 187 |
| 2004 | 101 | 232 |
| 2005** | 90 | 232 |
| 2006 | 82 | 86 |
| 2007 | 54 | 92 |
| 2008 | 51 | 244# |
| 2009 | 76 | 62 |
| 2010 | 78 | 198 |
| 2011 | 76 | 171 |

** Average and Maximum values calculated over a two year period

# next nearest value was 38

Assessment

Moderate status.

Change from 2010

No significant change

Lake: Ramor

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_275 | Bad | Good | 2027 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 67 | 90 |
| 2001 | 59 | 92 |
| 2002 | 73 | 80 |
| 2003 | 100 | 165 |
| 2004 | 68 | 75 |
| 2005 | 74 | 80 |
| 2006 | 92 | 102 |
| 2007 | 58 | 83 |
| 2008 | 52 | 110 |
| 2009 | 70 | 69 |
| 2010 | 76 | 36 |
| 2011 | 69 | 54 |

Assessment

## Bad status.

Change from 2010

Increase in maximum chlorophyll levels

Lake: Lough Sheelin

Catchment: Shannon

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| SH_26_709 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 32 | 36 |
| 2001 | 23 | 34 |
| 2002 | 28 | 32 |
| 2003 | 31 | 46 |
| 2004 | 32 | 62 |
| 2005* | 25 | 14 |
| 2006 | 25 | 8 |
| 2007 | 18 | 24 |
| 2008 | 28 | 14 |
| 2009 | 26 | 12 |
| 2010 | 30 | 4 |
| 2011 | 29 | 12 |

*includes one sample from January 2006

Assessment

Moderate status.

Change from 2010

No significant change

Lake: Sillan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_528 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 77 | 73 |
| 2001 | 57 | 50 |
| 2002 | 56 | 73 |
| 2003 | 66 | 52 |
| 2004 | 97 | 62 |
| 2005** | 103 | 74 |
| 2006 | 117 | 68 |
| 2007 | 46 | 66 |
| 2008 | 79 | 121 |
| 2009 | 73 | 54 |
| 2010 | 102 | 52 |
| 2011 | 100 | 46 |

** Average and Maximum values calculated over a two year period

Assessment

Moderate status.

Change from 2010

No significant change

Lake: Lough Tacker

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_363 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 47 | 32 |
| 2008 | 69 | 22 |
| 2009 | 69 | 16 |
| 2010 | 91 | 30 |
| 2011 | 94 | 30 |

Assessment

Moderate status.

Change from 2010

No significant change