Comhairle Chontae an Cabhain

CAVAN  COUNTY  COUNCIL

THE QUALITY OF RIVER & LAKE WATER IN COUNTY CAVAN

###### A Report for the Year 2012

J Keyes				E Doyle				Environment Section

County Manager			Director of Services	 		Cavan County Council

Environment				August 2013

### Index

Page

- Introduction				3

- River Water Quality				06

- Lake Water Quality				48

2.	Introduction

This report presents the summary monitoring data for 2012 for the rivers and lakes in County Cavan along with historical data; it can be viewed as a companion volume to those previously produced.

All chemical monitoring was undertaken by the Council’s Environment Section. Monitoring complies with the requirements of the monitoring programme prescribed by the EPA in compliance with the requirements of the Water Framework Directive(WFD). Approximately 96 river stations and 21 lakes are monitored. The monitoring programme allows comparison, on an annual basis, of the chemical quality of the rivers and lakes.

The Biological monitoring referred to in this report on river stations was undertaken by the EPA as part of its National Programme of Biological Assessment of River Water Quality. The EPA’s contribution is gratefully acknowledged.

The lake monitoring programme involves the monitoring of 21 lakes designated by the EPA. The number of stations monitored on each lake varies depending on a number of factors including lake size, historical data and uniformity of water body. Samples are taken at time intervals specified by the EPA. This involves a minimum of four samples taken once in April and three times between July and October.

The classification system of quality standards introduced in 2009 is defined in S.I. No. 272 of 2009 the EUROPEAN COMMUNITIES ENVIRONMENTAL OBJECTIVES (SURFACE WATERS) REGULATIONS 2009. The Classification tables are defined in schedules 4, 5 and 6 over 13 pages of these regulations.

In order to allow the reader assess the information regarding the quality of river water three tables are shown below.

Tables A and B set out the standards specified in the Local Government (Water Pollution) Act, 1977 (Water Quality Standards for Phosphorus), 1998 for rivers and lakes respectively. These regulations identify biological ratings and Molybdate-Reactive Phosphorus Annual Median concentrations for the rivers, and Chlorophyll and Total Phosphorus levels for the lakes, either of which must be at least maintained or improved in order to meet the relevant specified target standard.

Note: The Phosphorus regulations have been revoked however this information is provided as an aide to interpretation of the results

Table C sets out the standard specified in EC directive 78/659/EEC (Freshwater Fish).

Table A: ‘Phosphorus Regs’, Local Government (Water Pollution) Act, 1977 (Water Quality Standards for Rivers), 1998

| Existing Biological Quality Q Rating | Molybdate -Reactive Phosphorus annual Median concentration (mg/l P) | Target Biological Quality (Q) Rating |
| --- | --- | --- |
| 5 } 4-5 } Unpolluted 4 } | 0.015 0.020 0.030 | 5 4-5 4 |
| 3-4 }Slightly Polluted | 0.030 | 4 |
| 3 } Moderately 2-3 } Polluted | 0.05 0.070 | 3-4 3 |
| < 2 } Seriously Polluted | 0.070 | 3 |

Table B: ‘Phosphorus Regs’, Trophic Classification System for Lakes

| Lake Trophic Category | Target Trophic Status | Total Phosphorus Average Concentration (g P/l) | Annual Max. Chlorophyll (mg/m3) |
| --- | --- | --- | --- |
| Ultra-Oligotrophic | Ultra-Oligotrophic |  5 | < 2.5 |
| ### Oligotrophic | ### Oligotrophic | > 5  10 | > 2.5 < 8 |
| Mesotrophic | Mesotrophic | > 10  20 |  8 < 25 |
| Eutrophic | Mesotrophic | > 10  20 |  8 < 25 |
| Hypertrophic | Eutrophic | > 20  50 |  25 < 75 |

Table C: EC Directive 78/659/EEC (Freshwater Fish)

| Parameter | ### Units | Salmonid | ### Waters | ### Cyprinid | Waters |
| --- | --- | --- | --- | --- | --- |
|  |  | Guide | #### Mandatory | Guide | #### Mandatory |
| Total Ammonia | mg/l N | < 0.03 | < 0.78 | < 0.16 | < 0.78 |
| B.O.D. | mg/l O2 | 100% < 3 |  | 100% < 6 |  |
| D.O. | mg/l O2 | 50% > 9 | 50% > 9 | 50% > 8 | 50% > 7 |
| Total Phosphorus | mg/l P |  | 0.065 |  | 0.13 |
| PH | pH units |  | 6-9 |  | 6-9 |

3.	River Water Quality

Rivers are listed in alphabetical order.

River name and catchment: Included are the river name and the catchment name (namely: the Erne, the Boyne, the Shannon and the Glyde). Sub-catchments and water management units are not listed.

Sampling stations: Sampling stations are numbered according to the EPA assignment. In the case where the EPA has not assigned a sampling station number, no number is included. Stations are listed starting upstream and leading downstream and include stations that are biologically assessed. It is not always possible to use the same location for both chemical and biological assessment, as the requirements for both are different. A brief location description for each station is also given. The water body code is given for each station, and there may be more than one individual station in a single water body. The current ecological status as defined is given with target ecological status. The target ecological status cannot be lower than Good. A target date for reaching the target ecological status is included for each station.

Biological Quality Ratings: This report includes recent and historical data from the EPA’s National Biological River Monitoring Programme. The Q ratings for 2012 are provisional. The biotic index classification is listed in Table A.

MRP results

Median annual MRP results from 1999-2012 are included in the summary data.

#### Assessment

A summary assessment of the water quality is given.

Change from 2012

Any significant changes in quality from 2011 are also given.

Note: The analysis of MRP involves a specific procedure. Analytical procedures are designed to accurately detect concentration levels over a specific range. The upper and lower limits of this range are assigned as limits of quantification. The lower limit of quantification for MRP is assigned at 10µg P/l. In this section, if a result of  10µg P/l is recorded, this is reported as 10 µg P/l.

##### List of Rivers Monitored

River 							Catchment

Annalee							Erne

Ballinagh							Erne

Bawnboy 							Erne

Blackwater (Kells)						Boyne

Blackwater (Newtowngore)				Erne

Blackwater (Swanlinbar)					Erne

Bunnoe							Erne

Cavan								Erne

Chapel Lake Stream					Boyne

Cullies							Erne

Dee								Glyde

Dromore							Erne

Derrada		Erne

Drumane							Erne

Drumkeery Lough stream				Boyne

Drumsallagh						Glyde

Erne								Erne

Glyde								Glyde

Inny								Shannon

Knappagh							Erne

Laragh							Erne

Lislea								Boyne

Maudabawn							Erne

Mountnugent						Shannon

Moynalty							Boyne

Nadregeel Lough Stream					Boyne

Owenayle							Shannon

Owenmore (Glangevlin)					Shannon

Owensallagh						Erne

Rag								Erne

Roo								Erne

Shannon							Shannon

Sheelin Feeder Streams					Shannon

Stradone							Erne

Swanlinbar							Erne

Templeport Lough Stream				Erne

Woodford							Erne

River: Annalee

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 2nd br. u/s L.Sillan | 0080 | NW_36_1947 | Moderate | Good | 2021 |
| 2nd br. d/s L. Sillan | 0150 | NW_36_1181 | Poor | Good | 2021 |
| 1st br. d/s L. Tacker | 0250 | NW_36_1072 | Good | Good | 2009 |
| Br. Nr Anns fort | 0350 | - NW_36_1102 | Moderate | Good | 2021 |
| New Grove Br. | 0500 | - NW_36_1102 | Moderate | Good | 2021 |
| Br SE of Fort William | 0600 | - NW_36_1102 | Moderate | Good | 2021 |
| Br.NW of Rakenny Ho | 0800 | NW_36_2417 | Poor | Good | 2021 |
| Ballynallon Br | 0900 | NW_36_2417 | Poor | Good | 2021 |
| Ballyhaise Br. | 1000 | NW_36_2417 | Poor | Good | 2021 |
| Br. Nr Curraghanoe | 1150 | NW_36_2417 | Poor | Good | 2021 |
| 0.2Km u/s Cavan R confl | 1350 | NW_36_2417 | Poor | Good | 2021 |
| 0.2km d/s Cavan R confl | 1400 | NW_36_2417 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0080 | 2nd br. u/s L.Sillan | 3-4 | 3-4 | 3-4 | 4-5 | 3-4 | 4 | 3-4 | 4 |
| 0150 | 2nd br. d/s L. Sillan | - | 3-4 | 3 | 3 | 3 | 3 | 3 | 3 |
| 0250 | 1st br. d/s L. Tacker | 3-4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 | - | - |
| 0350 | Br. Nr Anns fort | 4 | 4 | 4 | 3-4 | 4 | 4 | 4 | 4 |
| 0500 | New Grove Br. | 4-5 | 4-5 | 4-5 | 3 | 4 | 4 | 4 | 4 |
| 0600 | Br SE of Fort William | 2-3 | 4 | 4-5 | 3-4 | 3-4 | 4 | 3-4 | 3-4 |
| 0800 | Br.NW of Rakenny Ho | 3-4 | 4-5 | 4-5 | 4 | 4-5 | 4 | 4 | 4 |
| 0900 | Ballynallon Br | 4 | 4 | 4 | 4 | 3-4 | 4 | 3-4 | 4 |
| 1000 | Ballyhaise Br. | 3-4 | 4 | 4 | 4 | 3-4 | 3-4 | 3-4 | 3-4 |
| 1150 | Br. Nr Curraghanoe | 3-4 | 4-5 | 4-5 | 4 | 3-4 | 4-5 | - | 4 |
| 1350 | 0.2Km u/s Cavan R confl | 4-5 | 4 | 3-4 | 4 | 3-4 | 3-4 | - | 3-4 |
| 1400 | 0.2km d/s Cavan R confl | 3-4 | 4 | 3-4 | 3-4 | 3 | 3-4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0080 | NM* | NM | 24 | 17 | 17 | 17 | 19 | 15 | 20 | 30 | 16 | 12 | 16 | 23 |
| 0150 | 60 | 50 | 47 | 30 | 36 | 34 | 61 | 62 | 61 | 63 | 24 | 40 | 45 | 23 |
| 0250 | NM | NM | 41 | 24 | 24 | 30 | 25 | 60 | 43 | - | - | - | - |  |
| 0350 | NM | NM | 35 | 21 | 24 | 35 | 33 | 23 | 44 | 30 | 24 | 30 | 21 | 23 |
| 0500 | 38 | 31 | 36 | 27 | 28 | 38 | 24 | 35 | 41 | 49 | 31 | 28 | 30 | 71 |
| 0600 | NM | NM | 32 | 30 | 23 | 35 | 34 | 35 | 49 | 50 | 56 | 33 | 33 | 37 |
| 0800 |  |  |  |  |  |  |  |  |  | 47 | 53 | 57 | 36 | 43 |
| 0900 | 52 | 42 | 40 | 40 | 24 | 42 | 41 | 35 | 44 | 45 | 56 | 54 | 38 | 41 |
| 1000 | NM | NM | 39 | 41 | 36 | 41 | 38 | 31 | 50 | 46 | 53 | 52 | 39 | 39 |
| 1150 | NM | NM | 39 | 41 | 35 | 40 | 36 | 34 | 33 | - | - | - | - |  |
| 1350 | 41 | 40 | 41 | 41 | 35 | 40 | 38 | 38 | 47 | 41 | 36 | 51 | 39 | 42 |
| 1400 | NM | NM | 55 | 47 | 43 | 45 | 40 | 37 | 53 | 29 | 40 | 67 | 44 | 44 |

*Not Measured

Assessment

This river ranges from poor through to good ecological status. The majority of stations are poor or moderate.

Change from 2012

Some increase in median ortho-phosphate levels at new grove bridge.

River: Ballinagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Ballinagh Bridge | 0200 | NW_36_2100 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | ### Station Location | Q’86 | Q’89 |
| --- | --- | --- | --- |
| 0200 | Ballinagh Bridge | 3 | 2-3 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | 114 | 109 | 128 | 108 | 152 | 117 | 96 | 87 | 112 | 81 | 127 | 87 | 77 | 103 |

Assessment

The current ecological status is good. The water quality is poor according to chemical data.

Change from 2011

Increase in median Ortho-P levels.

River: Bawnboy

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br U/S Brackley Lough | 0300 | - NW_36_1034 | Good | Good | 2009 |
| Br U/S Bellaboy Lough | 0500 | - NW_36_1034 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Br U/S Brackley Lough | 4-5 | 4-5 | 5 | NM | 4-5 | 4 | 4 |
| 0500 | Br U/S Bellaboy Lough | 4-5 | 4 | 4 | 4 | 4 | 4 | 3 |

MRP results

| Station | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | NM | 13 | 10 | 10 | 10 | 11 | 11 | 10 | 10 | 12 | 10 | 10 | 10 |
| 0500 | NM | 19 | 13 | 10 | 12 | 15 | 14 | 10 | 15 | 10 | 10 | 12 | 10 |

Assessment

The ecological status is good and the chemical results correspond with this status.

Change from 2012

No significant change.

River: Blackwater (Kells)

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. 2km NE of Balieborough | 0100 | EA_07_581 | Poor | Good | 2015 |
| U/S discharge pipe at Drumbannon | 0160 |  |  | Good |  |
| Br.at Drumbannon | 0170 | EA_07_581 | Poor | Good | 2015 |
| Footbridge u/s Castle lake | 0200 | EA_07_581 | Poor | Good | 2015 |
| 3rd bridge d/s Castle Lake | 0280 | EA_07_581 | Poor | Good | 2015 |
| Br. D/s parkers br | 0420 | EA_07_581 | Poor | Good | 2015 |
| Br in Killenkere | 0500 | EA_07_1035 | Moderate | Good | 2009 |
| Stramaquerty Br (nr Drumagolan) | 0600 | EA_07_1035 | Moderate | Good | 2009 |
| Br. u/s L. Ramor | 0800 | EA_07_1035 | Moderate | Good | 2009 |
| Nine eyes Br | 0900 | EA_07_1536_1 | Poor | Good | 2027 |
| Daly’s bridge | 1000 | EA_07_1536_1 | Poor | Good | 2027 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 | Q’12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. 2km NE of Balieborough | - | 4 | 4 | 4-5 | 4-5 | 4 | 4 | 4 |
| 0160 | Upstream discharge pipe at Drumbannon | - | - | - | - | - | - | - |  |
| 0170 | Br.at Drumbannon | 1 | 2 | 2-3 | 2 | 2-3 | 2-3 | 3 | 3-4 |
| 0200 | Footbridge u/s Castle lake | 1 | 2 | - | - |  | - | - | 2-3 |
| 0280 | 3rd bridge d/s Castle Lake | 3 | 3 | 3 | 3 | 3 | 3 | - | 3 |
| 0420 | Br. D/s parkers br | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 | 3-4 |
| 0500 | Br in Killenkere | 4 | 3-4 | 3-4 | 3-4 | 4 | 4 | 4 | 4 |
| 0600 | Stramaquerty Br (nr Drumagolan) | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 | 4 | 4 |
| 0800 | Br. u/s Lough Ramor | 4 | 3-4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 0900 | Nine eyes Br | 3 | 3 | 3 | 3 | 3 | 3 | - | - |
| 1000 | Daly’s bridge | 3-4 | 3 | 3 | 3-4 | 3 | 3 | 3 | 3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 32 | 18 | 19 | 17 | 10 | 10 | 23 | 17 | 13 | 19 | 15 | 12 | 20 | 16 |
| 0160 | - | - | - | - | 69 | 51 | 71 | 82 | 90 | 105 | 158 | 42 | 34 | 28 |
| 0170 | NM | NM | 40 | 53 | 65 | 59 | 47 | 119 | 99 | 104 | 91 | 42 | 35 | 28 |
| 0200 | 55 | 38 | - | - | - | - | - | - | - | - | - |  | - | - |
| 0280 | NM | NM | 26 | 24 | 17 | 24 | 29 | 38 | 55 | 101 | - |  | - | - |
| 0300 |  |  |  |  |  |  |  |  |  |  |  | 46 | 30 | 46 |
| 0400 | NM | NM | 16 | 15 | 20 | 26 | 22 | 30 | 49 | 26 | 17 | 16 | 26 | 35 |
| 0450 |  |  |  |  |  |  |  |  |  | 30 | 23 | 17 | 27 | 33 |
| 0500 | 23 | 23 | 34 | 20 | 15 | 27 | 23 | 30 | 48 | 24 | 30 | 22 | 28 | 37 |
| 0600 | NM | NM | 16 | 21 | 16 | 19 | 25 | 26 | 42 | 20 | 27 | 23 | 27 | 37 |
| 0800 | 29 | 28 | 19 | 25 | 15 | 19 | 20 | 20 | 38 | 19 | 29 | 23 | 32 | 40 |
| 0900 | NM | NM | 17 | 15 | 10 | 10 | 32 | 25 | 30 | - | - |  | - |  |
| 1000 | 10 | 23 | 22 | 15 | 14 | 10 | 23 | 17 | 45 | 35 | 23 | 15 | 16 | 22 |

Assessment

The river is in general poor based on the ecological status at the upper reaches near Baileborough and is classified as poor until Killenkere. The stretch from Killenkere to Lough Ramor is classified as moderate. Two stations in the lower reaches downstream of Lough Ramor are poor. This is probably as a result of lake effects.

Change from 2011

Continuing improvements in the upper reaches.

River: Blackwater (Newtowngore)

Catchment: Erne

Sampling Stations and Biological Quality Ratings

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br west of Knockmore | 0180 | NW_36_323 | Good | Good | 2009 |
| Br. NE of Stadiran | 0200 | NW_36_323 | Good | Good | 2009 |
| Br. D/s Blackwater Br. | 0400 | NW_36_148 | Moderate | Good | 2021 |
| Br. U/s Ballymagauran Lough | 0600 | - | - | - | - |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0180 | Br west of Knockmore | 5 | 5 | 4-5 | 5 | 4-5 | 4 | 4 |
| 0200 | Br. NE of Stadiran | 4-5 | 4-5 | 4 | 5 | 4 | 4 | 4-5 |
| 0400 | Br. D/s Blackwater Br. | 5 | 5 | 4 | - | 4 | 3-4 | 4 |
| 0600 | Br. U/s Ballymagauran L | 4 | 4 | 4-5 | - | 4 | - | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg p/l) | 2006 Med (µg p/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0180 |  |  |  |  |  |  |  |  | 10 | 10 | 10 | 10 | 18 | 20 |
| 0200 | NM | NM | 13 | 11 | 10 | 12 | 10 | 10 | 10 | 15 | 12 | 10 | 10 | 16 |
| 0400 | 12 | 11 | 13 | 14 | 10 | 12 | 10 | 10 | 14 | 16 | 10 | 10 | 10 | 13 |
| 0600 | NM | NM | 15 | 14 | 10 | 12 | 11 | 10 | 15 | - | - | - | - | - |

Assessment

The two stations in the upper reaches are classified as good. The bridge downstream of the Blackwater Bridge is classified as moderate. Chemical results indicate that river is in satisfactory condition.

Change from 2011

## No significant change.

River: Blackwater (Swanlinbar)

Catchment: Erne

Now known as Owensallagh 0800 Br. SE of Gorteen (on tributary) see Owensallagh

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. D/s Blackwater Br. | 0400 | - NW_36_1161 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | Br. D/s Blackwater Br. | 4-5 | 4 | 4-5 | 5 | 5 | 4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | NM | NM | 12 | 13 | 10 | 10 | 14 | 14 | 12 | 10 | 16 | 10 | 11 |

Assessment

Good.

Change from 2011

Now known as Owensallagh 0800 Br. SE of Gorteen (on tributary) see Owensallagh

River: Bunnoe

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. W of Killynenagh L | 0500 | NW_36_596 | Moderate | Good | 2021 |
| Br. U/s Annalee R confl | 0700 | NW_36_596 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Br. W of Killynenagh L | 3-4 | 3-4 | 3-4 | 3-4 | 3 | 3-4 | - | 3-4 |
| 0700 | Br. U/s Annalee R confl | 3-4 | 4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | NM | NM | 39 | 61 | 29 | 55 | 38 | 28 | 56 | 80 | 60 | 29 | 40 | 58 |
| 0700 | 39 | 50 | 47 | 63 | 37 | 54 | 42 | 40 | 55 | 63 | 90 | 33 | 50 | 45 |

Assessment

River is in moderate condition.

Change from 2011

Increase in median ortho-phosphate levels at Monaghan border.

River: Cavan

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br nr Breffni Park | 0040 | NW_36_1654 | Good | Good | 2021 |
| Br N of Killycannon | 0085 | NW_36_1654 | Good | Good | 2021 |
| Br. D/s lisdarn | 0200 |  |  |  |  |
| Br. d/s St 0200 | 0300 | NW_36_189 | Poor | Good | 2021 |
| Br. u/s Annalee. | 0400 | NW_36_189 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br nr Breffni Park | 3 | 3-4 | 3 | 4 | 3 | 3 | 3 | 3 |
| 0085 | Br N of Killycannon | - | 3-4 | 4-5 | 4 | 4 | 4 | 4 | 4 |
| 0200 | Br. D/s lisdarn | 2 | 3 | - | - | - | - |  |  |
| 0300 | Br. d/s St 0200 | 2-3 | 3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2 |
| 0400 | Br. u/s Annalee. | 2-3 | 3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0002 | - | - | - | - | - | 52* | - | 134** |  |  |  |  |  |  |
| 0003 | - | - | - | - | - | 80 | 43** | 64** |  |  |  |  |  |  |
| 0004 | - | - | - | - | - | 60 | 69** | 80** |  |  |  |  |  |  |
| 0005 | - | - | - | - | - | 77 | 33** | 64** |  |  |  |  |  |  |
| 0030 | - | - | - | - | - | 63* |  | 55** |  |  |  |  |  |  |
| 0040 | - | - | 41 | 36 | 43 | 43 | 36 | 31 | 47 | 74 | 31 | 47 | 49 | 60 |
| 0085 | NM | NM | 70 | 42 | 60 | 69 | 37 | 61 | 71 | 63 | 46 | 36 | 53 | 49 |
| 0300 | NM | NM | 107 | 83 | 59 | 75 | 66 | 104 | 89 | 86 | 52 | 60 | 66 | 131 |
| 0400 | 99 | 68 | 72 | 36 | 58 | 79 | 75 | 77 | 60 | 53 | 63 | 100 | 99 | 110 |

*8 samples taken

** 6 samples taken

Assessment

Overall this river is classified as poor status downstream of the town and good status upstream of the town.

Change from 2011

Some increase in median ortho-phosphate levels at station 0300.

River: Chapel Lake Stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br near Drumoosclin | 0400 | EA_07_931 | Good | Good | 2009 |
| Br U/S Blackwater Confl | 0700 | EA_07_931 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 | Q’12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | Br near Drumoosclin | - | - | 4 | 4 | - | - | - | - |
| 0700 | Br U/S Blackwater Confl | 3 | 4 | 4 | 4 | 4-5 | 4 | - | 4-5 |

MRP results

| Station | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | NM | 19 | 17 | 22 | 16 | 23 | 16 | 16 | - | - | - | - | - |
| 0700 | NM | 25 | 19 | 14 | 14 | 20 | 27 | 18 | 20* | 39 | 30 | 25 | 42 |

* based on 3 samples

Assessment

Good.

Change from 2011

Increase in median ortho-P levels.

River: Cullies

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Gulladoo L | 0300 | NW_36_2032 | Poor | Good | 2021 |
| Br. U/s Drumhart L | 0550 | NW_36_2032 | Poor | Good | 2021 |
| Br. Nr Kilbracken House. | 0600 | NW_36_2032 | Poor | Good | 2021 |
| Br. D/s Laheen L | 0650 | NW_36_188 | Poor | Good | 2021 |
| Br. D/s Disert L | 0750 | NW_36_188 | Poor | Good | 2021 |
| New Br. | 0900 | NW_36_188 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Br u/s Gulladoo L | 3 | 3 | 3 | 3-4 | - | 4 | 3-4 | 3-4 |
| 0550 | Br. U/s Drumhart L | 2-3 | 3-4 | 3-4 | 3 | - | 3-4 | 3-4 | 3-4 |
| 0600 | Br. Nr Kilbracken House. | 1-2 | 3-4 | 3-4 | 3 | 3 | 3 | 3 | 3 |
| 0650 | Br. D/s Laheen L | 3 | 3-4 | 3-4 | 3-4 | 3 | 4 | 3-4 | 3-4 |
| 0750 | Br. D/s Disert L | 2 | 3-4 | 3-4 | 3 | 4 | 4 | - | 3-4 |
| 0900 | New Br. | 2 | 2-3 | 2-3 | 3 | 2-3 | 3 | 3 | 3 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 11 | 14 | 17 | 13 | 10 | 10 | 10 | 10 | 22 | 13 | 10 | 14 | 12 | 17 |
| 0650 | NM | NM | 17 | 10 | 10 | 10 | 10 | 17 | 29 | 24 | 15 | 15 | 18 | 20 |
| 0750 | NM | NM | 18 | 10 | 10 | 17 | 13 | 11 | 24 | - | - | - | - | - |
| 0900 | 35 | 33 | 37 | 26 | 25 | 50 | 30 | 46 | 33 | 46 | 18 | 27 | 35 | 32 |

Assessment

River has a poor ecological status. In contrast the chemical data indicate that this river is relatively unpolluted.

Change from 2011

No significant change.

River: Dee

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Ervy L | 0016 | NB_06_733 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 | Q’12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0016 | Br u/s Ervy L | 4-5 | 4 | 4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0016 | 14 | 21 | 18 | 17 | 16 | 16 | 16 | 18 | 17 | 16 | 20 | 20 | 22 | 27 |

#### Assessment

This river has a poor ecological status but has low levels of median ortho-Phosphorus.

Change from 2011

No significant change.

River: Dromore

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| New Br. at Clementstown | 0600 | NW_36_237 | Poor | Good | 2021 |
| Old Bridge | 0700 | NW_36_237 | Poor | Good | 2021 |
| Killycreeney br | 0900 | NW_36_237 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Ballynascarva bridge | 3 | 3 | 3 | 3 | - | 3-4 | 3 | 3 |
| 0600 | New Br. at Clementstown | 3 | - | - | - | - | - | - | - |
| 0700 | Old Bridge | 3 | 3 | 3-4 | 3 | 3 | 3 | 3 | 3 |
| 0900 | Killycreeney br | 3-4 | 4 | 4 | 4-5 | 4 | 4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 36 | 38 | 35 | 26 | 33 | 30 | 38 | 31 | 49 | 32 | 22 | 30 | 24 | 42 |
| 0700 | 57 | 44 | 53 | 35 | 37 | 41 | 38 | 42 | 44 | 38 | 30 | 55 | 37 | 33 |
| 0900 | 58 | 48 | 53 | 40 | 39 | 45 | 40 | 40 | 65 | 68 | 44 | 57 | 43 | 36 |

Assessment

This river is in poor condition.

Change from 2011

Increase in Phosphorus levels at Monaghan border.

River: Derrada

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br West of Drumder | 0040 | - NW_36_1170 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’97 | Q’98 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br West of Drumderg | - | 4-5 | 4-5 | 4 | 4 | 4 |

### MRP results

| Station | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- |
| 0016 | 12 | 11 | 12 | 10 | 10 | 12 |

#### Assessment

This river has a poor ecological status but is currently satisfactory from a physiochemical and biological quality element. Ecological status is defined as poor because of macroinvertebrate monitoring in this waterbody, this would appear to be contrary to the actual Q result for this station.

Change from 2011

No significant change.

River: Drumane

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br 3km W of Ballyconnell | 0200 | NW_36_1285 | Good | Good | 2009 |
| Br. u/s Woodford R | 0500 | NW_36_1285 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br 3km W of Ballyconnell | 5 | 5 | 5 | - | - | 4 | 4-5 |
| 0500 | Br. u/s Woodford R | 4-5 | - | 4 | 4 | - | - | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | NM | NM | 12 | 11 | 10 | 11 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0500 | 10 | 10 | 12 | 11 | 10 | 10 | 10 | 12 | 10 | - | - | - | - | - |

Assessment

River is in good condition.

Change from 2012

No change.

River: Drumkeery Lough stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br d/s Skeagh L | 0100 | EA_07_581 | Poor | Good | 2015 |
| u/s Castle L | 0400 | EA_07_581 | Poor | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br d/s Skeagh L | 3 | 3 | 4 | - | - | - |
| 0400 | u/s Castle L | 3 | 3 | 2/3 | 2/3 | 2/3 | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | NM | NM | 18 | 11 | 14 | 15 | 15* | 14 |
| 0400 | 12 | 21 | 40 | 10 | 23 | 19 | 24 | 20 |

* Medians calculated over a two year period.

#### Assessment

Status is poor in this small stream.

Change from 2011

## Not designated by EPA for monitoring.

River: Drumsallagh

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br W of Drumsallagh | 0056 | NB_06_484 | Good | Good | 2009 |
| Br ESE of Corrinshigo House | 0066 | NB_06_484 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 | Q’12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0056 | Br W Drumsallagh | 5 | 3 | 4-5 | 4-5 | 4-5 | 4 | 4 | 4 |
| 0066 | Br ESE of Corrinshigo House | 4-5 | 4-5 | 4-5 | 4 | 4 | - | - | - |

MRP results

| Station | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- |
| 0056 | 17 | 13 | 13 | 16 | 19 | 27 |

#### Assessment

Status is good.

Change from 2011

Some increase in phosphorous levels.

River: Erne

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. 3km S/W of Crosskeys | 0100 | NW_36_350 | Moderate | Good | 2021 |
| Derrin Bridge | 0200 | NW_36_199 | Poor | Good | 2021 |
| Br d/s Derrin Bridge | 0300 |  |  |  |  |
| Br at Carrigan | 0400 | NW_36_199 | Poor | Good | 2021 |
| Legwee bridge | 0500 | NW_36_199 | Poor | Good | 2021 |
| Kilsarn bridge | 0700 | NW_36_199 | Poor | Good | 2021 |
| Dernaferst Bridge | 0770 |  |  |  |  |
| Dingins br. | 0900 | NW_36_1746 | Moderate | Good | 2021 |
| Br east of Corlismore | 1000 | NW_36_1746 | Moderate | Good | 2021 |
| Bellaheelin Br | 1100 | NW_36_1746 | Moderate | Good | 2021 |
| 1 km u/s Carrs lake | 1200 | NW_36_1746 | Moderate | Good | 2021 |
| Drumulig Br near school | 1270 |  |  |  |  |
| Bakers bridge | 1300 | XB_36_east_4 | Poor | Good | 2021 |
| Br at Kilconny, Belturbet (RHS) | 1400 | XB_36_east_4 | Poor | Good | 2021 |
| Kilconny, Belturbet (LHS) | 1410 |  |  |  |  |
| D/s Marina LHS | 1430 |  |  |  |  |
| 2.5 km d/s Belturbet | 1500 |  |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. 3km S/W of Crosskeys | 4 | 4 | 4 | 4 | 4 | 3-4 | 3-4 |
| 0200 | Derrin Bridge | 4-5 | 4 | 3-4 | 3-4 | 4 | 3-4 | 3-4 |
| 0300 | Br d/s Derrin Bridge | - | - | - | - | - | - | - |
| 0400 | Br at Carrigan | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | - | 3 |
| 0500 | Legwee bridge | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3 | 3-4 |
| 0700 | Kilsarn bridge | 4 | 4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 |
| 0770 | Dernaferst Bridge | - | - | - | - | - | - | - |
| 0900 | Dingins br. | 3 | 3 | 3 | 3 | 3-4 | 3-4 | 3-4 |
| 1000 | Br east of Corlismore | 4 | 4 | 4 | 4 | 4 | - | 4 |
| 1100 | Bellaheelin Br | 4 | 4 | 4 | 4 | 4 | 3-4 | 3-4 |
| 1200 | 1 km u/s Carrs lake | - | - | - | - | - | - | - |
| 1270 | Drumulig Br near school | - | - | - | - | - | - | - |
| 1300 | Bakers bridge | - | - | - | 3-4 | 3-4 | 3-4 | 3-4 |
| 1400 | Br at Kilconny, Belturbet (RHS) | 3-4 | 3 | 3 | 3 | 3-4 | 3-4 | 3-4 |
| 1410 | Kilconny, Belturbet (LHS) | - | - | 3 | 1 | 1 | - | 3-4 |
| 1430 | D/s Marina LHS | - | - | - | - | - | - | - |
| 1500 | 2.5 km d/s Belturbet | - | - | - | - | - | - | - |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 46 | 29 | 26 | 54 | 33 | 41 | 42 | 72 | 41 | 33 | 41 | 72 |
| 0200 | - | - | 29 | 39 | 28 | 62 | 43 | 35 | 56 | 86 | 53 | 42 | 58 | 54 |
| 0300 | 62 | 40 | - | - |  | - |  | - | - | - | - | - | - |  |
| 0400 | - | - | 70 | 70 | 50 | 91 | 60 | 55 | 140 | - | - | - | - |  |
| 0500 | - | - | 52 | 57 | 46 | 81 | 50 | 50 | 78 | 99 | 71 | 50 | 58 | 49 |
| 0700 | 63 | 53 | 60 | 61 | 47 | 70 | 61 | 45 | 73 | 75 | 79 | 40 | 63 | 63 |
| 0770 | 14 | 10 | - | - | - | - |  | - | - | - | - | - | - |  |
| 0900 | - | - | 35 | 22 | 16 | 19 | 15 | 13 | 52 | 52 | 21 | 50 | 53 | 56 |
| 1000 | - | - | 42 | 29 | 30 | 40 | 37 | 37 | 40 | - |  | - | - |  |
| 1100 | 41 | 41 | 37 | 32 | 33 | 47 | 37 | 41 | 61 | 65 | 34 | 48 | 44 | 57 |
| 1200 | 41 | 38 | 41 | 31 | 32 | 48 | 40* | 36 | 52 | 56 | 44 | 49 | 30 | 48 |
| 1300 | 40 | 23 | 27 | 26 | 23 | 29 | 39 | 51 | 63 | 42 | 38 | 36 | 50 | 36 |
| 1400 | 37 | 21 | 36 | 27 | 30 | 37 | 42 | 44 | 62 | 39** | 41 | 31 | 46 | 36 |

Assessment

## River is classified as poor to moderate throughout.

Change from 2011

Some increase in Phosphorus levels in the upper reaches of this river.

River: Glyde

Catchment: Glyde

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Cormey Br. | 0100 | NB_06_602 | Good | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 | Q’12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Cormey Br. | 4-5 | 4-5 | 4-5 | 4-5 | 4 | 4 | 4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 46 | 41 | 46 | 30 | 36 | 34 | 47 | 43 | 52 | 58 | 54 | 50 | 38 | 39 |

Assessment

This river is classified as moderate.

Change from 2011

Disappointing reduction in Biological rating at Cormey Bridge. No significant change.

River: Inny

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Ballinrink br | 0300 | SH_26_2664 | Poor | Good | 2021 |
| Ross bridge | 0400 | SH_26_2664 | Poor | Good | 2021 |
| Finea Bridge | 0500 | SH_26_2660 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’87 | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | Ballinrink br | 4 | 3-4 | 4 | 3 | 3 | 3-4 | 3-4 | 4 |
| 0400 | Ross bridge | 4 | - | - | - | - | - | - | - |
| 0500 | Finea Bridge | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | 23 | 19 | 34 | 19 | 16 | 22 | 32 | 24 | 21 | 19 | 10 | 17 | 19 | 13 |
| 0500 | 10 | 10 | 10 | 10 | 10 | 16 | 10 | 10 | 10 | 10 | 10 | 10 | 11 | 10 |

Assessment

Poor to Moderate. Low levels of Phosphorus at both stations.

Change from 2012

## No significant change.

River: Knappagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. U/s Annalee confl | 0700 | - NW_36_1068 | Poor | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0700 | Br. U/s Annalee confl | 3-4 | 3-4 | 3 | 3-4 | 4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | 21 | 20 | - | - | - | - | - | - | - | - | - | - | - | - |
| 0600 | 30 | 27 | - | - | - | - | - | - | - | - | - | - | - | - |
| 0700 | - | - | 22 | 22 | 21 | 24 | 22 | 20 | 29 | 20 | 17 | 21 | 22 | 27 |

Assessment

Poor status however Phosphorus levels are low in this river.

Change from 2011

## No significant change.

River: Laragh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 1st bridge d/s L. Acanon | 0015 | - NW_36_1557 | Good | Good | 2009 |
| Br W/SW Knockataggert | 0030 | NW_36_228 | Moderate | Good | 2021 |
| Br 1.5 km E Laragh | 0050 | NW_36_228 | Moderate | Good | 2021 |
| Br. In Laragh | 0080 | NW_36_228 | Moderate | Good | 2021 |
| Shanns br Lisclone | 0200 | NW_36_228 | Moderate | Good | 2021 |
| U/s Annalee confl | 0400 | NW_36_228 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0015 | 1st bridge d/s L. Acanon | 3-4 | 3 | 3 | - | - | - | - |
| 0030 | Br W/SW Knockataggert | - | - | - | - | 4-5 | 4 | 4 |
| 0050 | Br 1.5 km E Laragh | 4-5 | 4-5 | 4 | 4 | 4 | - |  |
| 0080 | Br. In Laragh | 4-5 | 4-5 | 4-5 | 4 | 4 | 3-4 | 4 |
| 0200 | Shanns br Lisclone | 3-4 | 4-5 | 4-5 | 3-4 | 3-4 | - |  |
| 0400 | U/s Annalee confl | 4-5 | 4-5 | 4-5 | 3-4 | 3-4 | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0015 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 16 |  |  |  |  |  |
| 0030 | - | - |  |  |  |  |  |  | 35 | 25 | 12 | 13 | 19 | 32 |
| 0050 | - | - | 21 | 18 | 10 | 19 | 17 | 18 | 22 |  |  |  |  |  |
| 0080 | - | - | 31 | 30 | 18 | 30 | 26 | 30 | 31 | 31 | 22 | 17 | 28 | 33 |
| 0200 | - | - | 28 | 30 | 18 | 29 | 27 | 24 | 29 |  |  |  |  |  |
| 0400 | 35 | 29 | 28 | 31 | 22 | 32 | 30 | 31 | 35 | 22 | 17 | 15 | 33 | 31 |

Assessment

Most of the river stations are within a water body classified as moderate

Change from 2011

Some increase in median orthophosphate levels in upper stretches.

River: Lislea

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 1st bridge u/s L. Ramor | 0100 | EA_07_1368 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 | Q’12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 1st bridge u/s L. Ramor | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | 25 | 40 | 32 | 34 | 34 | 24 | 30 | 33 | 41 | 32 | 37 | 38 | 37 | 44 |

Assessment

Good status.

Change from 2011

No significant change.

River: Maudabawn

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br E of Canningstown | 0070 | NW_36_2024 | Moderate | Good | 2021 |
| Br. 1 km S of Drumcondra | 0200 | NW_36_2024 | Moderate | Good | 2021 |
| bridge u/s Annalee river | 0600 | NW_36_2024 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0070 | Br E of Canningstown | 4 | 3-4 | 3-4 | 3 | 3-4 | 3-4 | 3-4 |
| 0200 | Br. 1 km S of Drumcondra | 4 | 4 | 3 | 4 | 4 | 4 | 4 |
| 0600 | bridge u/s Annalee river | 4 | 4 | 4 | 3-4 | 4 | 3-4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0070 | - | - | 25 | 27 | 24 | 35 | 23 | 26 | 42 | 53 | 38 | 28 | 32 | 74 |
| 0200 | - | - | 36 | 38 | 33 | 44 | 28 | 25 | 45 | 64 | 44 | 41 | 37 | 49 |
| 0600 | 33 | 26 | 20 | 30 | 26 | 40 | 23 | 19 | 37 | 60 | 41 | 28 | 35 | 44 |

Assessment

## Moderate.

Change from 2011

Significant increase in Phosphorous levels in Canningstown.

River: Mountnugent

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Near Rockville | 0040 | SH_26_2264 | Poor | Good | 2021 |
| bridge se Rockville | 0120 | SH_26_2264 | Poor | Good | 2021 |
| Br.near Derrylea | 0200 | SH_26_2742 | Moderate | Good | 2021 |
| Br. At Kildorra | 0300 | SH_26_2742 | Moderate | Good | 2021 |
| Br E of Killnacrot House | 0350 | SH_26_2742 | Moderate | Good | 2021 |
| Mountnugent br. | 0500 | SH_26_2742 | Moderate | Good | 2021 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’87 | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | Br. Near Rockville | - | 4 | 4-5 | 3-4 | 4 | 4 | - |  |
| 0120 | bridge se Rockville | 4 | - | 3 | 1/0 | - | - | 2-3 | 3 |
| 0200 | Br.near Derrylea | 1 | 4 | 3-4 | 3 | 4-5 | 4 | 3-4 | 3 |
| 0300 | Br. At Kildorra | - | - | - | - | - | - | 3-4 | 3-4 |
| 0350 | Br E of Killnacrot House | 3 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | - |  |
| 0500 | Mountnugent br. | 3 | 3-4 | 3-4 | 3-4 | 4 | 3-4 | 3-4 | 3-4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0040 | - | - | 39 | 24 | 16 | 23 | 14 | 11 | 21 | - | - | - | - | - |
| 0120 | - | - | 32 | 28 | 35 | 32 | 25 | 22 | 38 | 78 | 98 | 41 | 67 | 70 |
| 0200 | 40 | 33 | 33 | 24 | 29 | 24 | 18* | 11 | 43 | 45 | 43 | 25 | 42 | 66 |
| 0300 | 28 | 24 | 29 | 33 | 43 | 48 | 32 | 21 | 57 | 103 | 192 | 146 | 176 | 119 |
| 0350 | - | - | 26 | 19 | 19 | 22 | 16 | 13 | 36 | 39 | 54 | 48 | 44 | 43 |
| 0400 | 23 | 21 | - | - | - | - | - | - | - | - | - | - | - | - |
| 0500 | 22 | 24 | 35 | 20 | 18 | 21 | 17 | 12 | 33 | 40 | 46 | 42 | 46 | 38 |

Assessment

Unsatisfactory, all stations are classified as poor status.

Change from 2012

Increase in Phosphorus levels at bridge near Derrylea. Some reduction in Phosphorus levels at stations 0300.

River: Moynalty

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Near Lurganbane | 0050 | EA_07_1128 | Good | Good | 2009 |
| Cloggagh Br | 0070 | EA_07_940 | Moderate | Good | 2027 |
| Skearke Br. | 0090 | EA_07_940 | Moderate | Good | 2027 |
| br. D/s Rosehill Br | 0300 | EA_07_940 | Moderate | Good | 2027 |
| bridge u/s main channel confl | 0400 | EA_07_940 | Moderate | Good | 2027 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 | Q’12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | Br. Near Lurganbane | 4 | 4 | 3 | 3-4 | 3-4 | - | - | - |
| 0070 | Cloggagh Br | 3 | 3 | 3-4 | 3 | 4 | 3-4 | 3-4 | 4 |
| 0090 | Skearke Br. | - | 3 | 4 | 3-4 | 4 | - | - | - |
| 0300 | br. D/s Rosehill Br | 3-4 | 3 | 3 | 3 | 3-4 | 3-4 | 3-4 | 3 |
| 0400* | bridge u/s main channel confl | 3 | 3 | 3 | 3 | 3 | - | 3-4 | 3-4 |

*Now Mullagh Lough Stream

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | - | - | 77 | 74 | 50 | 74 | 60 | 56 | 69 | - | - |  |  |  |
| 0070 | - | - | 43 | 45 | 32 | 30 | 38 | 36 | 55 | 61 | 69 | 40 | 47 | 48 |
| 0090 | - | - | 33 | 39 | 26 | 30 | 30 | 28 | 32 | - | - |  |  |  |
| 0200 | 27 | 51 | - | - | - | 25 | - | - | 30 | - | - |  |  |  |
| 0300 | 37 | 49 | 31 | 29 | 27 | 31 | 25 | 33 | 52 | 71 | 34 | 32 | 31 | 80 |
| 0400* | - | - | 134 | 70 | 94 | 72 | 68 | 36 | 114 | 96 | 118 | 146 | 97 | 98 |

Assessment

Poor, with the exception of station 0050 all stations are within a water body classified as having a poor ecological status.

Change from 2011

Increase in Phosphorus levels at station 0300.

River: Nadregeel Lough stream

Catchment: Boyne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Assan Br. | 0100 | EA_07_1221 | Moderate | Good | 2009 |
| Billis Br | 0200 | EA_07_1221 | Moderate | Good | 2009 |
| Footbridge d/s billis br | 0300 |  |  | Good |  |
| br. SW of Lisgrey X-rds | 0400 |  |  | Good |  |
| Bridge nr Cranadillon | 0500 | EA_07_1357 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’90 | Q’94 | Q’97 | Q’00 | Q’03 | Q’06 | Q’09 | Q’12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Assan Br. | 3-4 | 3-4 | 4-5 | 3-4 | 3-4 | 4 | 3-4 | 4 |
| 0200 | Billis Br | 3 | 3 | 3 | 4 | 3-4 | 4 | - | - |
| 0300 | Footbridge d/s billis br | - | - | - | - | - | - | - | - |
| 0400 | br. SW of Lisgrey X-rds | 3 | - | - | - | - | - | - | - |
| 0500 | Bridge nr Cranadillon | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 4 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 23 | 29 | 23 | 26 | 18 | 19 | 26 | 24 | 16 | 12 | 27 | 28 |
| 0200 | 27 | 22 | 20 | 23 | 21 | 22 | 20 | 20 | 23 | 33 | 21 | 13 | 20 | 19 |
| 0500 | 14 | 18 | 16 | 15 | 11 | 16 | 17 | 13 | 25 | 22 | 12 | 16 | 19 | 24 |

Assessment

River water bodies are classified as moderate.

Change from 2011

No change.

River: Owenayle

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br NE of Aughrim | 0050 | SH_26_1095 | Moderate | Good | 2015 |
| Br D/S of Owenayle bridge | 0100 | SH_26_1095 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | Br NE of Aughrim | 4 | 4 | 4 | 4 | 3-4 | 3-4 | 3-4 |
| 0100 | Br D/S of Owenayle bridge | 4 | 4-5 | 4-5 | 4 | 4 | 4 | 4 |

MRP results

| Station | 1999 Med (µgP/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0050 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 11 | 12 | 10 | 10 | 14 |
| 0100 | 10 | 10 | 10 | 10 | 10 | 10 | 10* | 10 | 10 | 10 | 10 | 10 | 15 | 10 |

* Calculated over a two year period

Assessment

Moderate. However chemical results indicate good quality water.

Change from 2011

## No change.

River: Owenmore (Glangevlin)

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Carty’s Bridge | 0100 | SH_26_3419 | Moderate | Good | 2015 |
| Br. SE Carty’s Bridge | 0120 | SH_26_3419 | Moderate | Good | 2015 |
| U/s bellavally branch Confluence | 0140 | SH_26_3419 | Moderate | Good | 2015 |
| Br. NE Moneensauran | 0160 | SH_26_3273 | Good | Good | 2009 |
| B W of Legnacrow L | 0300 | SH_26_3539 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Carty’s Bridge* | 4-5 | 4-5 | 5 | 4-5 | 4-5 | 3-4 | 4 |
| 0120 | Br. SE Carty’s Bridge** | 4-5 | 5 | 4-5 | 5 | - | - | - |
| 0140 | U/s bellavally branch Confluence*** | 4 | 4-5 | 4-5 | 4-5 | - | - | - |
| 0160 | Br. NE Moneensauran | - | 4 | - | - | - | - | - |
| 0300 | B W of Legnacrow L | 4-5 | 5 | 4-5 | 4-5 | 3-4 | 3-4 | 4 |

* Bursan Stream

** Bellavally Stream

***Derrynananta Stream

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |  |
| 0120 | - | - | 10 | 10 | 10 | 11 | 10 | 10 | 10 | 10 | 10 | 10 | 12 |  |
| 0140 | - | - | 10 | 10 | 10 | 10 | 13 | 23 | 10 | 10 | 10 | 10 | 10 |  |
| 0160 | - | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | - | - | - | - |  |
| 0300 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |

Assessment

River water bodies are generally moderate. Chemical results indicate low Phosphorus levels.

Change from 2011

## No significant change.

River: Owensallagh

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br E of Drumcor | 0500 | - NW_36_1161 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | Br E of Drumcor | 5 | 5 | 3-4 | 4-5 | 4-5 | 4 | 4 |
| 0800 | Br SE of Gorteen (on Tributary) | 4-5 | 4 | 4-5 | 5 | 5 | 4 | 4 |

MRP results

| Station | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0500 | 10 | 10 | 10 | 13 | 13 | 15 | 10 | 11 | 10 | 10 | 10 | 10 |
| 0800 |  |  |  |  |  |  |  |  |  |  |  | 12 |

Assessment

River is in good condition.

Change from 2011

## No significant change.

River: Rag

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br D/S Clonty | 0200 | NW_36_550 | Good | Good | 2009 |
| Br D/S Togher | 0400 | NW_36_550 | Good | Good | 2009 |
| Br D/S Aghalane | 0500 | NW_36_550 | Good | Good | 2009 |
| Br. Nr Killywilly house | 0600 | - NW_36_1025 | Good | Good | 2009 |
| Br u/s Tee lough lr | 0800 | XB_36_east_4 | Poor | Good | 2021 |
| Bridge d/s Tee lough lr | 0900 |  |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’89 | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br D/S Clonty | - | - | - | - | - | - | - | - |
| 0400 | Br D/S Togher | - | - | - | - | - | - | - | - |
| 0500 | Br D/S Aghalane | - | - | - | - | - | - | - | - |
| 0600 | Br. Nr Killywilly house | 3 | 3 | 3 | - | 3-4 | 3-4 | - | 3 |
| 0800 | Br u/s Tee lough lr | 2-3 | 3 | 3 | 3-4 | 4 | 4 | 4 | 4 |
| 0900 | Bridge d/s Tee lough lr | 3 | 3 | 3 | - | - | - | - | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | - | 24 | 34 | 27 | 38 | 31 | 28 | 27 | 41 | - | - | - |  |  |
| 0400 | - | 30 | 42 | 37 | 41 | 25 | 36* | 51 | 44 | - | - | - |  |  |
| 0500 | - | 27 | 34 | 16 | 38 | 23 | 25 | 28 | 42 | - | - | - |  |  |
| 0600 | 40 | 29 | 44 | 38 | 24 | 29 | 24 | 31 | 42 | - | - | - |  |  |
| 0800 | - | - | 40 | 36 | 22 | 20 | 38 | 43 | 43 | 29 | 34 | 35 | 28 | 29 |
| 0900 | - | - | 26 | 32 | 26 | 29 | 30 | 37 | 57 | - | - | - |  |  |

#### Assessment

Generally good, however station 0800 is in a poor quality water body.

Change from 2011

No significant change.

River: Roo

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. W Barrran | 0200 | NW_36_2114 | High | Good | 2009 |
| Br SE of Thornhill house | 0400 | NW_36_2114 |  |  |  |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | Br. W Barrran | 4-5 | 4 | 4 | 5 | 5 | 5 |
| 0400 | Br SE of Thornhill house | - | - | 3-4 | - | - |  |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0200 | 10 | 12 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 14 | 10 |

Assessment

River is in good condition.

Change from 2011

## No significant change.

River: Shannon

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Shannon Br, Derrylahan | 0100 | SH_26_3539 | Moderate | Good | 2015 |
| Metal Br. S of Tullynafreave | 0200 | SH_26_3539 | Moderate | Good | 2015 |
| Dowra Br | 0300 | SH_26_3539 | Moderate | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’92 | Q’96 | Q’99 | Q’02 | Q’05 | Q’08 | Q’11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Shannon Br, Derrylahan | 4 | 4-5 | 4 | 4 | 4 | 3-4 | 4-5 |
| 0200 | Metal Br. S of Tullynafreave | 5 | 5 | 4-5 | 4-5 | - | 4-5 | 4-5 |
| 0300 | Dowra Br | 5 | 4-5 | 4-5 | 5 | 4 | 4-5 | 4-5 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | - | - | 10 | 13 | 10 | 10 | 11 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0200 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0300 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |

Assessment

Moderate status, however chemical results indicate low Phosphorus levels.

Change from 2011

## No significant change.

River: Sheelin Feeder Streams

Catchment: Shannon

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status Station | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- | --- |
| Belsgrove (Ballynakerry) Belsgrove (Ballyheelan) stream | 0300 |  |  |  |  |  |
| Sheelin Feed(Crover) Crover Stream | 0600 |  |  |  |  |  |
| Schoolhouse Bridge Belsgrove (Ballyheelan) stream | 0100 |  |  |  |  |  |
| Belsgrove(Ballynakerry) | 0400 |  |  |  |  |  |

### No Biological Quality Ratings available

Sampling stations and Biological Quality Ratings

| Station Number | Station Location | Q’96 |
| --- | --- | --- |
| 01 | Belsgrove bridge | - |
| 02 | Crover Bridge | - |
| 04 | Schoolhouse Bridge | - |
| 08 | Belsgrove(Ballynakerry) | - |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0300 | 24 | 36 | 22 | 19 | 17 | 26 | 18 | 20 | 34 | 39 | 18 | 19 | 10 | 16 |
| 0600 | 29 | 31 | 57 | 34 | 47 | 61 | 29 | 27 | 33 | 33 | 23 | 26 | 36 | 31 |
| 0100 | 34 | 32 | 37 | 27 | 17 | 21 | 25 | 15 | 29 | 30 | 13 | 15 | 22 | 17 |
| 0400 | - | - | - | - | 21 | 31 | 18 | 31 | 26 | 10 | 24 | 33 | 45 | 42 |

Assessment

Status has not been assigned under the WFD.

Change from 2012

## No significant change.

Note: These Rivers are monitored as part of the councils monitoring programme for Lough Sheelin.

River: Stradone

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| 2nd Br. D/s Lisnanagh L | 0075 | NW_36_245 | Good | Good | 2009 |
| Br u/s Laragh R confl | 0200 | NW_36_245 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0075 | 2nd Br. D/s Lisnanagh L | 3-4 | 4 | 3-4 | 4 | 4 | - | 4 |
| 0200 | Br u/s Laragh R confl | 4-5 | 4 | 3-4 | 4 | 4 | 4 | 4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0075 | - | - | 40 | 35 | 26 | 29 | 27 | 23 | 46 | 37 | 10 | 25 | 32 | 32 |
| 0200 | 38 | 38 | 33 | 32 | 27 | 25 | 25 | 23 | 40 | 32 | 17 | 25 | 30 | 33 |

Assessment

Good.

Change from 2011

No significant change.

River: Swanlinbar

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br. Nr Altbrean | 0100 | NW_36_18 | Good | Good | 2015 |
| 0.6km d/s Swanlinbar br | 0300 | NW_36_18 | Good | Good | 2015 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’07 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 | Br. Nr Altbrean | 5 | 5 | 5 | - | 4-5 | 4-5 | 4-5 |
| 0300 | 0.6km d/s Swanlinbar br | 4-5 | 4-5 | 4-5 | - | 3-4 | 4 | 4-5 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0100 |  | - | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| 0300 | 14 | 13 | 14 | 13 | 11 | 12 | 14 | 10 | 11 | 10 | 10 | 10 | 10 | 13 |

Assessment

Good. Chemical results indicate low Phosphorus levels.

Change from 2011

No change.

River: Templeport Lough Stream

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Br u/s Templeport Lough | 0600 | NW_36_1177 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’93 | Q’97 | Q’98 | Q’01 | Q’04 | Q’10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | Br u/s Templeport Lough | 3 | 3 | 4 | 3 | 3 | 4 |

### MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l | 2007 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0600 | 16 | 15 | 14 | 20 | 12 | 20 | 10 | 16 | 13 |

Assessment

## Good.

Change from 2011

Not monitored chemically in 2012.

River: Woodford

Catchment: Erne

WFD Status

| Station Location | Station Number | Water Body Code | Current Ecological Status RWB | Target Ecological Status RWB | Target Date |
| --- | --- | --- | --- | --- | --- |
| Ballyconnell Quay | 0400 | XB_36_east_5 | Good | Good | 2009 |

Sampling Stations and Biological Quality Ratings

| Station Number | Station Location | Q’85 | Q’89 |
| --- | --- | --- | --- |
| 0400 | Ballyconnell Quay | 4-5 | 4 |

MRP results

| Station | 1999 Med (µg P/l) | 2000 Med (µg P/l) | 2001 Med (µg P/l) | 2002 Med (µg P/l) | 2003 Med (µg P/l) | 2004 Med (µg P/l) | 2005 Med (µg P/l) | 2006 Med (µg P/l) | 2007 Med (µg P/l) | 2008 Med (µg P/l) | 2009 Med (µg P/l) | 2010 Med (µg P/l) | 2011 Med (µg P/l) | 2012 Med (µg P/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0400 | 17 | 12 | 15 | 17 | 11 | 20 | 14 | 12 | 13 | 10 | 10 | 10 | 15 | 20 |

* Baseline Q rating on this station based on ‘89 Q rating

Assessment

## Good.

Change from 2011

Slight increase in Phosphorus levels.

4	Lake Water Quality

Lakes are listed in alphabetical order.

#### Summary Data

Lake name and catchment:

River name and catchment: Included are the lake name and the catchment name (namely: the Erne, the Boyne, the Shannon and the Glyde). Sub-catchments and water management units are not listed.

The Lake water body code is given for each lake. The Current Ecological status as defined is given with target ecological status. The target ecological status cannot be lower than Good. A target date for reaching the target ecological status is included for each station.

#### Annual Status:

This table indicates the annual status of the lake in question and includes

- Average annual Total P (g/l),

- Maximum Chlorophyll,

#### Assessment:

A summary assessment of the water quality is given which is based on the scientific data.

Change from 2011:

Where applicable any significant changes in quality from 2011 are also given.

List of Lakes Monitored

Lake 							Catchment

Acurry		Erne

Ardan		Erne

Atrain		Erne

Bawn		Erne

Bunerky		Erne

Culliaghan		Erne

Derrycassan		Erne

Drumkeery		Boyne

Farnham		Erne

Garty		Erne

Glasshouse		Erne

Gowna		Erne

Kinale		Shannon

Mill Lough		Erne

Nadrageel		Boyne

Oughter		Erne

Ramor		Boyne

Lough Sheelin 		Shannon

Sillan		Erne

Lough Tacker 		Erne

Lake: Accury

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_242 | Good | Good | 2009 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 10 | 12 |
| 2008 | 21 | 10 |
| 2009 | 44 | 30 |
| 2010 | 31 | 22 |
| 2011 | 35 | 233 |
| 2012 | 18 | 22 |

Assessment

## Good status, under the RBD classification. Generally good quality trout lake.

Change from 2011

Significant improvement in phosphorous and chlorophyll levels.

Lake: Annagh

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_517 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 9 | 24 |
| 2008 | 29 | 18 |
| 2009 | 44 | 24 |
| 2010 | 40 | 14 |
| 2011 | 32 | 12 |
| 2012 | 14 | 28 |

Assessment

Moderate status, under the RBD classification. Generally good water abstraction lake.

Change from 2011

There is a reduction in Phosphorus levels and an increase in maximum chlorophyll levels.

Lake: Ardan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_432 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 10 | 12 |
| 2008 | 21 | 10 |
| 2009 | 44 | 30 |
| 2010 | 71 | 18 |
| 2011 | 39 | 10 |
| 2012 | 22 | 32 |

Assessment

Moderate status, under the new RBD classification.

Change from 2011

Welcome decrease in Phosphorus levels, but an increase in the Maximum Chlorophyll level recorded.

Lake: Atrain

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_618 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2010 | 55 | 24 |
| 2011 | 63 | 28 |
| 2012 | 58 | 18 |

Assessment

Moderate status, under the RBD classification.

Change from 2011

Decrease in Phosphorus levels.

Lake: Bawn

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_573 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 13 | 10 |
| 2008 | 41 | 4 |
| 2009 | 79 | 6 |
| 2010 | 63 | 16 |
| 2011 | 53 | 24 |
| 2012 | 26 | 6 |

Assessment

Moderate status, under the RBD classification.

Change from 2011

Decrease in Maximum chlorophyll levels and phosphorus.

Lake: Bunerky

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_624 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 29 | 22 |
| 2008 | 26 | 10 |
| 2009 | 37 | 12 |
| 2010 | 34 | 10 |
| 2011 | 53 | 12 |
| 2012 | 10 | 20 |

Assessment

Moderate status, under the RBD classification.

Change from 2010

Increase in Maximum Chlorophyll with a reduction in Phosphorus.

Lake: Culliaghan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_385 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 22 | 8 |
| 2008 | 51 | 6 |
| 2009 | 46 | 6 |
| 2010 | 87 | 44 |
| 2011 | 49 | 8 |
| 2012 | 25 | 12 |

Assessment

Moderate status, under the RBD classification

Change from 2011

Significant reduction in total Phosphorus levels.

Lake: Derrycasson

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_514 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 14 | 12 |
| 2008 | 34 | 12 |
| 2009 | 45 | 10 |
| 2010 | 34 | 24 |
| 2011 | 40 | 12 |
| 2012 | 23 | 14 |

Assessment

Moderate status, under the RBD classification

Change from 2011

Some reduction in Phosphorus levels.

Lake: Drumkeery

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_268 | Poor | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 30 | 34 |
| 2008 | 65 | 40 |
| 2009 | 48 | 16 |
| 2010 | 44 | 30 |
| 2011 | 100 | 207 |
| 2012 | 41 | 121 |

Assessment

## Poor.

Change from 2011

Significant decrease in phosphorus and chlorophyll levels. High chlorophyll result occurred in the autumn period.

Lake: Farnham

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_618 | High | High | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2010 | 265 | 260 |
| 2011 | 371 | 191 |
| 2012 | 103 | 52 |

Assessment

High status, under the RBD classification, however this status is based on extrapolated results and does not appear to accurately reflect the water quality of the lake. Current data indicates that this lake is polluted.

Change from 2011

Decrease in Phosphorus and maximum chlorophyll levels.

Lake: Garty

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_430 | Poor | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 21 | 10 |
| 2008 | 39 | 8 |
| 2009 | 44 | 16 |
| 2010 | 50 | 181 |
| 2011 | 63 | 40 |
| 2012 | 102 | 157 |

Assessment

## Poor.

Change from 2011

Disappointing increase in Phosphorus levels and maximum Chlorophyll levels.

Lake: Glasshouse

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_615 | Bad | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 15 | 10 |
| 2008 | 38 | 22 |
| 2009 | 70 | 20 |
| 2010 | 42 | 26 |
| 2011 | 52 | 36 |
| 2012 | 31 | 18 |

Assessment

## Current Bad status is as a result of the macrophyte classification.

Change from 2011

Reduction in average Phosphorus and Total Chlorophyll.

Lake: Gowna

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_524 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 61 | 121 |
| 2001 | 47 | 107 |
| 2002 | 72 | 56 |
| 2003 | 76 | 52 |
| 2004 | 72 | 125 |
| 2005 | 68 | 125 |
| 2006 | 60 | 56 |
| 2007 | 31 | 48 |
| 2008 | 42 | 30 |
| 2009 | 53 | 99 |
| 2010 | 61 | 137 |
| 2011 | 54 | 93 |
| 2012 | 34 | 82 |

Assessment

## Moderate.

Change from 2011

Reduction in maximum chlorophyll levels and phosphorus.

Lake: Kinale

Catchment: Shannon

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| SH_26_430 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 28 | 77 |
| 2001 | 24 | 56 |
| 2002 | 27 | 26 |
| 2003 | 28 | 18 |
| 2004 | 25 | 6 |
| 2005 | 26 | 24 |
| 2006 | 17 | 10 |
| 2007 | 10 | 10 |
| 2008 | 17 | 4 |
| 2009 | 20 | 15 |
| 2010 | 33 | 8 |
| 2011 | 32 | 6 |
| 2012 | 10 | 12 |

Assessment

Moderate status, under the new RBD classification, however physicochemical data is good. The water quality of this lake is inextricably linked to the quality of water in Lough Sheelin.

Change from 2012

Significant improvement in Phosphorous levels.

Lake: Mill Lough

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_597 | Moderate | Good | 2015 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 7 | 18 |
| 2008 | 22 | 28 |
| 2009 | 35 | 16 |
| 2010 | 62 | 57 |
| 2011 | 44 | 12 |
| 2012 | 14 | 10 |

Assessment

Moderate status.

Change from 2011

Reduction in Phosphorus levels.

Lake: Nadrageel

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_273 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 47 | 103 |
| 2001 | 47 | 85 |
| 2002 | 54 | 118 |
| 2003 | 59 | 111 |
| 2004 | 64 | 71 |
| 2005 | 49 | 50 |
| 2006 | 44 | 92 |
| 2007 | 31 | 38 |
| 2008 | 44 | 40 |
| 2009 | 55 | 28 |
| 2010 | 61 | 34 |
| 2011 | 41 | 22 |
| 2012 | 26 | 40 |

Assessment

Moderate status.

Change from 2011

There is a reduction in Phosphorus and an increase in maximum Chlorophyll levels.

Lake: Oughter

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_657 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 73 | 183 |
| 2001 | 64 | 129 |
| 2002 | 70 | 494 |
| 2003 | 94 | 187 |
| 2004 | 101 | 232 |
| 2005** | 90 | 232 |
| 2006 | 82 | 86 |
| 2007 | 54 | 92 |
| 2008 | 51 | 244# |
| 2009 | 76 | 62 |
| 2010 | 78 | 198 |
| 2011 | 76 | 171 |
| 2012 | 54 | 163 |

Assessment

Moderate status.

Change from 2011

Maximum Chlorophyll result was the only very elevated level in 2012 results, there is a reduction in average phosphorous.

Lake: Ramor

Catchment: Boyne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| EA_07_275 | Bad | Good | 2027 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 67 | 90 |
| 2001 | 59 | 92 |
| 2002 | 73 | 80 |
| 2003 | 100 | 165 |
| 2004 | 68 | 75 |
| 2005 | 74 | 80 |
| 2006 | 92 | 102 |
| 2007 | 58 | 83 |
| 2008 | 52 | 110 |
| 2009 | 70 | 69 |
| 2010 | 76 | 36 |
| 2011 | 69 | 54 |
| 2012 | 39 | 56 |

Assessment

## Bad status.

Change from 2011

Reduction in phosphorous levels.

Lake: Lough Sheelin

Catchment: Shannon

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| SH_26_709 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 32 | 36 |
| 2001 | 23 | 34 |
| 2002 | 28 | 32 |
| 2003 | 31 | 46 |
| 2004 | 32 | 62 |
| 2005* | 25 | 14 |
| 2006 | 25 | 8 |
| 2007 | 18 | 24 |
| 2008 | 28 | 14 |
| 2009 | 26 | 12 |
| 2010 | 30 | 4 |
| 2011 | 29 | 12 |
| 2012 | 16 | 36 |

*includes one sample from January 2006

Assessment

Moderate status.

Change from 2011

Phosphorus levels are encouraging.

Lake: Sillan

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_528 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2000 | 77 | 73 |
| 2001 | 57 | 50 |
| 2002 | 56 | 73 |
| 2003 | 66 | 52 |
| 2004 | 97 | 62 |
| 2005** | 103 | 74 |
| 2006 | 117 | 68 |
| 2007 | 46 | 66 |
| 2008 | 79 | 121 |
| 2009 | 73 | 54 |
| 2010 | 102 | 52 |
| 2011 | 100 | 46 |
| 2012 | 82 | 96 |

** Average and Maximum values calculated over a two year period

Assessment

Disappointing increase in Maximum Chlorophyll.

Change from 2011

No significant change.

Lake: Lough Tacker

Catchment: Erne

WFD Status

| Lake water body code | Current status | Target Ecological Status | Target Date |
| --- | --- | --- | --- |
| NW_36_363 | Moderate | Good | 2021 |

Annual status

| Year | Average Annual Total P(ug/l) | Maximum Chlorophyll |
| --- | --- | --- |
| 2007 | 47 | 32 |
| 2008 | 69 | 22 |
| 2009 | 69 | 16 |
| 2010 | 91 | 30 |
| 2011 | 94 | 30 |
| 2012 | 120 | 102 |

Assessment

Moderate status.

Change from 2011

Reduction in quality with an increase in Phosphorous levels and a single significantly elevated maximum chlorophyll.