COMHAIRLE CHONTAE an CHABHÁIN

CAVAN COUNTY COUNCIL

WATER SERVICES

BYE-LAWS

2007

These byelaws are to prevent the waste, misuse, undue consumption or contamination of the public drinking water supply or the permitting of deleterious material to enter the wastewater system. They also detail how charges for water and wastewater are to be levied and collected.

MADE UNDER

THE LOCAL GOVERNMENT ACT, 2001

#### Cavan County Council

Bye-Laws for Preventing the waste, misuse, undue consumption or contamination of the public drinking water supply or the permitting of deleterious material to enter the wastewater system. Bye-Laws to detail how charges for water and wastewater are to be levied and collected by Cavan County Council

Cavan County Council, in exercise of the powers conferred on it by Part 19 of the Local Government Act 2001 (No. 37 of 2001) hereby make the following Bye-Laws relating to the management, protection, conservation of drinking water and charges for water and wastewater in its administrative area .These byelaws are to prevent the waste, misuse, undue consumption or contamination of the public drinking water supply or permitting deleterious material to enter the wastewater system and outline penalties for same. They also detail how charges for water and wastewater are to be levied and collected.

##### PART 1		  PRELIMINARY

Short Title: These Bye-Laws may be cited as Cavan County Council Water and Wastewater Bye-Laws 2006.

Area of Application: These Bye-Laws shall apply to the administrative area of Cavan County Council (hereinafter known as ‘the Council) and include the Administrative area of Cavan Town Council.

###### Date of Commencement:  ……

    - Definitions.  In these Bye-Laws

    - 'Approval' means approval in writing by the Water Services Department, Cavan County Council.

    - 'Authorised Person' means a person who is authorised in writing by Cavan County Council for the purpose of these Bye-Laws.

    - ‘Bulk Water Meter' means a meter for measuring the total (bulk) volume of water which is being delivered to a development or estate.

    - ‘Council’ means Cavan County Council

    - 'Customer' means any person or body who is consuming water supplied by Cavan County Council and/or delivering wastewater to Cavan County Council or its agents or who is applying to the Council or its agents in relation to a water or wastewater services.

    - ‘Commercial user’ means any person or body who is consuming water supplied by Cavan County Council and/or delivering wastewater to Cavan County Council or its agents or who is applying to the Council or its agents in relation to a water or wastewater services other than for domestic use.

    - ‘Displacement device or equivalent’ means a displacement device fitted in a w.c. cistern to reduce the quantity of water used in a flushing cycle.

    - ‘Domestic premises’ means any building or part of a building used by a person as his or her place of private residence. It does not include any part of a building used for the provision, for the purposes of reward, with a view to profit or otherwise in the course of business, of accommodation, including self catering accommodation, unless the person to whom the accommodation is so provided uses the accommodation as his or her principal place of private residence.

    - Effluent is any liquid waste, from a domestic or non-domestic premises.

    - 'Flow Meter' means a water meter which measures the volume and/or records the flow of water in a watermain.

    - 'Free Flow Meter' means a water meter which does not impede or restrict the free flow of water in a watermain.

    - ‘Housing Estate’ means an estate consisting of a multiple of buildings primarily provided for residential use, including houses, flats and apartments.

    - ‘Long Service’ means a service pipe greater than 100m in length from the connection with the public watermain to the customer’s premises.

    - ‘Meter’ means a device which measures the volume and/or records the rate of flow of water in a watermain.

    - ‘Non-Domestic Premises’ means any building or part of a building which is not a domestic premises as defined in these Bye-Laws.

    - ‘Occupier’ means the person or persons occupying the premises whether owner, lessee, tenant or any other person having, for the time being, control of the premises.

    - ‘Water fit for Human Consumption’ means drinking water or potable water supplied by the Council and complying with the requirements of EU law as incorporated in national legislation.

    - 'Service Pipe' means so much of any pipe (including a branch pipe) for supplying water from the public watermain to any premises and which is subject to water pressure from that watermain, or would be so subject but for the closing of some valve.

    - ‘Service side leak’ a leak that occurs in a premises or between the premises and the Stop Tap.

    - Standing Charge is a fixed charge which includes a charge for

    - ‘Stop Tap (Valve & Meter) Box' (or ‘Boundary Box’) means an enclosure (usually placed in a footpath) or box, into which devices can be placed to control or measure the water supply entering a premises. It can also be used to identify and verify supply pipe water leakage, to facilitate the service testing of pressure and flow of water, and to monitor water quality.

    - 'Stopvalve or Stopcock' means a valve, other than a servicing valve, fitted in a customer supply pipe for controlling or stopping at will the flow of water.

    - ‘Sewer’ any pipe or conduit that carries wastewater as defined below.

    - ‘The Council’ means Cavan County Council.

    - ‘Wastewater’ Water that carries wastes from homes, businesses, and industries; a mixture of water that includes dissolved or suspended solids.

    - ‘Water Audit’ means a statement by a competent person or body (acceptable to the Council) and submitted to the Council in respect of an existing non-domestic premises to which a Water Conservation Policy Statement applies. The Council can request this statement annually at the customer’s expense. Such statement shall set out details of all water using equipment installed in the premises, the number of staff employed in the premises and an estimate of daily water requirement for the following 3 years. The audit shall identify any leaks or excessive use of water and identify how such leaks or excessive use is to be repaired/reduced.

    - ‘Water Conservation Policy Statement’ means a statement by a competent person or body (acceptable to the Council) and submitted to the Council for approval in relation to an existing non–domestic premises where required by the Council. Such statement shall set out details of all water mains supplying the premises; the internal plumbing arrangements; a full list of all water using appliances, with their capacities and consumption rates; and details of any leaks on the premises and how such leaks can be detected and repaired. Details of any water recycling and reuse shall be included in the statement.

    - ‘Water Management and Conservation Plan’ means a plan prepared by a competent person or body (acceptable to the Council) that shall be submitted to the Council for approval prior to the commencement of any development that is subject to permission under Planning and Development legislation. The plan shall set out details of how best practice in water conservation shall be applied in respect of the proposed development to include water mains and internal plumbing. The plan shall also set out details of how water wastage, leaks or excessive consumption may be identified and remedied.

    - ‘Water Services Department’ means the Department of Cavan County Council responsible for the supply of water to domestic and non-domestic premises and the treatment of effluent from domestic and non-domestic premises and acts as the Sanitary Authority for the County.

    - ‘WC Suite" means any suite consisting of a flushing cistern and pan or urinal into which water is discharged.

##### PART 2		PROTECTION OF WATER SUPPLY

2.1	Obligations applicable to all premises (Domestic and Non-Domestic):

    - Any person or body requiring a new or additional water supply or connection to the Council water distribution network will need to comply with the requirements set out in the Cavan County Council ‘Conditions of Connection’.

These shall include but not limited to;

Where an application has been made to the Council for a new water supply or

service connection, the Water Services Department Section shall, at the expense of the

customer, carry out or witness the appropriate connections provided always that:

a) the water mains and/or service pipes have been laid and tested to the

satisfaction of the Water Services Department Section

b) the Council has been paid in advance for all expenses due for the water

connection

c) any financial contributions due to the Council pursuant to the Planning and

Development Acts 2000 as amended or extended have been paid in full.

Proof of payment for the foregoing shall be a matter for the applicant

    - The customer shall ensure that the water distribution system in his/her premises meets the requirements of the Water Services Department and is maintained so as to protect public health, prevent leakage and conserve water.

    - The approval of the Council shall be required prior to the laying of any new watermain and prior to the connection of that watermain or customer supply pipe to the Council's water supply

    - All materials and fittings used in the provision of a water supply to a domestic or non-domestic premises shall:

- be of an appropriate quality and standard, and

- be suitable for the circumstances in which they are used.

For the purposes of these Bye-Laws, a material is of an appropriate quality or standard only if;

- it bears an appropriate CE marking in accordance with the Directive;

- it conforms to an appropriate harmonized standard or European technical approval;

- it conforms to an appropriate Irish or British Standard or some other national specification of an EEA State which provides an equivalent level of protection and performance: or

- it conforms to a specification approved by the Water Services Department.

    - All work associated with the provision of a water supply to a domestic or non-domestic premises shall comply with the requirements of these Bye-Laws.

    - Any additional pipe, fitting or apparatus connected to any installation supplied with water by the Council and any alteration to any existing pipe, fitting or apparatus shall be carried out by a competent person in accordance with the requirements of these Bye-Laws. The Water Services Department shall be notified in writing, in advance of any major or significant additions or alterations.

    - The Council shall maintain the portion of any service pipe laid in any road from the watermain pipe to the street control stop valve/boundary box. The customer shall be responsible for the section of the service pipe which extends from the street control Stopvalve/Boundary box into his/her premises. Renewal of the service pipe from the street control Stopvalve/Boundary Box in through the immediate premises boundary shall only be carried out in accordance with the Council’s requirements.

    - All pipes which supply or may supply drinking water shall be flushed to remove debris including excess flux that may have collected in the pipes during installation, renewal or repair.  After testing and flushing installations shall be disinfected in accordance with the requirements of the Water Services Department.

    - All water systems shall contain an adequate device or devices to prevent the occurrence of backflow.

    - Any apparatus which would have the potential effect of rendering drinking water unfit for human consumption shall not be fitted.

    - The fitting of a pump to the direct water supply from the public mains shall only be undertaken with the consent of and in accordance with the requirements of the Water Services Department.

    - Each premises shall be fitted with a cold water storage facility adequate and appropriate for the use of the premises and in accordance with the Council’s requirements.  Such facility shall be maintained in accordance with the manufacturer’s guidelines or other best practice.

    - Every customer shall ensure that water supplied to his/her premises for drinking purposes shall not deteriorate to such an extent that it is no longer suitable for human consumption.

    - All fire hydrants shall be of type approved by the Fire Officer of Cavan County Council. Currently a screw down type to B.S. 750: 1984.

    - Waters of varying degrees of softness, hardness or other characteristics may be received in any particular part of the water supply area. The customer shall ensure that the hot water and the cold water supply installations, in the water distribution in his/hers premises, are suitable for the type of water supplied.

    - Employees or agents of the Council engaged in the provision of water supply services may enter a premises at any reasonable time to inspect the water supply and/or carry out essential works in order to protect public health or at any time in cases where in the opinion of the Council, an emergency exists.

    - The Council may interrupt the water supply of the customer, without prior notice, for the purpose of protection of public health, inspection, leak detection or any other valid reason.  Notwithstanding the foregoing, the Council will use its best endeavours to notify customers in advance in cases where the interruption to supply is likely to exceed 4 hours.

    - The Council may, having regard to the Local Government (Delimitation of Water Supply Disconnection Powers) Act 1995 if appropriate, disconnect the water supply of a customer for any valid reason and, in particular, if in the opinion of the Council, the water supply is a risk to public health.

    - It is an offence to tamper with a watermain including all associated valves, hydrants and meters without the prior written permission of Water Services Department.

    - In the event of failure to comply with these bye-laws the Council may disconnect the customer. The cost of this disconnection and any subsequent reconnection will be borne by the customer.

    - It shall be the responsibility of the customer to notify the Council of any change in contact details or any change in ownership of the property. In the event of change of ownership, the existing consumer is liable for all charges against the property until such notification is given in writing to the Council.

    - The Council shall set, each year, a domestic allowance for buildings which have both domestic and non-domestic uses. Notice of any change in the domestic allowance will be given to all affected consumers.

2.2	Additional obligations applicable to Non-Domestic Premises:

    - Employees or agents of the Council engaged in the provision of water supply services may, at all reasonable times, enter the premises of a customer for the purposes of installing, replacing, reading, inspecting, disconnecting or removing the meter(s) and for any other purpose in connection with the supply of water.

    - In the event of sub-letting or disposal by the customer of the property or any part thereof, Water Services Department shall be so informed in advance in writing and provision shall be made for separate water connection facilities to each portion under separate ownership/control.

    - It shall be the responsibility of the customer to notify the Council, in writing, of any change in contact details or any change in ownership of the property. In the event of change of ownership, the existing consumer is liable for all charges against the property until such notification is given in writing to the Council.

PART 3		WATER CONSERVATION

###### 3.1	Obligations applicable to all premises (Domestic and Non-Domestic):

    - Where requested by the Council, no development shall be commenced (except in respect of a new single dwelling or extension to an existing single dwelling) prior to submission to the Council of a Water Management and Conservation Plan and approval by the Council in writing of the said Plan.

    - Each new WC suite shall be of multi flush type and approved by Water Services Department.

    - Each WC suite installed in a newly constructed premises or in a premises converted from another use shall have a maximum flush of 6 litres using a multi flush or a single flush facility.

    - Each new WC suite installed in an existing premises, whether it is an additional WC suite or a replacement for an existing suite, shall have a maximum flush of a determined volume, currently 6 litres using a multi flush or a single flush facility. This limit will be determined by Water Services Department annually.

    - Each WC cistern with a volume in excess of the determined volume, currently 6 litres, may be fitted with a Displacement Device or equivalent with the objective of achieving water conservation where, in the opinion of the Council, this is feasible.

    - Each new domestic premises shall be fitted with a Stop Tap (Valve and Meter) Box (or Boundary Box).

    - In accordance with the Council’s requirements, each new housing estate shall incorporate a Bulk Water Meter within its water mains.

    - A hosepipe (sprinkler or similar fitting) shall not be connected to a draw–off tap or other similar fitting for use either inside or outside any premises during periods of water shortages as defined by the Council unless approved in writing by the Council.

    - Any unauthorised use of a fire hydrant or use of a water supply laid on solely for fire fighting purposes for any other purpose shall be an offence and the Council may impose a fine for this offence and/or charge for the water used.

    - Plumbing systems and all fittings used in the supply of water shall be of a type which achieves the objective of conserving water.

    - Washing machines, dishwashers and other appliances shall be economic in the use of water and plumbed in a manner that achieves the objective of conserving water and in accordance with the Council’s requirements.

    - The Water Services Department may require a meter to be installed on any or all connections for the purpose of managing or conserving water.

    - Any hand held hosepipe used shall be fitted with a self-closing mechanism at the outlet of the hosepipe.

    - Porous hose irrigation systems or soil watering systems using fixed sprinkler heads shall not be used without the written approval of the Council.

    - The Council, having identified a service side leak, may give a notice to the customer to repair the leak within 3 working days. In the event that the leak is not repaired the Council may disconnect the water supply without further notice. If the Council should consider the leak to be of such a magnitude as to require emergency attention, the Council shall have the right to enter and carry out any necessary repair at the expense of the customer.

###### 3.2	Additional obligations in relation to Non-Domestic Premises and Commercial users.

    - Where requested by the Council, a Water Conservation Policy Statement shall be submitted, within a period not exceeding 3 months from the date of request, for approval to the Council by the occupier of any premises. Failure to submit such a statement shall be an offence. A new statement shall be submitted every 3 years or at such other times as determined by the Council or at any change of use or ownership. The occupier of premises to which a Water Conservation Policy Statement applies shall carry out a Water Audit at least once every year and shall submit the Audit Report to the Council if so requested. In the interests of water conservation following consultation with the occupier the Council may stipulate a maximum permitted usage of water in a premises and the occupier may only exceed such limit after prior written approval from the Council.

    - Each premises shall be connected to a Flow Meter or shall have its water flow assessed by the Council

    - Where automatic flushing cisterns are installed they shall be installed to achieve the objective of conserving water.

    - Free Flow Meters, of a type approved by the Council, shall be installed in all fire mains where deemed necessary by the Council.

    - A hosepipe shall not be used for the purpose of washing vehicles, watering gardens or shrubs/flowers or any such purpose except on installations where the use of the hosepipe is essential to the business being carried out and the installation meets the requirements of the Water Services Department.

    - No temporary connection shall be made to the Council’s water supply by way of stand pipe, hose pipe or other without the written permission of the Council. A licence for a temporary connection for construction purposes, mechanical street sweepers, rollers tankers or others may be obtained from Water Services Department.

PART 4		WATER SERVICE CHARGES

4.1		Water

## 4.1.1	The amount of water supplied to the non domestic customer shall be ascertained by means of a meter or meters and other apparatus supplied and installed by the Council, or persons acting on behalf of the Council, and such meter or meters shall remain the property of the Council.

## 4.1.2	The Council or persons acting on behalf of the Council shall read the meter(s). Only persons authorised by the Council may fix, connect, remove or work upon any equipment associated with the recording of the water supply and belonging to the Council.

## Where a meter is not in place at present and pending installation, the Council may levy a flat rate charge in consideration of the service provided.

## When, for any reason, a meter has failed to record correctly the water used, the Council reserves the right to adjust the customer's account to reflect reasonable usage on the basis of historical or comparative data.

## At the request of the customer, the Council will test the accuracy of the meter to ascertain if it is within the prescribed range of accuracy for the meter, which shall be ±5% unless agreed otherwise in writing. If it is outside this range the customer's account will be increased or reduced accordingly. Otherwise the recording of meters and other apparatus shall be deemed conclusive, and the customer may be charged for the cost of the test.

## It is an offence to interfere with a water meter as per 2.1.19 above. In the event of unauthorised interference, whether by the customer or otherwise, whereby water could have been consumed without being properly metered, the Council reserves the right to estimate the unrecorded consumption and include the charges for same in the customer's account. This is in addition to any fine imposed for the breach of these bye - laws.

## 4.1.7	If, in the opinion of the Council the volume of the customer’s water consumption is such that it is likely to produce what the Council determines to be a significant impact upon the water treatment works, then the Council may require the customer to enter into an agreement to reserve capacity at the said works for the purpose of producing water for the customer.

4.2		Wastewater

## 4.2.1	Without prejudice to any licensing obligations which exist or may be imposed the customer shall at no time permit to be discharged into the drainage system:

## any liquid matter or thing which is or may be liable to set or congeal at average sewer temperatures (between 4 and 25 degrees Celsius), or by which the free flow of sewage or storm water may otherwise be interfered with, or is capable of giving off any flammable or explosive gas or any noxious odour in sufficient concentration to cause a nuisance; (included are oils, fats and grease)

## waste water which by nature of volume or composition is likely to adversely affect the performance of a waste water treatment plant(s) of the Council

## any acid, alkali or other substance in sufficient concentration to cause corrosion to sewer network components;

## any matter that causes or is likely to cause a risk to human health or safety or the environment.

## The customer shall comply with any reasonable direction issued by the Council to ensure compliance with this clause.

## 4.2.2	A person shall not allow the discharge into the public sewer of rainwater run-off from roofs, paved areas or other surfaces, except as may be agreed with the Council in writing.

## 4.2.3	The Council shall be entitled to take spot samples of the discharged wastewater for the purposes of testing compliance with these bye-laws.

## 4.2.4	If, in the opinion of the Council the characteristics of the customer’s effluent are such that it is likely to produce what the Council determines to be a significant impact upon the waste water treatment works, then the Council may require the customer to enter into an agreement to reserve capacity at the said works for the purpose of treating the waste water of the customer.

4.2.5	Food Grinders will not be installed without the approval, in writing, of the Council.

## 4.3	Water Service Charges

## 4.3.1	There will be a single consolidated charge for Water Services Department, comprising of three parts, a standing charge, a charge for water supply and, a charge for waste water treatment.

## 4.3.2	The charge will consist of a unit charge related to volume (m3) of water used/discharged. Unless otherwise agreed, or where the customer has an independent water supply, the volume of waste water treated will be deemed to be equal to the volume of water supplied based on the water meter readings.

## 4.3.3	The consolidated unit charge will include (inter alia) contributions to cover:

- - the marginal capital costs of the water / waste water treatment works and delivery/drainage systems;

- - the average operational costs of the water / waste water treatment works and conveyance /drainage systems;

- - the average general administration, metering, monitoring, debt recovery and billing costs.

    - The cost of production and distribution of one unit of treated water, and the cost of collection and treatment of one unit of waste water shall be determined at year-end by the Council.

## 4.3.5	The Council may, as a condition of service, require the metering of the water supply, or wastewater discharge as appropriate, where the Customer has a private water supply, solely for the purpose of the determination of the volume of wastewater according to the principles described above.

4.4	Payments and Terminations.

- 4.4.1	The customer is liable to pay the Council all charges for all water consumed or estimated to have been consumed and all waste water discharged or estimated to have been discharged.

- 4.4.2 	 All invoices shall be sent to the billing address shown on the application form (or any other address notified to the Council).

    - All charges and other payments must be paid in full by the due date specified. The Council shall be entitled to charge an interest rate not exceeding 2% above the European Central Bank’s marginal lending facility on all accounts that remain outstanding after the due date specified.

    - Where a customer has not paid in full by the due date the amount specified in the invoice, the Council may serve a notice on the customer stating that it may disconnect the water supply to the premises. The notice shall be served at the billing address notified to the Council or at the premises to which the notice applies. If the invoice remains unpaid seven days after the service of such a notice, the Council may disconnect the water supply to the premises without further notice. The customer shall continue to be responsible for all unpaid amounts together with the disconnection charge and any future reconnection charges.

- 4.4.5  The Council shall not be liable for any injury, loss or damage, whether or not foreseeable, directly or indirectly resulting from:

- a) any failure, interruption, delay, suspension or restriction in providing water services      arising for any reason

- b) or any variations in water quality, taste, hardness, colour, pressure or any other quality parameter;

- c) or any unlawful or unauthorised use of the service by the customer or any other person.

- 4.4.6 The Council may disconnect the water supply of the customer for any valid reason, including:

- a) the customer fails to pay the Council the Water Services charges due, (subject to the statutory rights of the customer);

- b) or breach of any of the conditions of these Bye-Laws;

c) the water is connected to or used for any unauthorised development

d) the water is drawn through any unauthorised connection

e) for any other valid reason given by an Authorised person

4.4.7	The Council may in extreme cases of abuse, as determined by an authorised person, disconnect the water supply or wastewater connection to the collection network without prior notice. The cost of this disconnection and any subsequent reconnection will be borne by the customer.

- 4.4.8 The customer may apply for termination of the water supply and/or waste water treatment by giving seven days notice to the Council. The customer is liable for the costs incurred for his or her consumption of water up to the date of disconnection.

  - 4.4.9 The customer may arrange for termination of payments to be effected without disconnection, if satisfactory arrangements for the transfer of liability to a person acceptable to the Council are in place.

  - 4.4.10 	If a business ceases to trade, the property may revert to a domestic premises on production of written certification from the Revenue Commissioners or valued as ‘list non rateable’ by the valuation office or other source acceptable to the Council.

4.4.11		In the case of a multiple occupancy property with a single service connection it is the responsibility of the owner of the property to pay all charges against the property.

PART 5		OFFENCES.

  - A person who contravenes any provision of these Bye-Laws shall be guilty of an offence and shall be liable on summary conviction to a fine not exceeding €3,000.00 (together with, in the case of continuing contravention, a fine not exceeding €1,000 for every day on which the contravention is continued) or to imprisonment for a term not exceeding 6 months.

  - Nothwithstanding any fine or penalty imposed, the Council may recover any costs associated with loss of water, repairs or reconnections to the watermain or the repairs or release of any blockages caused to the sewer and/or damage to the treatment works.

  - An authorised person may request any person who appears to be contravening or to have contravened a provision of these Bye-Laws to refrain from such contravention and that person shall comply with the request of the authorised person.

  - A person who refuses to comply with the request of an authorised person or who obstructs or impedes an authorised person in the exercise of his/her functions under these Bye-Laws shall be guilty of an offence.

  - Where an authorised person is of the opinion that a person is committing or has committed an offence in relation to the provisions of these Bye-Laws, the authorised person may demand the name and address of such person and if that demand is refused or the person gives a name or address which is false or misleading, that person shall be guilty of an offence.

  - Where an offence under these Bye-Laws is committed by a body corporate and the offence is proved to have been committed with the consent or connivance of, or be attributable to any neglect on the part of, any director, manager, secretary or other similar officer of the body or any person who was purporting to act in any such capacity, that officer or person, as well as the body, shall be guilty of the offence.

  - Where the affairs of a body corporate are managed by its members, Section 5.6 shall apply in relation to the acts and defaults of a member in connection with his or her functions of management as if he or she were a director of the body corporate.

  - The Council may issue a person or body corporate that contravenes any provision of these Bye-Laws with a Fixed Payment Notice specifying that if an amount of € 60 is paid within 21 days of the date on which the Notice was issued, the Council shall not proceed with a prosecution.  Notice of the fixed payment shall be in the general form of the notice in the First Schedule to these Bye-Laws.

FIRST SCHEDULE

CAVAN COUNTY COUNCIL

WATER and WASTEWATER BYE-LAWS 2007

FIXED PAYMENT NOTICE FOR THE PURPOSES OF SECTION 206 OF THE LOCAL GOVERNMENT ACT, 2001

[BYE-LAW NUMBER 5.9]

NAME OF LOCAL AUTHORITY:	CAVAN COUNTY COUNCIL

TO:

Name:		_______________________________________

Address:	_______________________________________

_______________________________________

It is alleged that you have contravened the provisions of Cavan County Council Water and Wastewater Bye-Laws 2007 made under Part 19 of the Local Government Act, 2001 entitled ……………………………………………………………………………………

by…………………………………………………………………………………………………………………………………………………………………

(in general terms specify nature of contravention) at …………………………………………………………………..

on ……………………………………………………………………..  .  During this period of 21 days beginning on the date of this notice you may pay the sum of €……………………………………, accompanied by this notice, at the offices of the local authority named in this notice, located at …………………………………………………………………………………………………………………………………………………………………… .

A prosecution in respect of the alleged contravention will not be instituted during the said period and if the sum of €………………………………………….. is paid during that period, no prosecution will be instituted at any time in respect of the said alleged offence.

Signed:	___________________________	Date:	_____________________

(Authorised Officer)

IMPORTANT:	Payment will be accepted at the offices of the local authority specified above and must be accompanied by this notice.  Payment may be made by post.  Cheques etc., should be made payable to Cavan County Council.

A receipt will be given.

You are entitled to disregard this notice and defend the prosecution of the alleged contravention in Court.

SECOND SCHEDULE

Authorised Persons

Director of Services with responsibility for Water Service

Senior Engineer Water Service

Senior Executive Engineer Maintenance

Each Area Engineer

Each Caretaker