Appendix 21 – Aquifer Categories

Refer to Map 14

There are eight aquifer categories defined in Groundwater Protection Schemes (DoEHLG/EPA/GSI, 1999) as follows:

Regionally Important (R) Aquifers

- Karstified bedrock (Rk)

- Fissured bedrock (Rf)

- Extensive sand and gravel (Rg)

Locally Important (L) Aquifers

- Sand and gravel (Lg)

- Bedrock which is Generally Moderately Productive (Lm)

- Bedrock which is Moderatley Productive only in Local Zones (Ll)

Poor (P) Aquifers

- Bedrock which is Generally Unproductive except for Local Zones (Pl)

- Bedrock which is Generally Unproductive (Pu)

Note that during the course of the National aquifer delineation for the Water Framework Directive (2000), a further aquifer category was established: Lk – locally important karstified bedrock.  Regionally important karstified bedrock aquifers (Rk) may, depending on the degree and nature of the karstification, be further characterised as either Rkc – dominated by conduit flow or Rkd – dominated by diffuse flow.

- “Excellent” well yeilds defined as >400 m³/d.  “Good” well yeilds defined as >100 m³/d but les than 400 m³/d

- Productivity class ranges from I to V.  Class I implies that significant quantities of groundwater can be abstracted with little consequent drawdown of the groundwater level in the borehole.  A productivity class of V indicates that the drawdown of the groundwater level in a borehole can be significant for a given abstraction rate.

- The amound of groundwater rechage, and the degree of aquifer connectivity compartmentalisation, are considered as additional factors when determining the aquifer classificatons.

| Aquifer Category | T values (m²/day) | Productivity | Borehole Yields | Potential extent of flow system | Large springs | Lithology |  |  | Structure | Surface water discharge zones in areas of thin or free draining subsoil | Annual fluctuation in water levels |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Types | Dolomitic | Karst features |  |  |  |
| Rf | Most >50 Several >500 | Mostly I & II | Excellent yields very common | Regional | Potentially | Thin bedded sandstones, limestones, volcanics | Potentially | Little or none | Volcanics and thick bedded limestones generally highly fractured | Lowland drainage density <0.5 km/km² Annual baseflow >60% annual river flow Low flows >21/sec/km² and low flows >20% average flows (Rkd) may have lower low flows). | Generally <10m |
| Rkd | Variable. A few >500 | Mostly I & II but fair proportion may be lower | Excellent yields very common | Regional | Potentially | Pure of dolomitic limestones | Potentially | Abundant | Thick bedded limestones generally highly fractured |  | Generally <15m |
| Rkc | Variable. A few >500 | Probably all classes, average may be III | Extremely variable | Regional | Potentially | Pure limestones | Potentially | Abundant | Thick bedded limestones generally highly fractured |  | Often > 15m |
| Lm | Some >50. A few >500 | Average III | Excellent yields very common | Regional to local | Potentially | Pure limestones, thin-bedded sandstones, volcanics | Potentially | Occasional | Volcanics and thick bedded limestones generally highly fractured |  | No criteria |
| Lk | As for Rkc or Rkd | As for Rkc or Rkd | As for Rkc or Rkd | Local | No | As for Rkc or Rkd | As for Rkc or Rkd | As for Rkc or Rkd | As for Rkc or Rkd | No criteria | As for Rkc or Rkd |
| Ll | Some >50. A few >500 | Average III – IV, some II | Some excellent yields | Local (occasionally longer along fault zones) | Perhaps but unusual | Impure limestones, sandstones, shales, others | Perhaps, but not extensive | Occasional (in limestones) | No criteria | High drainage density, low baseflows | No criteria |
| Pl | Most <50. One or two >500 | Mostly V & IV, some II | Excellent yields very rare if any | Local | No | Impure limestones, sandstones, shales, others | No | None | No criteria | Values will be complicated by upland climatic setting and steep slopes | No criteria |
| Pu | <50 | Mostly V & IV | No excellent yields. Good yields rare if any | Very localised | No | Impure limestones, sandstones, shales, others | No | None | No criteria |  | No criteria |